COMPLETE LEARNING HANDBOOK

# Embedded Linux
Kernel and Firmware

A practical learning program

Lessons, projects, assessments and interview preparation for embedded Linux applications, kernel and BSP development, and MCU firmware with an RTOS.

18 modules     15 projects     200 interview questions

| Career track | Work you will practise |
| --- | --- |
| Embedded Linux applications | Processes, I/O, concurrency, services and reproducible images |
| Kernel and BSP development | Device tree, drivers, resource ownership and subsystem interfaces |
| MCU firmware and RTOS | Startup, peripherals, tasks, communication and recovery |

52 weeks  |  572 planned study hours

Includes complete project source, separate assessment keys, an editable progress workbook and recorded validation evidence in the accompanying folders.

Prepared edition 27 October 2026

# How to use this package

Extract the complete ZIP into one folder. This handbook is the main reading document; the folders beside it contain the editable learning material, runnable source, exercises, test fixtures and evidence. Keep that folder structure intact.

| Your task | Open this item |
| --- | --- |
| Read the complete program | The Word handbook at the package root |
| Plan and record progress | The Excel workbook in 10_Progress_Tracking |
| Begin with a diagnostic | Diagnostic.md in 04_Skill_Assessments/questions |
| Build a project | The selected project README.md in 07_Projects |
| Find a supporting file | FILE_INDEX.md at the package root |

Your first study session

1. Read the environment setup and record your host and compiler versions.

2. Take the initial diagnostic before opening the answer key.

3. Study M01 and M02, then build B01 from its project runbook.

4. Attempt the independent exercise and retain your source, test log and explanation.

5. Update the tracker only when you have your own evidence for the stated skill.

Choose a route through the material

For systematic learning, follow the weekly roadmap and its prerequisites. For a specific job, use the skills matrix to prioritise the required track. For an approaching interview, use the 30-day revision plan after checking your foundations.

Read the evidence accurately

The 52-week schedule budgets 11 hours per week. Prerequisite remediation and physical board bring-up can extend the plan. Your proficiency starts as not assessed. Reference tests show what was checked in this package; they do not establish your competence.

The validation chapter separates host tests, emulation, object compilation and MCU cross-builds. Physical hardware execution and several full-image and kernel-runtime checks remain pending. Each project states its own limits.

# Contents

Select a chapter title to jump to it. The module and project indexes on the next two pages provide direct links to individual lessons and runbooks.

1 Start here	6

Starting profile, study method and environment setup

2 Roadmap and requirements	9

52-week sequence, prerequisites and job skills mapping

3 Learning modules	23

18 lessons with independent exercises

4 Fifteen project runbooks	52

Basic, intermediate, advanced and capstone implementations

5 Skill assessments	83

Diagnostic, quizzes, timed tasks, debugging and role mocks

6 Interview strategy and portfolio	96

30-day revision, interview technique and evidence presentation

7 AI integration	102

Reusable assistance workflows and verification practices

8 Interview question and answer bank	106

200 questions with explanations, follow-ups and scoring

9 Assessment and exercise answer keys	191

Reference answers for comparison after an independent attempt

10 Validation and progress	205

Executed checks, pending work and progress records

11 Primary references	209

Source catalogue, research method and refresh guidance

Use the Navigation pane in Word to browse headings. The Contents link in the footer returns here. After editing the document, update fields in Word to refresh page references.

# Module index

Follow prerequisites in the roadmap before taking a later module. Each lesson includes an independent exercise; the reference solutions are grouped in Chapter 9.

| ID | Lesson | Page |
| --- | --- | --- |
| M01 | Linux tools and reproducible engineering | 23 |
| M02 | C memory ownership and defined behavior | 24 |
| M03 | Data structures and defensive protocols | 26 |
| M04 | Build systems linking and C++ ownership | 28 |
| M05 | Processor and operating system foundations | 29 |
| M06 | Linux I/O processes and event loops | 31 |
| M07 | Concurrency synchronization and IPC | 32 |
| M08 | Cross compilation boot and device tree | 34 |
| M09 | Kernel execution contexts and resource lifetimes | 36 |
| M10 | Character and platform drivers | 37 |
| M11 | Peripheral buses sensor subsystems DMA and power | 39 |
| M12 | Bare metal startup peripherals and faults | 40 |
| M13 | RTOS scheduling and Linux to MCU communication | 42 |
| M14 | Buildroot Yocto and image maintenance | 43 |
| M15 | Debugging tracing and performance | 45 |
| M16 | Persistence security and reliable updates | 46 |
| M17 | Integration testing and capstone engineering | 48 |
| M18 | AI assisted learning and embedded inference | 50 |

Begin with the shared C and Linux foundations. Then allocate depth to embedded Linux applications, kernel and BSP development, or MCU firmware and RTOS according to your target role.

# Project index

Start each project with its runbook, prerequisites and validation scope. Complete source, configuration, tests and recorded evidence are in the matching folder under 07_Projects.

| ID | Level | Project | Page |
| --- | --- | --- | --- |
| B01 | Basic | Embedded C utility library | 53 |
| B02 | Basic | Linux process and resource inspector | 55 |
| B03 | Basic | UART terminal and framed protocol tool | 56 |
| B04 | Basic | Multithreaded sensor simulator and logger | 58 |
| B05 | Basic | Cross compilation and QEMU application debugging | 59 |
| I01 | Intermediate | Custom Buildroot appliance | 61 |
| I02 | Intermediate | Kernel character driver and user space client | 63 |
| I03 | Intermediate | Device tree GPIO interrupt platform driver | 65 |
| I04 | Intermediate | Custom Yocto telemetry image | 67 |
| I05 | Intermediate | RTOS telemetry connected to Linux | 69 |
| A01 | Advanced | Sensor driver using the IIO subsystem | 71 |
| A02 | Advanced | Linux latency and performance investigation | 73 |
| A03 | Advanced | Signed A B update and rollback simulator | 74 |
| C01 | Capstone | Industrial monitoring and edge gateway | 77 |
| C02 | Capstone | Linux MCU robot control and diagnostics simulation | 80 |

Build from the complete folder

Keep the project source, configuration and tests together. The runbook explains the target, commands, expected behaviour, failure cases and remaining checks. The host verification runner can check the supported offline portions of the package.

A reference implementation is a starting point for your own work. To demonstrate the skill, change a requirement, test a failure and explain the result independently.

# 1 Start here

## Your starting point and study method

This program develops evidence for three related jobs: embedded Linux system software, Linux kernel and board support, and microcontroller firmware with a real time operating system. Choose one primary hiring target after the diagnostic. Learn the shared foundations once, then allocate depth according to the target job description. Completion means you can explain, change, test and debug your own implementation.

### Working assumptions

No personal proficiency, specific job description, interview deadline, board inventory or computer specification has been supplied. Your starting skills are not assessed. The planning baseline is junior to mid level preparation with 10 to 12 study hours each week. The 52 week schedule contains 572 planned hours at 11 hours per week. This includes lessons, projects and assessment; project effort is not added a second time. Budget a further 80 to 180 hours if the diagnostic reveals weak C foundations or if you complete physical board bring-up. A full three-track study therefore merits approximately 12 to 18 months as an initial planning estimate, with longer duration possible. Nine to twelve months is a focused-track target only if prerequisites are already demonstrated.

The minimum project scope uses a Linux host, emulated applications, a real FreeRTOS host port, and explicitly identified host simulations for system integration. Physical GPIO, I2C, electrical timing, motor behavior and power interruption remain separate validation work. The robot capstone is an educational demonstrator. Its software stop condition is not an industrial safety function.

### Choose your primary track

| Track | Work you must demonstrate | Additional depth after shared foundations |
| --- | --- | --- |
| Embedded Linux system software | A service that handles real I/O failures and ships in a reproducible image | Process lifecycle, sockets, IPC, image integration, persistence, updates and observability |
| Kernel and BSP | A driver whose contexts, synchronization and resource lifetimes you can defend | Device tree, bus/subsystem APIs, IRQs, DMA, power management, configuration and board bring-up |
| MCU firmware and RTOS | A bounded state machine with measurable timing and recovery | Startup/linker configuration, peripherals, ISR contracts, scheduling, watchdogs and firmware updates |

Do not claim all three specialties from completing one demonstration. For a kernel position, hardware and kernel runtime evidence matter; a Python sensor simulator does not establish driver competence. For an MCU position, host-port execution does not establish interrupt latency or stack sufficiency on the selected chip.

### First study session

1. Read the environment guide and record the host/compiler versions.

2. Take the diagnostic in 04_Skill_Assessments before opening its answer key. Save your attempt and time used.

3. Start M01 and M02. Build B01 from its README, then close the reference code and implement the independent ring-buffer exercise.

4. Record a source commit or file hash, test log and five-minute verbal explanation. Leave your tracker status as not assessed until evidence is scored.

5. Retake the weak topics after a delay with changed input data. A remembered solution is insufficient evidence of transfer.

### A sustainable study week

Allocate three hours to lessons and source reading, four to implementation, two to tests and fault investigation, one to independent recall, and one to review and evidence. Shift lesson time toward projects when integrating a capstone. End each week by explaining one bug, one design tradeoff and one result without AI assistance.

An exit test passes only if its required behaviors and critical correctness gates pass. Out-of-bounds access, a demonstrated race, unsafe resource lifetime, accepting invalid commands, or claiming unrun evidence requires correction even if other marks are high. Keep failed attempts: a defensible diagnosis and regression test are valuable interview evidence.

### Depth labels

Explain means reason accurately with an example. Implement means build the feature from a requirement. Debug means diagnose an unfamiliar failing case with evidence. Recognize means identify where the concept applies and find its authoritative contract. Defer means postpone until the JD or hardware demands it. C ownership, partial I/O, concurrency and failure handling require implement and debug depth across tracks. RCU and DMA begin at recognize/explain depth for a junior application role and become deeper requirements for a relevant driver role.

## Development environment and version policy

### Host path

Use a disposable x86_64 Linux virtual machine for kernel experiments. Ubuntu 24.04 LTS is the general user-space teaching host; use an Ubuntu 22.04 LTS VM for the frozen Yocto 5.0.9 exercise because that release's supported-host list includes 22.04. Windows users can start C and POSIX exercises in WSL2, but the kernel you are modifying or loading a module into must be the intended disposable guest. A container shares its host kernel and is not an independent kernel test machine. [YOCTO-HOST]

For the general path, a planning allocation of 4 CPU cores, 8 GB RAM and 30 GB free space is comfortable for the small host projects. For image builds, prefer 8 CPU cores, 16 to 32 GB RAM and 150 to 250 GB free SSD space; these are planning estimates, not measured requirements. Yocto 5.0.9 documents 90 GB disk and 8 GB RAM for a representative core-image-sato build, with more useful for caches and complex images. Check the selected release before provisioning. [YOCTO-HOST]

These commands describe a learner-owned Ubuntu VM. They were not all executed in the authoring environment. Review package installation in your own VM:

sudo apt update

sudo apt install build-essential git ripgrep gdb gdb-multiarch gdbserver strace file binutils \

make cmake pkg-config python3 python3-venv socat unzip xz-utils \

gcc-aarch64-linux-gnu qemu-user qemu-system-arm qemu-system-x86 \

bc bison flex libssl-dev libelf-dev libncurses-dev device-tree-compiler

gcc --version

python3 --version

uname -a

Use the specific extra host-package list in I01 and I04 for their image builds. Create a dedicated writable work directory with no spaces, keep source and build outputs separate, and retain .config, source tags and command lines. Do not run ordinary compilation as root. Never run a kernel module built for one kernel against a different kernel's headers or release just to see if it loads.

### Version contract

| Component | Teaching baseline | Meaning |
| --- | --- | --- |
| C | C11 source, GCC 13.3 host checks where available | Portable core code with explicit POSIX extensions |
| C++ | C++17 ownership examples | Optional deeper C++ depends on the JD |
| Linux drivers | Upstream v6.12 source API | Frozen teaching target; not a claim that 6.12.0 is current or patched |
| Buildroot | 2025.02.1 | Frozen full-system QEMU exercise; use its generated kernel contract |
| Yocto | 5.0.9 Scarthgap | Frozen qemux86-64 layer exercise on documented host |
| FreeRTOS | Kernel V11.1.0 POSIX port | Actual RTOS task/queue code on host; MCU timings pending |
| QEMU | Record installed version and machine type | Linux user mode and full-system emulation are distinct |

The exact per-project VERSIONS.md takes precedence where a BSP needs a different kernel. An installed host compiler version is observed evidence; the named learner baseline is a target configuration. Keep those columns separate. Before connecting a product to a network or deploying it, move the frozen examples to a maintained release, review API differences and rerun acceptance tests. Do not silently substitute a new release and retain old validation claims.

### What emulation establishes

QEMU Linux user mode runs a foreign-architecture application and translates system calls using the host kernel. It does not boot the target Linux kernel. Full-system QEMU boots a guest kernel and devices in the machine model; a generic virt machine is not a Raspberry Pi electrical model. A host protocol test proves framing logic for its tested cases; it does not prove line voltage, real UART DMA, cable noise tolerance or interrupt latency. [QEMU-USER] [QEMU-VIRT]

### Hardware route when you reach it

Use Raspberry Pi 4 Model B for the optional Linux GPIO/I2C exercises and a NUCLEO-F446RE for MCU learning. The project-specific hardware notes are authoritative for wiring. Read board revision markings, jumper states, connector labels and peripheral voltage requirements before connecting. A USB-to-UART adapter must expose compatible logic levels; RS-232 line levels are a different interface. A logic analyzer is useful for framing and timing; an oscilloscope is needed for electrical waveforms. No purchase is needed for B01 through B04 or the host capstone simulations. [PI-HW] [NUCLEO]

Keep power disconnected while changing connections. The first physical robot demonstration should use a low-energy bench setup with motors lifted, current limits and a separate reachable means to remove actuator power. Scaling to an industrial robot requires separate electrical, mechanical and functional-safety engineering beyond this learning package.

# 2 Roadmap and requirements

## Roadmap and prerequisite sequence

The core schedule contains 52 weeks at 11 hours per week, for 572 hours. It assumes deliberate practice with the complete reference implementations available after an independent attempt. Weeks are planning slots, not automatic promotions. Repeat or split a week when its exit test fails. Project estimates overlap the scheduled lab hours; they are not extra hours to add to the total.

### Phase milestones

| Weeks | Phase | Core evidence | Gate |
| --- | --- | --- | --- |
| 1 to 4 | Shared C and tools | B01 tests and independent parser/ring exercise | G01 |
| 5 to 10 | Linux system software | B02 B03 B04 with error and shutdown tests | G02 |
| 11 to 22 | Cross builds images and kernel drivers | B05 I01 I02 I03 plus matching-kernel checks | G03 |
| 23 to 28 | MCU and RTOS foundations | I05 real RTOS host port and physical-port plan | G04 |
| 29 to 40 | Image integration sensors performance updates | I04 A01 A02 A03 and production review | G05 |
| 41 to 50 | Two integrated capstones | C01 and C02 acceptance tests and defenses | Capstone tests |
| 51 to 52 | Hiring evidence | Final practical primary-track mock and portfolio | G06 |

The complete week-by-week records include prerequisites, objectives, lesson links, exercises, revision and exit tests in Weekly_Roadmap.csv and the companion workbook. Update the evidence column with your own files, logs and commits. Supplied implementation test logs describe the package, not your proficiency.

### Prerequisite map

M01 tools enables M02 C. M02 enables M03 parsing and M04 build/link ownership. M05 architecture and M06 POSIX require C competence. M07 concurrency follows POSIX resource handling. M08 cross/boot follows build tools and architecture. M09 kernel contexts follows concurrency and boot. M10 drivers follows kernel lifetimes; M11 hardware subsystems follows drivers. M12 MCU startup follows linking/architecture; M13 RTOS follows concurrency and MCU foundations. M14 image systems follows cross/boot and services. M15 debugging runs throughout and deepens after concurrency/kernel study. M16 production follows image integration. M17 capstones combine protocol, RTOS and recovery. M18 AI assists every phase only after the independent attempt.

Use the prerequisite diagram in the handbook to identify parallel study: you can study MCU startup while a Linux image builds, but do not begin a concurrent driver before you can explain synchronization and object lifetime.

### Full path and role emphasis

For embedded Linux roles, give extra repetitions to M06, M07, M14, M15 and M16, with C01 as the main defense. For kernel/BSP roles, deepen M08 through M11 and obtain real driver runtime/hardware evidence before claiming board bring-up skill. For firmware roles, deepen M12 and M13 and complete the physical MCU port, timing and peripheral measurements. The same completed B01 parser can support all three tracks, while the evidence standards for specialized claims differ.

The optional C02 AI extension is scheduled as a choice in week 49. If core control or protocol acceptance is weak, use that week for correction and verification. Optional machine learning must not displace required control behavior.

### Minimum interview path

This is a revision and gap-closure path for someone who already passes G01 and basic POSIX work. It is not a beginner-to-kernel-developer promise.

| Week | Shared work | Primary-track extension | Required evidence |
| --- | --- | --- | --- |
| 1 | Diagnostic C pointers bounds integer rules | Review target JD and label explicit requirements | Independent C exercise and gap list |
| 2 | I/O partial progress errors timeouts | Linux service or MCU framing | One fault-tested parser/I/O task |
| 3 | Threads ownership shutdown | Kernel contexts or RTOS queues | Race/deadlock diagnosis and fix |
| 4 | Build cross tools debugging | Boot/DT for BSP or startup/linker for MCU | Reproducible build and failure explanation |
| 5 | One project end-to-end | I02 for kernel I05 for firmware C01 for applications | Demo with limits and test evidence |
| 6 | Timed coding and debugging | Mock for chosen role | Correct solution under stated tool rules |
| 7 | Architecture and failure design | JD-specific gaps | Design answer with explicit tradeoffs |
| 8 | Final mock and portfolio | Targeted remediation | No unresolved critical correctness failure |

At 10 to 12 hours per week, this path provides roughly 80 to 96 hours. It leaves substantial gaps in production depth, hardware integration and secondary tracks. Use the separate 30-day calendar only after foundations, and prioritize the actual interview scope if time is shorter.

### Remediation

If G01 fails, repeat ownership and parser exercises with smaller inputs, use a memory diagram and add boundary tests before proceeding to concurrency. If G02 fails, reduce the system to one pipe or queue, trace every descriptor/buffer owner and test shutdown. If G03 fails, pause new driver features, annotate context and lifetime, compile against the exact source, then reproduce in a disposable guest. If G04 fails, simplify to two tasks and one bounded queue, measure progress and define the watchdog. If G05 fails, enumerate persisted states and inject a failure after each state transition. If a capstone fails, map the failure to its lowest responsible interface and correct the requirement/test before adding features.

For a failed gate, record the failed requirement, evidence, root-cause hypothesis, targeted exercise and retest date. Retest with changed data after at least one study session of separation. Do not average away a memory safety, concurrency, unsafe-command or evidence-integrity failure.

Figure 1 Shared foundations lead to distinct kernel and MCU depth before system integration.

## Weekly study sequence

### Week 1 Diagnostic and environment

Prerequisites and modules: None; M01. Planned time: 11 hours.

Record host versions and reproduce a clean build. Milestone: B01 setup.

Practice and exit: Compile from an empty build directory and explain one diagnostic. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 2 C memory and integer behavior

Prerequisites and modules: M01; M02. Planned time: 11 hours.

Defend ownership and bounds for every API. Milestone: B01 API contracts.

Practice and exit: Reject invalid lengths and classify undefined behavior correctly. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 3 Ring buffers and state machines

Prerequisites and modules: M02; M03. Planned time: 11 hours.

Implement bounded storage and invariant tests. Milestone: B01 ring and bit utilities.

Practice and exit: Pass wraparound full empty exact-fit tests independently. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 4 Defensive packet parsing

Prerequisites and modules: M02 M03; M03 M15. Planned time: 11 hours.

Recover after corruption with bounded memory. Milestone: B01 complete.

Practice and exit: G01 exit plus parser exercise without reference code. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 5 Compilation linking and relevant C++

Prerequisites and modules: G01; M04. Planned time: 11 hours.

Inspect ELF and demonstrate move-only ownership. Milestone: B02 build setup.

Practice and exit: Diagnose one missing-symbol and one architecture mismatch. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 6 Processes memory and proc

Prerequisites and modules: M04; M05 M06. Planned time: 11 hours.

Read process data with race-aware errors. Milestone: B02 inspector.

Practice and exit: Handle disappearing and inaccessible processes without crash. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 7 POSIX I/O and process lifecycle

Prerequisites and modules: M05 M06; M06. Planned time: 11 hours.

Handle short I/O EINTR EOF and descriptor ownership. Milestone: B02 complete.

Practice and exit: Independent bounded pipe-reader test and resource accounting. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 8 UART and framed transport

Prerequisites and modules: B01 B02; M03 M06. Planned time: 11 hours.

Implement termios configuration and framing. Milestone: B03 complete.

Practice and exit: Fragmented corrupt concatenated and timeout PTY tests. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 9 Pthreads queues and shutdown

Prerequisites and modules: M06; M07. Planned time: 11 hours.

Protect queue predicates and wake all waiters. Milestone: B04 producer consumer.

Practice and exit: Shutdown while queue empty and full with no deadlock. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 10 Concurrency debugging

Prerequisites and modules: M07; M07 M15. Planned time: 11 hours.

Diagnose a race and preserve its regression test. Milestone: B04 complete.

Practice and exit: G02 exit and explain thread backtraces without AI. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 11 Cross toolchain and sysroot

Prerequisites and modules: G02; M08. Planned time: 11 hours.

Identify host target and ELF interpreter. Milestone: B05 cross build.

Practice and exit: Produce foreign executable with documented ABI and tool versions. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 12 Application emulation and debugging

Prerequisites and modules: M08; M08 M15. Planned time: 11 hours.

Run cross binary and inspect remote execution. Milestone: B05 complete.

Practice and exit: Explain user-mode emulation limits and target loader errors. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 13 Boot rootfs and Buildroot

Prerequisites and modules: B05; M08 M14. Planned time: 11 hours.

Annotate boot stages and save a defconfig. Milestone: I01 base config.

Practice and exit: Reproduce configuration with the custom external tree. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 14 Custom package and startup

Prerequisites and modules: I01 config; M14. Planned time: 11 hours.

Install and start a service in the appliance. Milestone: I01 package.

Practice and exit: Verify executable config and startup in the generated image. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 15 Persistence and recovery

Prerequisites and modules: I01 package; M14 M16. Planned time: 11 hours.

Preserve valid configuration across restart. Milestone: I01 complete.

Practice and exit: Fault test invalid config and interrupted state write. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 16 Kernel build contexts and modules

Prerequisites and modules: M08 M07; M09. Planned time: 11 hours.

Build against exact kernel source and annotate sleepability. Milestone: I02 build.

Practice and exit: Correct matching-kernel artifact and context review. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 17 Character device I/O

Prerequisites and modules: M09; M10. Planned time: 11 hours.

Define ABI and user-copy behavior. Milestone: I02 read write.

Practice and exit: Normal boundary partial and invalid-buffer test design. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 18 Wait queues and poll

Prerequisites and modules: I02 I/O; M10. Planned time: 11 hours.

Implement blocking and readiness semantics. Milestone: I02 poll.

Practice and exit: Blocking nonblocking and poll tests in matching guest. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 19 Concurrency and lifecycle

Prerequisites and modules: I02 poll; M09 M10. Planned time: 11 hours.

Review open close unload ownership. Milestone: I02 complete.

Practice and exit: No lifecycle or lock defects in adversarial review. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 20 Device tree and platform matching

Prerequisites and modules: M10; M08 M10. Planned time: 11 hours.

Validate binding and compatible matching. Milestone: I03 binding overlay.

Practice and exit: Compile overlay and explain deferred-probe diagnostics. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 21 GPIO interrupts and debounce

Prerequisites and modules: I03 binding; M11. Planned time: 11 hours.

Handle logical polarity and bounded IRQ work. Milestone: I03 driver.

Practice and exit: Object build and specified board event tests recorded separately. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 22 Removal and driver defense

Prerequisites and modules: I03 driver; M09 M10. Planned time: 11 hours.

Prove cleanup order under queued work. Milestone: I03 complete.

Practice and exit: G03 exit including probe-failure and remove-under-load plan. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 23 MCU startup and linker map

Prerequisites and modules: M04 M05; M12. Planned time: 11 hours.

Explain reset vectors data initialization and stack. Milestone: I05 hardware study.

Practice and exit: Annotate selected vendor startup and memory map. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 24 Peripherals and bounded firmware

Prerequisites and modules: M12; M12. Planned time: 11 hours.

Design timers UART ISR and fault evidence. Milestone: I05 state machine.

Practice and exit: Trace startup to one timed event and define hardware checks. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 25 RTOS tasks queues and ownership

Prerequisites and modules: M07 M12; M13. Planned time: 11 hours.

Build the real FreeRTOS host port. Milestone: I05 RTOS build.

Practice and exit: Queue sequencing and correct ownership under task scheduling. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 26 Linux to MCU protocol

Prerequisites and modules: I05 RTOS; M03 M13. Planned time: 11 hours.

Define version sequence integrity and restart handling. Milestone: I05 integration.

Practice and exit: Fragmentation corruption and restart tests. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 27 Watchdog and timing

Prerequisites and modules: I05 protocol; M12 M13. Planned time: 11 hours.

Supervise real progress and measure limits. Milestone: I05 fault handling.

Practice and exit: Stalled task produces the documented watchdog outcome. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 28 RTOS defense and remediation

Prerequisites and modules: I05 faults; M13 M15. Planned time: 11 hours.

Explain priority inversion and host-port limits. Milestone: I05 complete.

Practice and exit: G04 exit plus independent state-machine exercise. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 29 Yocto layers and recipes

Prerequisites and modules: I01 M08; M14. Planned time: 11 hours.

Create own layer and pin release metadata. Milestone: I04 layer.

Practice and exit: Parse metadata and trace recipe to package installation. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 30 Image service and storage

Prerequisites and modules: I04 layer; M14. Planned time: 11 hours.

Integrate telemetry with chosen init system. Milestone: I04 image.

Practice and exit: Boot image and inspect service configuration health. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 31 SDK and reproducible release

Prerequisites and modules: I04 image; M04 M14. Planned time: 11 hours.

Cross build with SDK and preserve inputs. Milestone: I04 complete.

Practice and exit: Clean configuration reproduction and SDK application proof. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 32 IIO and device datasheet

Prerequisites and modules: I03 M10; M11. Planned time: 11 hours.

Decode registers and select subsystem. Milestone: A01 driver.

Practice and exit: Conversion tests including negative and extreme values. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 33 Sensor errors and interface

Prerequisites and modules: A01 conversion; M11 M15. Planned time: 11 hours.

Return bus failures and expose useful channels. Milestone: A01 integration.

Practice and exit: Fault injection with no stale success and clear status. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 34 Power and peripheral lifecycle

Prerequisites and modules: A01 driver; M09 M11. Planned time: 11 hours.

Review suspend resume and resource ownership. Milestone: A01 complete.

Practice and exit: Object build plus separately recorded board validation gates. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 35 Latency experiment design

Prerequisites and modules: B04 M07; M15. Planned time: 11 hours.

Measure wakeup delay with raw data. Milestone: A02 baseline.

Practice and exit: Repeatable workload and distribution with observed limits. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 36 Tracing and measured improvement

Prerequisites and modules: A02 baseline; M15. Planned time: 11 hours.

Discriminate contention I/O and queue causes. Milestone: A02 complete.

Practice and exit: Before-after comparison preserves correctness and workload. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 37 Signing and update threat model

Prerequisites and modules: M14 M07; M16. Planned time: 11 hours.

Verify authenticated payload and compatibility. Milestone: A03 signing.

Practice and exit: Tampered wrong-key and valid signature outcomes. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 38 A/B trial boot and health

Prerequisites and modules: A03 signing; M16. Planned time: 11 hours.

Implement persistent update state transitions. Milestone: A03 state machine.

Practice and exit: Trial commit and rollback behavior with explicit health. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 39 Interrupted updates and recovery

Prerequisites and modules: A03 state; M16 M17. Planned time: 11 hours.

Inject faults at persisted-state boundaries. Milestone: A03 complete.

Practice and exit: Recovery always chooses a verified allowed state. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 40 Production review and gaps

Prerequisites and modules: A01 A02 A03; M16 M17. Planned time: 11 hours.

Audit privileges inventory persistence and recovery. Milestone: Release checklist.

Practice and exit: G05 exit with critical faults corrected before integration. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 41 Gateway requirements and interfaces

Prerequisites and modules: G05; M17. Planned time: 11 hours.

Specify one communication path and acceptance IDs. Milestone: C01 requirements.

Practice and exit: Every mandatory requirement has an observable acceptance test. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 42 Gateway integration

Prerequisites and modules: C01 requirements; M03 M13 M17. Planned time: 11 hours.

Connect telemetry producer parser and storage. Milestone: C01 data path.

Practice and exit: End-to-end ordered data with bounded queues and errors. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 43 Gateway configuration and health

Prerequisites and modules: C01 path; M16 M17. Planned time: 11 hours.

Persist settings and report staleness. Milestone: C01 config health.

Practice and exit: Restart malformed config and stale-link acceptance. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 44 Gateway faults and updates

Prerequisites and modules: C01 health; M15 M17. Planned time: 11 hours.

Exercise drop corruption restart and update status. Milestone: C01 tests.

Practice and exit: All mandatory gateway acceptance and failure tests pass. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 45 Gateway demonstration and defense

Prerequisites and modules: C01 tests; M17. Planned time: 11 hours.

Explain architecture one bug and measured result. Milestone: C01 complete.

Practice and exit: Ten-minute demo plus design modification without reference. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 46 Robot boundaries and simulation

Prerequisites and modules: C01 M13; M05 M13 M17. Planned time: 11 hours.

Separate Linux supervision from MCU control. Milestone: C02 plant model.

Practice and exit: Deterministic state transitions and explicit actuator limits. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 47 Command freshness and diagnostics

Prerequisites and modules: C02 model; M03 M13. Planned time: 11 hours.

Implement session sequence timeout and status. Milestone: C02 link.

Practice and exit: Stale corrupt and duplicate commands cannot extend permission. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 48 Robot fault injection

Prerequisites and modules: C02 link; M15 M17. Planned time: 11 hours.

Test endpoint restart stalls and recovery. Milestone: C02 core complete.

Practice and exit: Mandatory robot acceptance cases pass in simulated time. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 49 AI verification and optional inference

Prerequisites and modules: C02 core; M18. Planned time: 11 hours.

Audit AI suggestions or run optional anomaly pipeline. Milestone: C02 extension or core hardening.

Practice and exit: Independent code explanation plus baseline comparison if AI used. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 50 Robot demonstration and defense

Prerequisites and modules: C02 tests; M17 M18. Planned time: 11 hours.

Defend limits and plan physical validation. Milestone: C02 complete.

Practice and exit: Ten-minute demo and explicit simulated versus physical evidence. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 51 Role-specific final practical

Prerequisites and modules: G01 G02 G03 G04 G05; M01 M18. Planned time: 11 hours.

Implement and debug an unfamiliar variant. Milestone: Final exam.

Practice and exit: Pass correctness reasoning resource concurrency and communication gates. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

### Week 52 Portfolio and interviews

Prerequisites and modules: Final practical; M17 M18. Planned time: 11 hours.

Tailor evidence to one actual JD and run mocks. Milestone: G06 readiness.

Practice and exit: Three role mocks or primary-track deep mock with remediation plan. Use the linked module lab and project runbook; revise the previous week from memory and correct one failed test.

## Reference skills and JD gap closure

The supplied matrix is a reference framework authored for this program. It is not a transcription of an employer's requirements and does not score your proficiency. Every current-evidence and skill-gap field starts at not assessed. The priority column expresses curriculum emphasis only; replace it with the actual must-have or nice-to-have language when a JD is supplied.

### Convert a real JD into evidence requirements

Save the JD text, employer URL and access date. Give each explicit requirement a separate JD identifier and preserve its original wording. Add inferred recommendations in separate rows labelled inferred. Identify the role track, the expected action and depth, then connect the requirement to prerequisites, modules, project code, question IDs and an assessment. Do not infer a requirement for every topic simply because the job title contains embedded.

For example, an explicit requirement to develop Linux I2C drivers maps to M08 through M11 and A01. Evidence should include datasheet interpretation, subsystem choice, matching/binding, error handling and kernel/hardware runtime checks. A host conversion test supports the numerical portion but does not close the entire driver gap. An explicit requirement to maintain a telemetry service maps primarily to M06, M07, M14, M16 and C01; custom kernel development may be optional.

### Prioritize after the diagnostic

First correct critical C and concurrency failures that would invalidate later work. Next prioritize explicit must-haves with no evidence, then high-frequency responsibilities represented in the actual JD, and finally nice-to-haves. Record the impact of each gap in plain language: cannot safely handle partial reads, cannot reproduce a target image, or cannot explain driver teardown. Avoid a numerical confidence score without assessment evidence.

For each gap define one specific completion artifact: a changed source file and regression test, a boot log with version record, a timing report with raw data, or a recorded project defense. Set the next task small enough to finish in one or two study sessions. Reassess the requirement after the task; do not mark a skill mastered because the supplied reference code already exists.

### Record depth honestly

Use not assessed, learning, implemented with assistance, independently demonstrated and needs refresh as learner states. Track execution evidence separately: authored, compile checked, host tested, emulator tested and named-hardware tested. These are different axes. A driver can be authored and object-compiled while the learner still needs to understand it and hardware validation remains pending.

The question bank contains a module column. Filter by that column to choose oral checks, then replace the matrix's module-based question selector with exact Q IDs relevant to the selected employer requirement. An oral answer supplements a practical demonstration; it does not replace one.

## Reference skills summary

| ID | Reference skill | Modules and project evidence |
| --- | --- | --- |
| RF001 | C pointers and object lifetime | M02; B01 |
| RF002 | Arrays strings and bounds | M02 M03; B01 |
| RF003 | Integer promotions overflow and shifts | M02; B01 |
| RF004 | const volatile atomics | M02 M07; B04 |
| RF005 | Alignment padding byte order | M02 M03; B01 B03 |
| RF006 | Ring buffers and state machines | M03; B01 I05 |
| RF007 | Defensive parsing and protocol design | M03 M13; B03 I05 |
| RF008 | Complexity and bounded resources | M03 M07; B01 B04 |
| RF009 | C++ RAII and move ownership | M04; B02 |
| RF010 | Compiler linker ELF and symbols | M01 M04; B05 |
| RF011 | Make CMake and dependency graphs | M01 M04; B01 B05 |
| RF012 | Git review and automation | M01; B01 |
| RF013 | Linux shell permissions and logs | M01 M06; B02 |
| RF014 | Processes threads and scheduling | M05 M07; B02 B04 |
| RF015 | Virtual memory MMU MPU caches | M05; B02 |
| RF016 | POSIX file I/O and descriptor ownership | M06; B02 B03 |
| RF017 | poll epoll deadlines and readiness | M06; B03 I02 |
| RF018 | Signals and graceful shutdown | M06 M07; B04 |
| RF019 | Mutexes conditions and deadlock | M07; B04 |
| RF020 | Pipes sockets and shared memory | M06 M07; C01 |
| RF021 | proc sysfs dev and discovery | M06; B02 I03 |
| RF022 | Cross compilation sysroots ABI | M04 M08; B05 |
| RF023 | Boot chain U-Boot and initramfs | M08; I01 |
| RF024 | Device tree bindings overlays | M08 M10; I03 |
| RF025 | Pinctrl clocks regulators | M08 M11; I03 A01 |
| RF026 | Kernel config modules and build | M09; I02 |
| RF027 | Kernel memory allocation contexts | M09; I02 |
| RF028 | Kernel locks and PREEMPT_RT distinctions | M09; I02 |
| RF029 | IRQ threaded workqueues and timers | M09 M12; I03 I05 |
| RF030 | Driver model probe remove devm | M09 M10; I02 I03 |
| RF031 | Wait queues poll and uaccess | M10; I02 |
| RF032 | ioctl UAPI compatibility | M10; I02 |
| RF033 | RCU publication and reclamation | M09; I02 |
| RF034 | GPIO descriptor and interrupt APIs | M11; I03 |
| RF035 | UART I2C SPI register protocols | M11 M13; B03 A01 I05 |
| RF036 | IIO hwmon and subsystem choice | M11; A01 |
| RF037 | DMA API mapping and ownership | M11; A01 |
| RF038 | Runtime PM suspend resume | M11; A01 |
| RF039 | Startup vectors linker script map | M12; I05 |
| RF040 | Timers PWM ADC watchdog | M12; I05 C02 |
| RF041 | ISR priorities and DMA firmware | M12 M13; I05 |
| RF042 | RTOS tasks queues stack budgets | M13; I05 |
| RF043 | Priority inversion and timing analysis | M13 M15; I05 A02 |
| RF044 | Firmware bootloader and updates | M12 M16; I05 A03 |
| RF045 | Buildroot packages defconfig br2-external | M14; I01 |
| RF046 | Yocto layers recipes images SDK | M14; I04 |
| RF047 | Services configuration persistence | M14 M16; I01 C01 |
| RF048 | Networking and SocketCAN | M06 M11; C01 |
| RF049 | GDB cores strace and fault isolation | M15; B02 B04 |
| RF050 | perf ftrace latency distributions | M15; A02 |
| RF051 | ASan UBSan TSan KASAN KCSAN | M15; B01 B04 I02 |
| RF052 | Unit integration HIL and CI | M17; C01 C02 |
| RF053 | Signing verified boot A/B rollback | M16; A03 |
| RF054 | Least privilege SBOM licenses dependencies | M16; I04 C01 |
| RF055 | AI review and verification workflow | M18; C01 C02 |
| RF056 | Optional embedded inference | M18; C02 |
| RF057 | Architecture and failure tradeoffs | M17; C01 C02 |
| RF058 | Honest project and behavioral defense | M17 M18; C01 C02 |

The workbook and CSV retain the complete 14-field traceability matrix, including source, role, priority, depth, prerequisites, unassessed evidence, assessment and required completion artifact.

# 3 Learning modules

## M01 Linux tools and reproducible engineering

### Objectives and prerequisites

Prerequisites: none.

Find files and logs confidently, compile a program, preserve a change in Git and capture enough context for someone else to reproduce a failure.

### Lesson

A compiler transforms source into object code; a linker resolves references between objects and libraries. An executable can still fail before main if its interpreter or shared libraries are unavailable. Treat source, compiler flags, target architecture and runtime libraries as one reproducibility record. A successful compile proves neither execution nor correct behavior.

On Linux, pathnames and file descriptors are different things. A pathname locates an object when a system call resolves it. A file descriptor is an integer handle in one process. Standard input, output and error normally use descriptors 0, 1 and 2. Redirect stdout into a data file and stderr into a log so machine-readable results remain parseable. A shell exit status of zero conventionally reports success; capture it before another command replaces $?.

Start with pwd, ls -l, rg --files, rg pattern path, head, tail, less, find, stat, df -h, du -sh and ps. chmod changes permission bits; chown changes ownership. Directory execute permission controls traversal. Permissions are checked using credentials and access policy; writing code that works only as root masks integration errors. Learn systemctl status, journalctl -u and dmesg as different observation tools for services and kernel messages; availability and permissions depend on the host.

Use Git to store meaningful changes. Inspect git diff before committing, name the behavioral change, and include its test. A branch is a movable reference, not a copy of every file. git bisect narrows the first bad commit only when the pass/fail test is reliable. Never mark an untestable revision good. For an unfamiliar repository, read its README and build instructions, inspect configuration and run a known test before making changes. [GIT]

Make expresses dependency relationships. If a header affects an object, the dependency graph must record it; otherwise an incremental build can differ from a clean build. A one-line shell script that ignores a failed compile can accidentally run yesterday's binary. Use explicit status checks, quote paths and remove or isolate stale outputs. In Bash, set -eu helps but is not a replacement for understanding command, pipeline and conditional behavior.

### Worked lab and expected behavior

In B01, run make test and inspect the command and test output. Then run the compiler directly on a small independent example:

printf '%s\n' '#include <stdio.h>' \

'int main(void) { puts("ready"); return 0; }' > hello.c

gcc -std=c11 -Wall -Wextra -Wpedantic -Werror -g hello.c -o hello

./hello > result.txt

file hello

readelf -h hello

Expected behavior is one ready line and a host-architecture ELF executable. These are acceptance expectations, not fabricated execution logs. If gcc is missing, fix the tool installation. If the program cannot execute, inspect architecture, loader path, mount permissions and shared libraries before editing C code. Introduce a missing semicolon and confirm the compiler failure prevents a new executable. Then restore and commit the fix plus the observed error explanation.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain why an old executable can run after a failed build. Explain file permissions on directories. Defend how your commit can be reproduced. Interview bank: build/tooling questions linked to M01.

### Mastery checklist

Reproduce a clean build from source.

Separate program data from error logs.

Explain one compiler and one runtime failure.

Review a diff before committing.

### Primary references

[GIT] Git official tutorial. [GCC-WARN] GCC warning options. [LD] GNU linker scripts.

### M01 independent exercise

Timebox 45 minutes. Make a small repository with a program that prints a supplied integer. Reject invalid input. Deliver source, build command, three tests, a useful commit message, tool versions and a log showing a clean rebuild. Do not use AI for the first attempt.

## M02 C memory ownership and defined behavior

### Objectives and prerequisites

Prerequisites: M01.

Track object lifetime and bounds, explain pointer declarations, distinguish language behavior categories and choose qualifiers correctly.

### Lesson

A C pointer carries an address-like value and a type; it does not carry the allocation length or proof that an object is still alive. Define an ownership rule at each API: the caller borrows a buffer for the call, transfers it, or receives a new allocation to release. A pointer to an automatic local object becomes invalid when that object's lifetime ends. Copying the pointer does not extend the lifetime. Null checks cannot detect a dangling non-null pointer.

Arrays and pointers interact but are not the same object. In most expressions an array converts to a pointer to its first element, which loses its size. A parameter declared int a[] is adjusted to a pointer parameter. Pass the element count explicitly, and calculate allocation sizes with a checked multiplication before calling malloc. For an element size s, reject count greater than SIZE_MAX / s. Use sizeof *p so the allocation stays aligned with the pointed-to type.

const int *p prevents modification through p; int *const p fixes the pointer variable. Neither implies a particular memory region. volatile concerns accesses observable to the implementation and is often relevant to vendor-defined MMIO. It does not make a read-modify-write atomic or establish ordering of ordinary shared objects. Use C atomics or a mutex for inter-thread communication. In Linux kernel code use the kernel's accessors and synchronization contracts. [GCC-VOL] [K-VOL]

Unsigned arithmetic wraps modulo its range, which can still be a security or logic defect. Signed overflow has undefined behavior. Integer promotions can surprise bit manipulation: use an unsigned type of known width, validate shift counts and apply a mask deliberately. Do not cast a received byte buffer to a packed structure and assume correct alignment, endianness or ABI layout. Decode fields byte by byte or copy into a suitable object with an explicitly specified representation.

Undefined behavior means the language gives no required result. Unspecified behavior permits more than one allowed choice without requiring documentation. Implementation-defined behavior requires the implementation to document its choice. Do not invent one output for an expression with unsequenced modifications, an invalid pointer access or signed overflow. For an interview, name the violated rule first. The public C11 committee draft is a reference for these foundational contracts, not a substitute for checking the selected compiler dialect. [C11]

Memory corruption often appears far from its origin. A write one byte beyond a buffer can damage a neighbor; the resulting crash may occur during a later free. A sanitizer can expose a failing execution, but a clean test covers only the paths exercised. Pair bounds reasoning with boundary tests, static diagnostics and review. [GCC13]

### Worked lab and expected behavior

Use B01's parser and ring-buffer tests as the worked example. Build the normal target first, then use its sanitizer target if available. For a self-contained ownership example:

int read_u16_be(const unsigned char *p, size_t n,

unsigned *value)

{

if (!p || !value || n < 2) return -1;

*value = ((unsigned)p[0] << 8) | p[1];

return 0;

}

The caller owns p and value; the function borrows them only during the call. Inputs {0x01,0x02} decode to 258. A one-byte input returns an error without reading past its end. If a sanitizer reports a stack lifetime error, compare where the object is created, returned and accessed. Do not suppress the report before understanding ownership.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain array decay, const placement, padding and storage duration using your code. Classify three invalid-output questions before predicting values. Interview bank: C memory, UB and synchronization.

### Mastery checklist

State ownership for every pointer.

Reject invalid lengths before access.

Explain qualifiers without claiming synchronization.

Reproduce and fix a memory error independently.

### Primary references

[C11] WG14 N1570 public draft. [GCC13] GCC 13.3 instrumentation. [GCC-VOL] GCC volatile access. [K-VOL] Linux volatile guidance.

### M02 independent exercise

Timebox 60 minutes. Write a bounded buffer append API with capacity, used length and input length. Preserve the existing buffer on rejected input. Test exact fit, one-byte overflow, empty input, size overflow and overlapping source/destination. State whether overlap is supported. Then explain why volatile int counter; counter++ is not thread-safe.

## M03 Data structures and defensive protocols

### Objectives and prerequisites

Prerequisites: M02.

Implement a bounded ring buffer and state machine, define a binary wire format and recover from corrupted or incomplete input.

### Lesson

A ring buffer reuses a fixed storage array. Choose an invariant before coding: either store a count, reserve one slot, or use monotonic counters whose wrap arithmetic is explicitly bounded. A count-based single-threaded design maintains 0 <= count <= capacity. Push writes at the tail only when count < capacity; pop reads at the head only when count > 0. Each operation is constant time. A concurrent ring needs an additional synchronization design; the data structure alone is not thread-safe.

Prefer a small state machine when input arrives over time. A parser can search for a prefix, collect a header, validate a length, collect a payload, and verify integrity. State transitions must define what happens on an invalid byte, a timeout and reset. The receiving transport may split one frame across many reads or combine many frames into one read. A packet boundary in your application is not a UART or TCP read boundary.

Specify the protocol before implementing it: framing, version, message type, sequence, bounded payload length, byte order, integrity algorithm and the exact bytes covered by the integrity check. A cyclic redundancy check detects accidental corruption; it does not authenticate the sender. If replay or malicious commands matter, add an appropriate authenticated channel or message authentication and a key-management design.

An API should distinguish incomplete input from invalid input. Both may consume bytes, but their recovery rules differ. Reject a length before allocating or indexing. Resynchronize deliberately after a bad checksum so the next valid frame can be found. Counters for invalid length, integrity failure, unsupported version and timeout turn field failures into observable behavior.

For a command such as set-speed, make retries idempotent or include a sequence/transaction identifier. An acknowledgement states what was accepted, not necessarily what was physically achieved. Add a timestamp or age limit where stale commands are unsafe. Receiver-side limits remain active when the upstream process stalls. These are original protocol design choices for the exercises; they must be validated against the actual product requirements.

### Worked lab and expected behavior

Build and test B01 and B03. Inspect the protocol specification in B03 rather than assuming its field order from this lesson. Use its pseudo-terminal test to send a valid frame one byte at a time, then a corrupt frame followed by a valid frame. Acceptance requires the valid frame to survive fragmentation, corruption to be rejected and synchronization to recover without a process restart.

For a count-based ring with capacity 3, push 10, 20, 30; reject 40; pop 10 and 20; push 40 and 50; then pop 30, 40, 50. This concrete sequence exercises wraparound and a full/empty distinction. If the final order differs, inspect the update order of head, tail and count. If a parser hangs after corrupt input, log state transitions and consumed-byte counts, not only the final error.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Why is a checksum insufficient for security? How do you distinguish duplicate commands from retransmissions? What makes your ring full? Interview bank: coding, protocol debugging and system design.

### Mastery checklist

State ring invariants before implementation.

Test every split position of a frame.

Recover after corruption.

Bound memory, retries and waiting.

### Primary references

[READ] Linux read semantics. [WRITE] Linux write semantics. B01 and B03 contain original protocol contracts and tests.

### M03 independent exercise

Timebox 75 minutes. Add an unsupported-version counter and a maximum inter-byte interval to a parser. Feed random fragments, a bad length, a truncated frame, duplicate sequence numbers and two concatenated valid frames. Deliver transition rules, tests and a memory bound. Do not copy the solution implementation.

## M04 Build systems linking and C++ ownership

### Objectives and prerequisites

Prerequisites: M02.

Inspect ELF objects, diagnose link/runtime mismatches and use a small RAII wrapper without copying an owning resource.

### Lesson

Preprocessing expands includes and macros; compilation produces assembly; assembly produces object files; linking chooses definitions and assigns final locations. Use gcc -E, gcc -S, gcc -c, nm, readelf and objdump to see the stages. A declaration permits a reference; a definition supplies storage or executable code. Multiple definitions, missing definitions and an incompatible calling convention are different problems.

The application binary interface defines details such as argument passing, register usage, data layout and symbol conventions. A C function called from C++ may need extern "C" linkage. A library built for another CPU, incompatible floating-point ABI or libc environment is not repaired by adding an include directory. Inspect the architecture, interpreter and required symbols. A sysroot groups target headers and libraries for cross compilation; it is distinct from the host compiler's own runtime. [CMAKE]

A linker script places sections into memory regions. Bare-metal firmware needs addresses that agree with the chip's flash/RAM map, startup copy/zero code and vector placement. An initialized variable can have a load image in flash and a runtime address in RAM. .bss usually consumes runtime zero-initialized storage without storing all those zero bytes in the load image. Read the map file to establish memory usage; binary file size alone is not the RAM footprint. [LD]

In C++, resource acquisition is initialization, or RAII, ties cleanup to object lifetime. An owning file-descriptor wrapper closes in its destructor and disables accidental copying; moving transfers ownership and invalidates the old owner. A borrowed integer descriptor is not an owner. Keep destructor behavior nonthrowing and define whether a close failure can be reported separately. std::unique_ptr expresses exclusive memory ownership; shared ownership can hide cycles and unclear shutdown. Optional C++ depth includes templates, standard containers and exception policy when required by the JD. [CPP]

Make is adequate for a small C reference project. CMake describes targets and usage requirements, while Yocto/Buildroot assemble systems and dependencies at another level. They solve different scopes. Include compiler flags with the target, keep tests callable, and verify one clean build to detect undeclared dependencies. Reproducibility additionally needs pinned inputs, timestamps/tool behavior, paths and controlled external downloads.

### Worked lab and expected behavior

Run readelf -h and nm on B01's build/test_util executable and B05's resulting executable. To inspect a separate object, compile a source file with gcc -c and inspect the resulting .o. Inspect a host executable's interpreter with readelf -l. Compare an undefined reference at link time with a missing shared library at runtime. For C++, attempt the independent exercise first, then run make test in this module's lab directory to compile and execute fd_owner.cpp. Acceptance is that each successful open has one owner and one close, including early error paths.

Troubleshoot by stage: a missing header is a compile search-path problem; an undefined reference requires the intended definition/library and correct linkage; Exec format error requires target/loader inspection. Do not randomly add library flags until a build happens to pass.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain static versus dynamic linking, ABI versus API, and unique versus shared ownership. Interview bank: linking, C++ and cross-platform errors.

### Mastery checklist

Diagnose a failure by build stage.

Inspect ELF architecture and symbols.

Explain LMA versus VMA in a firmware image.

Demonstrate one resource owner through a move.

### Primary references

[LD] GNU ld linker scripts. [CMAKE] CMake 3.28 toolchain manual. [CPP] C++ Core Guidelines. [CLOSE] Linux close semantics.

### M04 independent exercise

Timebox 60 minutes. Implement a move-only C++17 owner for a POSIX file descriptor. Add get(), release() and a destructor. Test moving, releasing, invalid descriptor destruction and scope exit. Separately explain why memcpy of a live std::string object is invalid ownership handling.

## M05 Processor and operating system foundations

### Objectives and prerequisites

Prerequisites: M02.

Explain virtual memory, privilege, scheduling and device access, and separate application processors from microcontrollers.

### Lesson

An application processor normally runs a rich OS with a memory management unit, or MMU, translating virtual addresses into physical mappings. A process has an address-space view, credentials and resources. Threads in one process share that view but have separate execution contexts and stacks. A context switch changes the executing task; a system call crosses a privilege boundary without necessarily switching to another process.

A microcontroller often uses a simpler memory map and a memory protection unit, or MPU, where present. An MPU controls permissions over regions rather than providing the general virtual-address translation expected by Linux applications. Firmware may execute directly from flash and manage peripherals itself. A successful Linux application build says nothing about the startup vector or linker script needed for an MCU image.

The CPU can reorder operations subject to architectural rules; compilers can transform code subject to language rules. A cache improves access latency but introduces visibility and ownership questions. Coherence and ordering are different: seeing a consistent value for one location does not define the order in which different locations become visible. Locks and atomic operations provide particular synchronization contracts; a compiler barrier alone does not necessarily constrain the CPU.

Memory-mapped input/output, or MMIO, assigns device registers to an address range. Reads can have side effects and writes can clear bits or start hardware. Do not use ordinary memory assumptions such as speculative reads, arbitrary-width access or read-modify-write without the register contract. Linux drivers use MMIO accessors, mappings and the DMA API. A DMA address is the address a device uses; it is not generally a CPU pointer or physical address. [DMA]

Interrupts report an event asynchronously. Keep the hard interrupt work bounded and defer sleepable operations to an appropriate context. Interrupt priority, masking and dispatch latency depend on the architecture and OS configuration. A high task priority cannot overcome a disabled-interrupt interval, a blocking bus transaction or an invalid control design.

The scheduler chooses runnable work. Throughput, fairness and deadlines are different objectives. A regular Linux scheduling observation is not a hard real-time guarantee. To claim a deadline, define the worst acceptable delay, the workload, interrupts, competing tasks and measurement method, then test the appropriate target and analyze bounds. An average latency conceals a long tail.

### Worked lab and expected behavior

Run B02 on your process and another accessible process. Compare thread count and resident memory; inspect mappings separately in /proc/PID/maps; do not interpret virtual size as physical RAM consumption. Inspect /proc/self/maps and explain executable, stack, library and anonymous mappings. Use readelf -S to relate executable sections to runtime mappings without assuming a one-to-one layout.

Draw the path from a user buffer through a system call, driver and device. Annotate which address is a user pointer, CPU virtual address and DMA address. If a tool's memory numbers disagree, identify units, shared pages, sampling time and permissions before concluding one tool is broken.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain process versus thread, MMU versus MPU and cache coherence versus ordering. Interview bank: architecture, OS and timing.

### Mastery checklist

Trace one operation across privilege boundaries.

Distinguish CPU and device addresses.

State the limits of a host timing experiment.

Explain application processor versus MCU startup.

### Primary references

[DMA] Linux DMA mapping guide. [CMSIS] Arm CMSIS startup. [NVIC] Arm NVIC documentation. [KLOCK] Linux lock types.

### M05 independent exercise

Timebox 45 minutes. Explain a hypothetical data acquisition system with Linux reading an MCU over UART. Assign sampling, hard limits, timestamps, buffering and UI responsibilities. Identify which claims require real board measurements. Deliver one architecture diagram and a timing budget with explicitly estimated numbers.

## M06 Linux I/O processes and event loops

### Objectives and prerequisites

Prerequisites: M01 M02 M05.

Write correct partial-I/O loops, manage child processes and implement bounded timeouts and graceful termination.

### Lesson

read asks for up to a count of bytes. A positive return is progress, which may be smaller than requested. Zero means end-of-file for ordinary files and streams in their normal modes, but terminal timeout modes need their own contract. A negative result reports an error through errno. write can also complete partially. Preserve your offset, handle interruption and integrate nonblocking retry with readiness rather than spinning. [READ] [WRITE]

A read-ready event means an operation should not block according to that interface; it does not promise a complete application message. poll scans a descriptor set; epoll maintains an interest set and returns events. With edge-triggered nonblocking epoll, drain the operation until EAGAIN or a terminal condition so unread work does not remain without a new edge. A hangup can coexist with buffered readable data. [EPOLL]

Use a monotonic clock for elapsed-time deadlines. Recompute remaining time after EINTR, otherwise repeated signals can extend a timeout indefinitely. Decide what your API promises after partial progress and failure. A caller often needs both the number of bytes transferred and the reason it stopped. A blocking send-all helper without a deadline can hang a control path forever.

fork creates a child execution context and exec replaces the process image; the child does not become a separate program until exec succeeds. File descriptors can be inherited. Use close-on-exec where appropriate, close unused pipe ends, and reap child status with waitpid to avoid zombies. Shared file-description offsets can explain surprising I/O after duplication or fork.

Signals interrupt ordinary control flow. Most library calls, including typical formatted logging, are not safe inside an asynchronous handler. A simple handler can set a suitable flag or write to a self-pipe under the correct restrictions; a design can also consume blocked signals synchronously in a dedicated thread. Publish a shutdown request, stop accepting work, wake blocked workers, drain or discard according to policy, join and release resources. [SIGNAL]

/proc exposes process and kernel state, /sys describes kernel objects and attributes, and /dev contains device interfaces. They have different contracts. Device names can change; discover a device by stable attributes or managed rules rather than assuming that the first USB serial adapter is always the desired one. udev normally manages device events on distributions that use it; a small BusyBox image may instead use devtmpfs and a different policy.

### Worked lab and expected behavior

Use B03's pseudo-terminal tests. Verify that a fragment sent before a timeout is preserved or discarded according to the published transaction policy. Confirm that malformed input does not leave the terminal settings changed after exit. For B02, pass a nonexistent PID and verify a clear error return rather than a crash. Try a process that disappears during inspection and document that snapshots are inherently racy.

To debug a blocked read, inspect which descriptor is open, whether its peer is alive, whether termios is in canonical mode and whether your timeout uses the intended clock. Use strace when installed to distinguish no data from an application that never issued a read.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain why one read cannot be assumed to equal one message. Compare epoll trigger modes and explain a signal-safe shutdown. Interview bank: POSIX, debugging and IPC.

### Mastery checklist

Demonstrate fragmented and concatenated messages.

Bound blocking operations.

Distinguish EOF, timeout and error.

Reap children and close unneeded pipe ends.

### Primary references

[READ] read manual. [WRITE] write manual. [EPOLL] epoll_ctl manual. [SIGNAL] signal-safety manual. [PROC] proc_pid_stat manual.

### M06 independent exercise

Timebox 90 minutes. Write a pipe reader that accepts multiple newline-delimited records, handles records split across reads, limits a record to 256 bytes and exits cleanly when the writer closes. Demonstrate a partial final record, an oversized record and interruption. Specify whether the partial final record is accepted.

## M07 Concurrency synchronization and IPC

### Objectives and prerequisites

Prerequisites: M06.

Protect invariants, use condition variables correctly, reason about deadlock and choose an IPC mechanism for a measured need.

### Lesson

A data race is not merely a bad interleaving observed in a test. In C, conflicting unsynchronized accesses can invalidate language-level reasoning. Protect a shared invariant, not just isolated assignment statements. A queue's count, indices and contents must agree; locking only count leaves the queue inconsistent.

A mutex provides exclusive access under its contract. A condition variable waits for a predicate protected by a mutex. The wait atomically releases that mutex and blocks, then reacquires it before returning. Recheck the predicate in a loop because wakeups can be spurious and another consumer can take the item first. A notification is not a stored unit of data; the predicate records the state. [COND]

For a bounded producer-consumer queue, the producer waits while full and not stopping; the consumer waits while empty and not stopping. Shutdown changes a state variable while holding the mutex and wakes every class of waiter. Decide whether consumers drain queued items or exit immediately. Join threads before freeing queue storage. A thread that is asleep forever is still a resource-lifetime bug.

Deadlock prevention begins with a global lock order or with a design that avoids holding multiple locks. Logging and callbacks can acquire hidden locks. Calling an unknown callback while holding an internal lock exports a dangerous lock-order dependency. Keep critical sections small, but do not move an operation outside protection if it needs the invariant. Priority inheritance may reduce priority inversion for supported mutexes; it cannot fix a circular wait or an unbounded critical section.

Atomics are appropriate when a well-defined atomic invariant is simpler than a mutex. For a publication flag, release/acquire can connect initialization with observation when used correctly. Relaxed operations maintain atomicity of the atomic object but do not publish unrelated data. Learning to recognize these contracts matters before writing a lock-free queue. Use a mutex-based queue first and measure before replacing it.

Pipes provide streams, Unix sockets provide local communication with useful connection semantics, shared memory gives direct access but still requires synchronization and lifetime rules, and message queues provide bounded messages under their particular APIs. Serialization costs and failure isolation can be worth more than avoiding one copy. For a small telemetry application, a Unix socket with explicit framing often gives a clearer ownership boundary than shared-memory structures with process-shared synchronization.

### Worked lab and expected behavior

Run B04's multithreaded logger tests. Change producer rate and queue capacity. Record produced, consumed and dropped counts according to the chosen policy; the counts must reconcile. Request shutdown while empty and while full. Expected acceptance is no deadlock, no use-after-free, bounded memory and the documented drain policy.

If ThreadSanitizer is available and supported, run it in a separate build; it cannot generally be combined with AddressSanitizer in one executable. A tool failure caused by host memory mapping is not a clean race result. Diagnose a hang with all thread backtraces and identify the predicate each blocked thread expects. [GCC13]

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain why volatile cannot repair the queue. Draw one deadlock cycle and remove it. Interview bank: concurrency and scenario questions.

### Mastery checklist

Name the protected invariant.

Demonstrate shutdown in all waiting states.

Explain spurious wakeup and lost notification.

Separate atomicity, ordering and lifetime.

### Primary references

[COND] POSIX condition-wait manual. [GCC13] GCC sanitizer options. [KLOCK] Kernel lock context comparison.

### M07 independent exercise

Timebox 90 minutes. Add a second consumer to a bounded queue. Reproduce why replacing while(predicate false) with if(predicate false) is invalid. Add an orderly stop that wakes blocked producers and consumers. Deliver a lock-order note and tests for queue sizes 1 and 2.

## M08 Cross compilation boot and device tree

### Objectives and prerequisites

Prerequisites: M04 M05.

Build for a foreign target, identify boot-stage evidence and describe hardware with validated bindings.

### Lesson

A cross compiler runs on the host and generates code for a target. Its target triple, headers, libraries and linker need to agree. A sysroot is a target filesystem view used during development; it is not automatically a bootable image. B05 deliberately starts with Linux user-mode QEMU to isolate application and loader questions before I01 introduces a whole guest machine. [CMAKE] [QEMU-USER]

Boot is platform-specific. A common application-processor chain starts in immutable boot ROM, loads early firmware that initializes memory, then a bootloader, kernel and an initial userspace program. Some systems use UEFI; others use U-Boot and vendor stages. Do not draw one universal sequence and call it every ARM board. Observe serial output at each transition and identify the first missing milestone. Kernel logs cannot diagnose a failure before the kernel began executing.

An initramfs is unpacked into an early root filesystem in memory. It can supply /init and discover or mount the final root. A persistent root filesystem holds userspace and configuration according to the image design. A kernel panic about no working init differs from inability to mount root: the former can involve a missing executable, interpreter or libraries, while the latter can involve storage drivers, filesystem support or a wrong root argument.

Device tree describes hardware through nodes and properties governed by bindings. A compatible string allows matching a device description to driver support. Addresses, interrupt specifiers, clocks, regulators and pins have parent-specific meanings. A device-tree blob is data, not executable driver code. A correct-looking node cannot compensate for a missing driver or disabled clock. Use the kernel's schema validation and the selected board's actual source. [DT] [DTSCHEMA]

An overlay changes a base tree and depends on symbols, target labels and firmware support. Its use on one board release does not prove that the same overlay works elsewhere. GPIO descriptors express logical state and active polarity; pin control configures electrical/multiplexing characteristics separately. Record the exact base device tree, board revision and overlay with the driver evidence.

A board support package combines bootloader, kernel, device descriptions, firmware, toolchain/image integration and patches for a supported platform. Count the maintenance burden of downstream patches. A board booting once establishes a milestone, but repeated cold boot, shutdown, peripheral recovery and update behavior still need verification.

### Worked lab and expected behavior

In B05 compare file and readelf -h for the host and AArch64 binaries. In I01 capture a full-system guest serial boot log and identify the kernel start and init/service start. In I03 compile the supplied overlay against the documented tree and run schema checks as instructed. Emulator and board steps are pending unless your own log demonstrates them.

If a foreign dynamically linked binary says it cannot find its loader, inspect the ELF interpreter and supply the intended target sysroot; do not copy host libraries. If a driver never probes, examine node status, compatible matching, kernel configuration and deferred dependencies before changing interrupt code.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain sysroot versus rootfs, initramfs versus persistent root, and device tree versus driver. Interview bank: boot, BSP and cross-compilation.

### Mastery checklist

Identify host and target for every tool.

Capture and annotate boot stages.

Validate a device-tree binding.

Explain which dependencies can defer probe.

### Primary references

[CMAKE] CMake toolchains. [QEMU-USER] QEMU userspace. [QEMU-VIRT] Generic virt machine. [DT] Linux device tree. [DTSCHEMA] Binding schemas.

### M08 independent exercise

Timebox 60 minutes. Given an executable that exists but reports No such file or directory, diagnose a missing ELF interpreter. Then write a boot-failure decision note distinguishing no serial output, bootloader failure, root-mount panic and service failure. Include the next observation for each.

## M09 Kernel execution contexts and resource lifetimes

### Objectives and prerequisites

Prerequisites: M07 M08.

Choose context-appropriate allocation and locks, reason about deferred work and prevent teardown races.

### Lesson

Kernel code can run in process context, interrupt-related contexts and deferred execution. The permitted operations depend on the context and configuration. A sleepable operation is forbidden when the context cannot schedule. GFP_KERNEL can reclaim and sleep; an atomic allocation flag changes behavior but does not make a poorly designed interrupt path reliable. Preallocate or defer work when the design permits it. kmalloc and vmalloc have different contiguity properties; neither authorizes bypassing the DMA API. [KALLOC]

For the baseline non-PREEMPT_RT driver examples, a spinlock protects short state updates where sleeping is not allowed, while a mutex protects sleepable task-context work. PREEMPT_RT changes spinlock_t semantics; raw_spinlock_t retains strict spinning behavior. State the selected configuration before asserting generic lock behavior. Disabling local interrupts alone does not serialize another CPU. [KLOCK]

A workqueue moves work to a worker context, but queuing it does not automatically solve lifetime. A device can be removed while work is pending or running. A typical teardown plan first prevents new work and user access, then disables or synchronizes event sources, cancels/drains deferred work, unregisters interfaces and releases storage in the required order. The exact order depends on which callbacks may requeue work and which resources they need. [WORK]

Managed device resources simplify probe failure and detach cleanup, but they do not grant indefinite validity to pointers stored in open files or asynchronous callbacks. Review every long-lived reference and its owner. Probe often builds state before publishing an interface; a partially initialized object must not be visible to users. Error paths should unwind only resources that were actually acquired. [DRIVER]

Read-copy-update, or RCU, can allow efficient concurrent readers under a particular read-side discipline. A writer publishes a replacement and waits for an appropriate grace period before freeing an old object that readers may still see. RCU does not automatically serialize writers or make arbitrary object mutation safe. At junior level, be able to explain publication and delayed reclamation; implement it only where the data structure and subsystem justify it. [RCU]

Modules execute with kernel privilege. A bad pointer can crash the system and a subtle race can corrupt unrelated data. Use a disposable matching-kernel VM, preserve logs and enable suitable debugging options. A successful object compile checks syntax and some API usage, not locking, teardown or real peripheral behavior.

### Worked lab and expected behavior

Study I02's read/write and wait paths. Mark every shared field, its lock, and every point that can sleep. Then draw a lifetime timeline from module load through open, concurrent I/O, close and unload. If a file operation can outlive device removal, identify the reference that keeps its state alive or the rule preventing removal.

Use kernel W=1 build checks and sparse/checkpatch where available; record which tools actually ran. In a guest, run simultaneous readers/writers and unload only after descriptors are closed. Debugging extensions include lockdep and KASAN, selected for the kernel and target. A compiler warning is resolved by checking the contract, not by casting away the type mismatch.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Why can GFP_KERNEL sleep? Does devm solve every use-after-free? What changes under PREEMPT_RT? Interview bank: kernel locking and lifecycle.

### Mastery checklist

Annotate sleepable and atomic paths.

Identify every asynchronous owner.

Design teardown before initialization.

Separate build evidence from runtime evidence.

### Primary references

[KALLOC] Kernel allocation guide. [KLOCK] Kernel locking. [WORK] Workqueues. [DRIVER] Driver model. [RCU] RCU fundamentals.

### M09 independent exercise

Timebox 75 minutes. Review a design whose IRQ queues work using a device pointer, while remove frees the pointer immediately. Write a teardown order and test plan, and explain where devm alone is insufficient.

## M10 Character and platform drivers

### Objectives and prerequisites

Prerequisites: M09.

Implement an educational character interface, define blocking and poll semantics, and choose a standard subsystem for real hardware.

### Lesson

A character driver is useful for learning how file operations reach kernel code. Define its user ABI first: byte stream or records, blocking behavior, nonblocking EAGAIN, EOF, short operations, errors and concurrency. copy_to_user and copy_from_user report bytes not copied, not the usual negative errno convention. They can fault; do not use them under a context or lock that forbids sleeping. Handle partial transfer deliberately rather than silently losing queued data. [UACCESS]

A wait queue waits for a predicate such as data available or shutdown. Registration, state changes and wakeups must form a race-free protocol. After waking, another reader may have consumed the data, so recheck under the correct synchronization. poll registers interest using poll_wait and reports the currently satisfied conditions. Readiness may change before the caller performs I/O. A correct nonblocking operation remains necessary even when poll exists.

File-private state can hold per-open data, but shared device state needs a lifetime rule. .owner = THIS_MODULE helps protect module code while open file operations use it; it is not a complete hot-unplug memory ownership design. If the exercise intentionally permits unload only after close, state that scope and test it. Production removable devices need a more complete reference-counted lifetime.

Platform drivers bind devices that are not discoverable through a self-enumerating bus such as PCI. Firmware description can provide resources, and probe resolves them through device-managed or explicit APIs. A successful probe publishes a usable interface; a failed probe must unwind cleanly. The .remove callback signature and helper APIs can change across kernel releases, so build against the frozen source and review exact declarations before migrating. [DRIVER]

An ioctl should be used only when existing file operations and subsystem interfaces do not fit. Define fixed-width fields, alignment, compatibility, size and direction consistently; never expose a raw kernel pointer or a layout with implicit architecture-dependent padding. A stable user ABI costs maintenance. Standard subsystems such as input, hwmon, IIO and GPIO already supply discovery and user interfaces. Use the existing subsystem for a real device when its semantics fit rather than inventing a custom character device for every sensor. [IIO]

### Worked lab and expected behavior

Build the I02 driver against its named kernel tree and its user-space client with the host compiler. Run in the matching disposable guest. Exercise empty blocking read, empty nonblocking read, full write, short user buffer, concurrent users and poll wakeup. A host-only client compile cannot validate any kernel behavior.

In I03 inspect matching compatible strings and GPIO descriptor names across binding, overlay and driver. If probe returns an error, retain the errno and distinguish an unavailable dependency from invalid configuration. Check logical active state and debounce policy before assuming every interrupt represents a physical event.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Why do uaccess helpers return a residual count? How does poll avoid lost wakeups? When is a custom ioctl justified? Interview bank: driver implementation and design.

### Mastery checklist

Publish an explicit user ABI.

Test blocking/nonblocking behavior.

Review user-copy failure handling.

Explain subsystem selection and teardown.

### Primary references

[UACCESS] Kernel uaccess API. [DRIVER] Linux driver model. [GPIO] GPIO consumers. [IIO] Industrial I/O subsystem.

### M10 independent exercise

Timebox 90 minutes. Review an educational FIFO driver that removes bytes before copy_to_user and returns EFAULT when the copy partly fails. Explain its data-loss behavior, choose documented semantics and propose a safe test. Then define poll results for empty/full/shutdown states.

## M11 Peripheral buses sensor subsystems DMA and power

### Objectives and prerequisites

Prerequisites: M10.

Read a datasheet into an interface contract, select a subsystem and explain data, DMA and power lifetimes.

### Lesson

UART is an asynchronous serial transport with configured baud, data and stop bits; it does not define application framing. I2C uses addressed transactions and requires attention to register conventions and bus errors. SPI usually has a controller, chip select, clock polarity/phase and device-specific transfers; there is no universal register command format. A driver must honor the actual datasheet, not the bus name alone.

Translate one sensor register completely: address, reset value, read/write behavior, byte order, signedness, scale, reserved bits and valid operating modes. A signed sample often needs sign extension before scaling. A wrong scale can produce plausible values, so test known negative, zero and full-scale bit patterns independently of the bus. A communication success does not mean the device is configured or settled.

Linux IIO provides a model for measurement channels, raw values, scales, buffers and triggers. hwmon often fits system health sensors; input fits user input events. The A01 driver is an educational subsystem exercise whose selected device and upstream alternative are described explicitly. Do not bind an educational replacement and an upstream driver to the same device simultaneously. [IIO]

Regmap can organize register access and caching, but cached values must respect volatile/precious registers and power transitions. A failed bus transfer should preserve state invariants and return an interpretable error; endless retries can block a shutdown or control path. Define a retry budget and recovery mechanism such as resetting a peripheral only when the hardware and ownership contract permit it.

For direct memory access, configure the device's DMA capabilities and use the DMA API to map buffers. Coherent allocations still require ordering and ownership discipline; streaming mappings require the appropriate synchronization transitions. Check mapping failure, use the required direction and never let the CPU or device use a buffer after its lifetime or mapping ends. A CPU virtual pointer is not the value to write into a device's DMA address register. [DMA]

Runtime power management suspends an idle device independently of whole-system sleep. A transaction must hold a valid usage reference and handle resume failure. On suspend, decide what happens to interrupts, in-flight transfers, cached registers and wakeup sources. On resume, restore the hardware state before reporting data valid. System suspend/resume adds coordination with the broader platform. [RPM]

### Worked lab and expected behavior

Use A01's conversion tests before connecting the sensor. Read the selected datasheet and annotate the corresponding source masks and scale. On hardware, check identity/mode and read the interface through its subsystem, then unplug or force a bus error under the project's procedure. Acceptance includes valid conversion, error propagation and successful recovery without stale data being reported as fresh.

For a DMA design review, draw CPU owns, device owns and completion transitions. Label each mapping/synchronization operation and the point where memory may be freed. For power management, inject a resume failure and confirm no register read occurs against a suspended device.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

When is hwmon preferable to IIO? What is a DMA mapping? Why must power resume precede register access? Interview bank: buses, drivers and power.

### Mastery checklist

Decode signed values without relying on a packed cast.

Identify standard subsystem fit.

Explain DMA ownership transitions.

Test bus and resume failures.

### Primary references

[IIO] Linux IIO. [DMA] Linux DMA mapping. [RPM] Runtime PM. [GPIO] GPIO consumer rules. A01 provides its exact device datasheet.

### M11 independent exercise

Timebox 75 minutes. Convert 12-bit two's-complement samples stored left-aligned in a 16-bit register to integer milliunits at 0.0625 unit per LSB. Test +25, -1, zero and the negative extreme. Then list the extra evidence needed to claim an I2C driver works on a board.

## M12 Bare metal startup peripherals and faults

### Objectives and prerequisites

Prerequisites: M04 M05 M11.

Explain reset-to-main, link memory correctly and design bounded peripheral handling and watchdog supervision.

### Lesson

Before main runs, startup establishes the processor's initial execution environment. On a Cortex-M system, the vector table supplies the initial stack pointer and exception entry addresses. Startup and the C runtime initialize data and zero-initialized storage, and board-specific initialization sets clocks and other essentials. The exact sequence is supplied by the selected device package; inspect it rather than replacing it with a generic internet file. [CMSIS]

A linker script and startup code form one contract. Symbols delimit data to copy from flash into RAM and bss to clear. Interrupt vectors must be placed where the processor expects them or where relocation config points. The stack grows into real memory and may collide with another region. Examine map output, reserve alignment correctly and measure stack high-water marks with an appropriate method; a successful link alone does not show enough runtime stack.

Configure a peripheral in a deliberate order: enable its clock, configure pins, reset/configure the peripheral, clear relevant pending status, prepare buffers/interrupt routing and then enable activity. The device reference manual decides the details. A timer can schedule periodic work without a blocking delay loop. PWM duty is not motor speed by itself; load, supply, feedback and control behavior matter. ADC readings need reference, calibration, sampling-time and input-network context.

An interrupt service routine, or ISR, should capture enough state to avoid data loss, acknowledge the source according to hardware rules and defer long processing. The nested vectored interrupt controller uses architecture-specific priority numbering and grouping; do not equate numerical priority values with intuitive high/low priority without reading the API contract. [NVIC]

A watchdog should be refreshed only after essential tasks show progress. A timer ISR that feeds it regardless of failed application work defeats its purpose. Define startup grace periods, reset-reason logging and a safe boot recovery path. A reset loop may indicate a bad configuration or persistent fault; record enough evidence without wearing out storage or repeatedly energizing actuators.

Fault handlers should preserve useful registers and fault status where supported, then enter a defined bounded state or reset policy. Printing from a HardFault handler through a driver that depends on interrupts can deadlock. A minimal retained crash record with an integrity check is often more useful than complicated output in a broken context.

### Worked lab and expected behavior

For the physical path, inspect the NUCLEO-F446RE board manual, STM32F446 reference manual, linker map and vendor startup file named in I05's hardware notes. Locate reset handler, vector table, data-copy and bss initialization. Observe a GPIO toggle from a timer and measure its period with a logic analyzer; a terminal print interval is not the same measurement.

On the host, review the equivalent bounded state-machine logic and unit tests, but record physical startup, peripheral and interrupt measurements as pending. If firmware faults before main, inspect stack/vector addresses, clock setup and memory placement before changing application logic.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

What happens before main? How do load and run addresses differ? Why can printf fail in an ISR? Interview bank: firmware startup and fault handling.

### Mastery checklist

Explain the selected startup file.

Read a map and stack budget.

Bound ISR work.

Demonstrate watchdog purpose and reset evidence.

### Primary references

[CMSIS] Arm startup contract. [NVIC] Arm interrupt controller APIs. [NUCLEO] ST board documentation. [LD] Linker scripts.

### M12 independent exercise

Timebox 60 minutes. Design a watchdog supervisor for acquisition, communication and control tasks. Specify how each reports progress, what happens when one stalls and what reset evidence survives. Explain why an ISR-only feed is inadequate.

## M13 RTOS scheduling and Linux to MCU communication

### Objectives and prerequisites

Prerequisites: M07 M12.

Use tasks and queues, analyze timing and priority inversion, and make an MCU communication boundary recoverable.

### Lesson

An RTOS task is an independently scheduled execution context. Its priority matters only when it is ready to run and no higher-priority or interrupt work prevents progress. Blocking on a queue is usually preferable to polling because it states the wait condition and yields CPU time. Periodic work should use an absolute cadence or the RTOS's appropriate delay-until API to avoid accumulating processing time into the period.

A queue transfers fixed-size items or references according to its API. If you queue a pointer to a stack buffer and return, the consumer can use invalid data. If a buffer pool owns payloads, specify how ownership returns on normal receive, timeout, cancellation and shutdown. A queue's maximum depth is also a timing choice: large queues can hide stale data. Decide whether to block, drop oldest, drop newest or enter a fault state under overload.

Priority inversion occurs when a high-priority task waits on a resource held by a lower-priority task that is itself delayed by other work. FreeRTOS mutexes provide a simplified priority inheritance mechanism, and their behavior with multiple held mutexes is deliberately limited. Mutexes are task-context objects, not objects an ISR can wait on. Use the port's appropriate FromISR APIs and obey its interrupt-priority rules. [FREERTOS-MUTEX]

A schedulability argument needs execution times, periods/deadlines, blocking and interrupt costs. Utilization alone is not a proof for arbitrary tasks. Begin by measuring task execution and queue waiting under load on the MCU. Host-port FreeRTOS execution is useful for task/queue behavior but includes host scheduling and cannot establish the chip's worst-case timing.

The Linux-MCU link should carry a bounded framed protocol with version, sequence and integrity checking. Define boot handshakes, capability negotiation, heartbeats, retries and reboot detection. After either endpoint restarts, stale buffered commands must not be mistaken for current intent. Use a session or explicit reset rule and require a current valid command before enabling actuation. A stop or limit on the MCU must remain effective when Linux is stalled.

Keep firmware updates distinct from normal application messages. Validate compatibility and image integrity, preserve a recovery path and handle interruption. A CRC checks corruption; a signature verifies an authorized signer under a trust model. An update format, bootloader state and reset-vector handoff must agree on the actual MCU.

### Worked lab and expected behavior

Run I05's real FreeRTOS POSIX-port application and host integration tests using its setup instructions. Inspect queue producer/consumer behavior, protocol handling and shutdown. Record RTOS source tag and host compiler. Then change task rates or inject a communication timeout and explain the observed queue policy.

For the physical path, use the selected Nucleo port notes and a logic analyzer to measure ISR-to-task latency and serial timing. These measurements are separate from host tests. If the firmware blocks, inspect task states and queue ownership; increasing every queue size does not diagnose the root cause.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Can an ISR take a mutex? Does priority inheritance prevent deadlock? What happens to stale commands on reboot? Interview bank: RTOS and protocol scenarios.

### Mastery checklist

State task and queue ownership.

Explain priority and ISR constraints.

Bound command age and recovery.

Separate host logic from MCU timing proof.

### Primary references

[FREERTOS-MUTEX] FreeRTOS mutex documentation. [NVIC] Arm interrupt rules. I05 includes its exact RTOS source and port references.

### M13 independent exercise

Timebox 90 minutes. Define a Linux setpoint protocol with a 200 ms freshness requirement, sequence numbers and MCU reboot detection. Specify states DISABLED, ARMED and FAULT, including valid transitions. Test lost heartbeat, duplicate messages, invalid CRC and endpoint restart.

## M14 Buildroot Yocto and image maintenance

### Objectives and prerequisites

Prerequisites: M08 M06.

Choose the build-system scope, package a service and preserve a reproducible image configuration.

### Lesson

Buildroot creates a toolchain, root filesystem and often a bootloader/kernel for an embedded image. Keep project changes in a br2-external tree and save a minimal defconfig plus any selected component configs. A changed option can require a clean rebuild; copying one newly compiled file into an old image can conceal inconsistent dependencies. The release-local manual and project configuration are the execution contract. [BUILDROOT]

Yocto is a broader build ecosystem using OpenEmbedded metadata and BitBake tasks. A recipe describes fetching, configuring, compiling and packaging a component; a layer organizes metadata and configuration. An image selects packages and policies. A machine selects hardware integration; a distribution configuration selects wider policy. An SDK gives application developers a compatible cross environment. These roles are different, so keep application recipe changes separate from hardware policy where possible.

Pin source revisions and record layer branches, configuration, checksums and license information. A recipe that fetches a moving branch without a fixed revision can change without your code changing. Reusing a build cache speeds development but does not excuse an undeclared dependency. Reproduce a release from a clean documented setup and compare the intended outputs; a timestamp difference must be explained rather than ignored.

Service integration includes the executable, configuration, user account, writable directories and lifecycle policy. A service should fail clearly on invalid configuration, use bounded restart behavior and expose health. A systemd unit belongs only in an image that uses systemd; a BusyBox init script has a different contract. Writing to /var on a read-only root requires a writable mount or an explicit transient-storage design.

Separate immutable image content from persistent device configuration. A new image must understand the stored configuration or migrate it transactionally with a rollback policy. Test full storage, malformed configuration and interrupted writes. A persistent partition is not automatically safe if the application rewrites a file in place.

Build systems can generate license manifests and software bills of materials, or SBOMs. Yocto's SPDX output describes components and relationships useful for release inventory. An SBOM is not proof that the image is secure or that every license obligation has been satisfied; it is evidence used in those reviews. [YOCTO-SBOM]

### Worked lab and expected behavior

Build I01's minimal appliance first. Verify the custom program is installed and starts through the intended init system. Then use I04 to add a telemetry recipe and service in its own Yocto layer. Record configuration and inspect the generated image manifest. Generate an SDK using I04's command and build the client with that SDK.

Expected image behaviors require guest boot logs. A recipe parsed without errors has not been compiled, packaged or booted. If a service is missing, inspect image package inclusion, recipe install paths and init-system integration in that order. Avoid editing files directly inside tmp/work as the permanent fix.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

Explain recipe versus layer versus image. How do you pin a release? What does an SDK contain? Interview bank: build systems and deployment.

### Mastery checklist

Rebuild from saved configuration.

Trace a source file to an installed package.

Demonstrate service lifecycle.

Explain persistent state across upgrades.

### Primary references

[BUILDROOT] Official manual and pinned release-local manual. [YOCTO-HOST] Yocto 5.0.9 requirements. [YOCTO-SBOM] Yocto SPDX generation. I04 includes exact layer/recipe references.

### M14 independent exercise

Timebox 90 minutes excluding build time. Package a telemetry executable, one default configuration file and service startup. Change configuration persistence policy so an image upgrade preserves user settings. Deliver recipe/package source, image selection and reboot tests.

## M15 Debugging tracing and performance

### Objectives and prerequisites

Prerequisites: M06 and M07 for user-space work; M09 is additionally required for kernel-specific sections.

Form falsifiable bug hypotheses, interpret crash evidence and measure latency without confusing averages with guarantees.

### Lesson

Start a debugging note with expected behavior, actual behavior, reproducibility and the smallest relevant environment. Separate facts from hypotheses. Change one discriminating variable or add one observation that distinguishes the leading explanations. A fix that makes a failure disappear once is not a root-cause argument; demonstrate the mechanism and add a regression test.

GDB shows the stopped execution state. Backtraces identify frames; locals and register values add context, but optimized builds can omit variables or transform control flow. Preserve the exact executable and debug information that produced a core. A symbol file from another build can create a convincing false trace. In multithreaded hangs, inspect every thread and identify wait dependencies. [GDB]

strace observes system calls and signals, making it useful for permissions, missing files, repeated timeouts and blocking I/O. It changes timing and cannot show every application-level operation. perf samples execution and performance events where permissions and hardware support allow it. ftrace exposes kernel events and function-related tracing, including scheduling and latency paths, subject to kernel configuration. Trace the subsystem that can answer your question instead of enabling everything and drowning the event of interest. [FTRACE]

AddressSanitizer and UndefinedBehaviorSanitizer help user-space memory and selected language errors; ThreadSanitizer targets races in supported builds. KASAN detects kernel memory safety problems with modes dependent on architecture/configuration. KCSAN samples for races and its lack of a report is not a proof of absence. Instrumentation changes memory/timing, so compare like-for-like builds and do not present instrumented timing as production performance. [GCC13] [KASAN] [KCSAN]

For performance, specify metric, workload, warmup, duration, repetitions and environment before optimizing. Report a distribution: count, median, high percentiles and maximum observed delay. The maximum observed is not a proven worst case. CPU utilization and end-to-end latency can move in different directions; batching reduces overhead but can increase waiting. A02 measures wakeup delay on the host and records this limitation.

A kernel oops contains the fault location, process context, registers, call trace and kernel build information. First preserve it, then resolve symbols against the matching build. An oops can taint or damage state, so do not assume continued execution remains reliable. Reproduce in a disposable guest with the smallest workload and the appropriate detector.

### Worked lab and expected behavior

Run A02's baseline and contention experiments with its fixed input and output schema. Compare identical sample counts and workloads. Explain the expected mechanism before changing the implementation. Retain raw measurements and the summarizing script. If tracing is unavailable due to permissions, say it was not executed and use an available observation without claiming equivalent coverage.

Take a broken-code challenge in the assessment folder. Use one hypothesis per step, record the evidence and write the smallest regression test that fails before the fix. A test that only asserts the current implementation's internal choices is weaker than one that checks the externally required behavior.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

What does a backtrace prove? Why does a clean sanitizer run not prove safety? How would you investigate a tail-latency regression? Interview bank: debugging and performance.

### Mastery checklist

Preserve exact build and raw evidence.

Falsify a hypothesis with an observation.

Report latency distributions honestly.

Add a regression test tied to the failure.

### Primary references

[GDB] GDB backtraces. [FTRACE] Kernel ftrace. [KASAN] KASAN. [KCSAN] KCSAN. [GCC13] GCC sanitizers.

### M15 independent exercise

Timebox 75 minutes. Diagnose a logger whose average latency is low but rare delays exceed 100 ms. Propose observations to distinguish scheduler contention, synchronous disk writes, mutex waits and application batching. Specify what evidence would justify each fix.

## M16 Persistence security and reliable updates

### Objectives and prerequisites

Prerequisites: M14 M07.

Make configuration writes recoverable, distinguish update trust from boot trust and test rollback state transitions.

### Lesson

Crash-safe persistence starts with a failure model. A process crash, OS crash and sudden power loss have different effects. For an atomic file replacement on a suitable local filesystem, write a temporary file in the same directory, flush the file, rename it into place and flush the directory as required by the durability contract. Preserve permissions and validation. Atomic rename does not by itself prove durability on every filesystem or storage device. Recovery tests must match the target media and failure modes.

A signed update authenticates an image or manifest under a trusted verification key. Hashing detects change but does not identify an authorized signer. Secure or verified boot checks the boot chain according to the platform's trust root; it is a separate mechanism from an application's update verification. A correctly signed but vulnerable old image can still be a downgrade risk, so version/rollback policy and protected state matter. [UBOOT]

An A/B arrangement writes an inactive slot, verifies its contents, selects it for a limited trial boot, checks health and commits or rolls back. The old slot remains the recovery candidate until the new one is accepted. Define which state survives interruption at each transition. An application health check that merely confirms process existence can commit a device whose sensors or persistent configuration are unusable. [RAUC]

The A03 project implements this state machine over files and uses real signature verification. It is a host simulator, not a bootloader, flash partition manager or hardware secure-boot chain. It establishes software behavior for exercised signature, state and interruption cases. Porting it to a board requires partition layout, bootloader integration, trusted-key storage, recovery entry, storage durability and power-cut tests. Reuse a maintained update framework when the product scope warrants it.

Apply least privilege to services: only required device access, files, network endpoints and capabilities. Validate untrusted lengths, protocol states and configuration. Avoid secrets in logs and source trees. Maintain dependency versions, provenance and a vulnerability-response process. An SBOM supports inventory; reviewing reachable exposure and deploying fixes remain engineering tasks. [YOCTO-SBOM]

Recovery is part of the interface. A technician should be able to identify the running version, previous known-good image, reset reason and last update result. Protect access to diagnostic endpoints, but make the information available through an intentional service path. Define behavior for full storage and corrupted state instead of assuming writes always succeed.

### Worked lab and expected behavior

Run A03's tests for an authentic image, modified payload, invalid signature, interrupted installation, failed health and successful commit. Inspect the generated public/private key separation; generate lab keys locally and never treat them as production keys. Then execute C01's invalid-configuration and restart tests.

For each update phase, write the expected active slot before and after a crash. Acceptance requires deterministic recovery to a verified allowed state, not merely a script returning zero. Physical power interruption and boot ROM trust remain pending until target tests exist.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

How do signing, secure boot and rollback differ? What does atomic rename guarantee? What makes a health check meaningful? Interview bank: production and system design.

### Mastery checklist

Identify trust and recovery boundaries.

Test every persisted-state transition.

Preserve a known-good image.

State untested storage and hardware assumptions.

### Primary references

[UBOOT] U-Boot verified boot. [RAUC] RAUC basics. [YOCTO-SBOM] SBOM generation. [FSYNC] fsync durability. [RENAME] rename semantics.

### M16 independent exercise

Timebox 90 minutes. Add a compatibility identifier and monotonic version policy to an update manifest. Test a valid upgrade, wrong board ID, same version, old version, altered payload and a crash before health commit. Explain how recovery interacts with downgrade prevention.

## M17 Integration testing and capstone engineering

### Objectives and prerequisites

Prerequisites: M03 M13 M16.

Turn requirements into acceptance tests, integrate earlier work and present evidence with clearly bounded claims.

### Lesson

A capstone should make a small system work end to end. C01 uses one primary communication path and focuses on acquisition, configuration, health and recovery. C02 separates Linux supervision from MCU control behavior and starts with a deterministic robot simulation. Reusing a parser is useful; merely renaming the parser project is not system integration.

Write requirements as observable behavior with an ID, precondition, stimulus and acceptance condition. For example: when valid telemetry stops, the gateway changes health to stale within the configured deadline without corrupting the last stored configuration. The test should manipulate time and inputs so it can check both just-before and just-after the threshold. Distinguish requirement limits from measured results.

Use unit tests for conversion and parsing, integration tests for process/protocol boundaries and system tests for boot/recovery behavior. Mocks isolate a contract but can faithfully reproduce a wrong assumption. Compare at least one transaction with the real interface later. Hardware-in-the-loop adds board behavior, but its sensors, loads and timing coverage still need to be described.

Fault injection should follow the architecture: drop, delay, duplicate or corrupt a frame; restart either endpoint; fill storage; break configuration; fail a health check; interrupt update; disconnect a sensor; stall a control task. Define the expected bounded response first. A test that never asserts the fault outcome merely proves the harness ran.

Continuous integration, or CI, should start with deterministic host checks that need no devices. Compile warnings, unit tests and protocol integration belong there. Heavier image builds and hardware tests can run separately with versioned artifacts. Never upload a successful status when an important test was skipped; report passed, failed and not run separately. KUnit offers an in-kernel unit testing framework where kernel logic can be isolated appropriately. [KUNIT]

The portfolio should contain a concise architecture, a demo script, requirements-to-test mapping, raw logs, a failure investigation and a limitation statement. Say what you wrote and which dependencies you used. Explain one design you changed after evidence. A credible account of a bug and its fix often demonstrates more depth than a long feature list.

### Worked lab and expected behavior

Run C01 and C02's minimum host demonstrations and test suites. Inspect the acceptance IDs in their documentation. Add one new fault without changing the implementation first; predict the outcome, run it and explain any difference. Use the aggregate validation report only as an index; the project logs contain the evidence.

For a final defense, draw the architecture from memory, explain the wire format, identify every persistent state and walk through one restart during an active transaction. Then modify a small requirement under time pressure and update its test. This demonstrates transfer beyond a memorized demo.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

How do you know the system works? What did your mock not test? What is your own contribution? Interview bank: project defense and systems design.

### Mastery checklist

Map every capstone requirement to a test.

Demonstrate recovery under one injected fault.

Explain scope and untested hardware.

Modify behavior without breaking existing acceptance.

### Primary references

[KUNIT] Kernel unit testing. C01 and C02 provide original requirements and acceptance tests. Project-specific sources govern actual hardware contracts.

### M17 independent exercise

Timebox 120 minutes. Add one capstone requirement: after three consecutive invalid frames, record a diagnostic event without resetting the valid command freshness timer. Write acceptance cases for two, three and four corrupt frames, an intervening good frame and a restart. Define whether the counter is consecutive or cumulative.

## M18 AI assisted learning and embedded inference

### Objectives and prerequisites

Prerequisites: M02 M15 M17.

Use AI to improve reasoning without losing independent competence, and evaluate a small inference extension against a non-AI baseline.

### Lesson

Use the sequence independent attempt, hint, implementation, test and explanation without AI. Begin with a concrete requirement and your own hypothesis. Ask for the smallest useful hint before asking for a complete implementation. After using generated code, inspect ownership, bounds, errors, synchronization and the exact versioned API. A plausible function name or explanation is not documentation.

Supply context deliberately: architecture, source tag, compiler diagnostics, minimal reproducer, expected behavior and constraints. Ask the assistant to separate observations from hypotheses and to propose a test that distinguishes them. For a kernel answer, require the exact declaration in the selected source tree. If a suggested API is missing, investigate the mismatch; do not paste a fabricated prototype into your project to make compilation progress.

AI-generated tests can mirror the implementation's mistake. Define an external behavioral oracle and adversarial cases first. Check boundaries, resource exhaustion, partial progress, concurrency and teardown. Run both a normal build and relevant instrumentation where supported. Review fixes independently and preserve the original failure as a regression. On an assessment, record whether AI was allowed; a good assisted score does not establish unassisted competence.

Before sharing code or logs, remove credentials, customer identifiers and confidential material unless the chosen environment and policy explicitly allow it. Keep secrets outside prompts and repositories. Respect interview rules; never present assisted work as an independent performance when that would be misleading.

For product AI, start from a measurable need such as sensor anomaly detection. Define a non-AI baseline, collect representative data and split training, validation and test by time or device to avoid correlated samples leaking across sets. Choose a threshold on validation data only. Report false alarms and missed faults under the relevant prevalence, not accuracy alone. Synthetic training data is useful for checking a pipeline, but does not establish field accuracy.

Inference belongs in a layer with a defined resource and timing budget. Linux user space can host a suitable runtime; a supported MCU runtime can run small models under tighter memory/operator constraints. LiteRT for Microcontrollers has a distinct deployment model and supported operations; verify compatibility before selecting a board. Quantization can reduce model costs but requires accuracy and hardware evaluation. [LITERT] [ONNX]

Keep deterministic control limits independent of the model. A stale, uncertain or failed inference produces a defined fallback rather than extending motor authorization. Measure model bytes, working memory, latency distribution and CPU usage on the chosen target. C02's optional model extension is a small educational pipeline; its synthetic results must remain labeled as such.

### Worked lab and expected behavior

Use the reusable prompts in 06_AI_Integration. Take one assessment bug, attempt it independently, then request only competing hypotheses. Test the leading hypothesis and explain the result without reopening the response. Keep an assistance log with the supplied context, suggestion accepted/rejected and evidence.

Run C02's optional anomaly example only after the core simulator passes. Compare it with its deterministic baseline on a held-out set. Treat host execution and synthetic dataset metrics as limited evidence. A model that performs poorly should remain an analysis exercise; it must not replace a working control boundary.

### Independent work

Attempt exercises.md before opening solutions.md.

### Review and interview connections

How did you verify generated code? Why can random dataset splits leak? What happens when inference fails? Interview bank: AI verification and capstone defense.

### Mastery checklist

Keep an assistance log.

Verify APIs against selected versions.

Explain and modify code independently.

Compare inference with a baseline and preserve fallback.

### Primary references

[LITERT] Google LiteRT for Microcontrollers. [ONNX] ONNX Runtime quantization. Core engineering contracts use the sources in the relevant preceding modules.

### M18 independent exercise

Timebox 60 minutes. Ask an AI to review one of your parser functions using the exact protocol and tool versions. Independently verify every suggested API and change. Then write a one-page explanation of a wrong or unsupported suggestion and the test that exposed it.

# 4 Fifteen project runbooks

Each project has its own runbook, complete source for its stated minimum target, build/configuration files, tests, independent exercise, solution guidance and evidence record. Start with the README and VERSIONS.md before running commands. Estimates in the runbooks are learner practice estimates; the weekly roadmap allocates that practice within its 572-hour core schedule.

| ID | Level | Project runbook | Validation scope |
| --- | --- | --- | --- |
| B01 | Basic | B01 Embedded C utility library | Host build and tests; ASan and UBSan with leak detection disabled |
| B02 | Basic | B02 Linux process and resource inspector | Host build live proc and deterministic fixture tests |
| B03 | Basic | B03 UART terminal and framed protocol tool | Host build PTY framing corruption timeout and terminal cleanup tests |
| B04 | Basic | B04 Multithreaded sensor simulator and logger | Host build queue shutdown output-failure tests and separate TSan run |
| B05 | Basic | B05 Cross compilation and QEMU application debugging | AArch64 static/dynamic cross builds and QEMU user execution |
| I01 | Intermediate | I01 Custom Buildroot appliance | Host persistence tests and Buildroot external configuration resolution |
| I02 | Intermediate | I02 Kernel character driver and user space client | Userspace build and Linux 6.12 kernel composite-object compile |
| I03 | Intermediate | I03 Device tree GPIO interrupt platform driver | Linux 6.12 object compile and offline overlay application |
| I04 | Intermediate | I04 Custom Yocto telemetry image | Native telemetry build/test and complete Yocto metadata authored |
| I05 | Intermediate | I05 RTOS telemetry connected to Linux | Real FreeRTOS POSIX tests and STM32 firmware cross-build/link |
| A01 | Advanced | A01 Sensor driver using the IIO subsystem | Conversion tests kernel object compile and offline overlay application |
| A02 | Advanced | A02 Linux latency and performance investigation | Host functional tests and raw idle/loaded latency experiments |
| A03 | Advanced | A03 Signed A B update and rollback simulator | Signature tamper interruption health and rollback host simulation tests |
| C01 | Capstone | C01 Industrial monitoring and edge gateway | Gateway tests and real FreeRTOS to FIFO to gateway integration |
| C02 | Capstone | C02 Linux MCU robot control and diagnostics simulation | Deterministic robot tests simulation and optional synthetic AI evaluation |

### Minimum target and physical integration

Hardware absence is handled through explicit target choices. B05 uses foreign-architecture Linux user emulation. I05 includes both an executed real-RTOS host port and a cross-built physical STM32 target. A03's complete minimum is a signed file-slot update simulator. C01 integrates the real RTOS host producer with a gateway and includes image metadata. C02's minimum is a deterministic robot simulation. Kernel drivers and board overlays are authored and compile checked, with actual board behavior left unverified.

These choices preserve the learning progression while avoiding claims that simulated peripherals, boot selection or braking establish physical behavior. To complete target-level competence, follow each project's named hardware/runtime acceptance procedure and save your own evidence. Extensions are separate from the complete minimum implementation.

### Reproduce the offline checks

From the package root run python3 tools/verify_host_package.py. It rebuilds and tests 14 available host suites or components and reports external-target portions explicitly. It needs a Linux host, Python 3, a C/C++ compiler, make and OpenSSL. It does not download dependencies, load modules, run QEMU or require root. Run I05 and cross/emulator targets separately after following their setup instructions.

Generated build outputs are not substitutes for source. Rebuild them with the provided commands and capture versions. The project evidence logs remain useful even when a target dependency is not installed locally.

## B01 Embedded C utility library

Build a small library that can be reused by a serial tool and MCU application. Its minimum implementation contains a bounded byte ring, checked bit updates, a CRC function, and a binary frame encoder/decoder. All storage is caller owned; there are no hidden heap allocations. Plan 8–12 hours after modules M02 and M03. This project supports the common C foundation of all three career tracks.

### Outcomes and acceptance

Explain the invariant 0 <= count <= capacity, preserve FIFO order across wraparound, reject bit positions outside 0–31, and parse a stream fragment without reading beyond its length. Demonstrate every payload size from zero to 128 bytes, every truncation point of each valid packet, bad CRCs, oversized declarations, and at least one known CRC test vector. Pass make test and the supported sanitizer configuration. A correct return value alone is insufficient if an invalid input changes memory outside the output object.

### Environment and files

Use a Linux host, GCC with C11 support, Make, and a terminal. The recorded build uses GCC 13.3.0 on x86_64. No board or wiring is required. The source also avoids Linux-specific interfaces, making it suitable for later MCU reuse after target compilation and timing checks. VERSIONS.md records the actual environment.

include/util.h is the API contract; src/util.c is the complete reference implementation; tests/test_util.c exercises boundaries and malformed input. Read exercises/EXERCISES.md before solutions/SOLUTIONS.md. evidence/ contains validation records. There is no hardware directory because this is a portable library.

### Design and protocol

The ring stores a separate count so all allocated slots can be used. Both indices wrap modulo capacity. Full insertion and empty removal return false without changing state. Objects must be initialized successfully before other ring functions; callers must not modify their fields independently. A ring instance has one owner or requires an external lock.

The wire format is A5 5A | version u8 | type u8 | length be16 | sequence be16 | payload | CRC be16. Version is 1; length is payload length, at most 128. The CRC uses polynomial 0x1021, initial value 0xFFFF, no reflection, no final XOR, and covers bytes beginning with version through the payload. For 123456789, it yields 0x29B1. This is a teaching protocol specification, not a claim of compatibility with an existing industrial protocol.

The decoder returns MORE for an incomplete candidate, INVALID with one byte consumed for resynchronization, or OK with the full frame length. It writes a completed output only after structural and CRC checks. Network order is decoded byte by byte; the implementation never casts unaligned packet memory to a C structure. CRC detects accidental errors and provides no authentication.

### Guided build and debugging lab

Run from this project directory:

make test

make sanitize

gdb ./build/test_util

In GDB, set break ring_push, run, inspect r->count, and step through the full-buffer rejection. Set a conditional breakpoint at frame_decode when n == 8 and follow the oversized-length test. Expected test output begins with PASS and names the checked cases; the exact executed output is preserved in evidence/host_tests.log.

If LeakSanitizer cannot inspect processes in a restricted container, use ASAN_OPTIONS=detect_leaks=0 make sanitize and record that leak detection was disabled. That command was the successful recorded configuration here. Address and undefined-behavior instrumentation remained enabled. The code allocates no heap memory, but this does not make leak detection generally unnecessary in later projects.

Before adding features, explain why replacing the count with write_at == read_at creates ambiguity between empty and full. Change one loop boundary locally, show the failing test, and restore it. make clean removes only this project's build directory.

### Troubleshooting and limitations

A CRC mismatch often comes from including sync bytes, using little-endian CRC storage, or applying reflection accidentally. An off-by-one bug at length 128 should be investigated with the byte count 8 + payload + 2. Sanitizers cannot prove correct synchronization or correct hardware register behavior. This library deliberately supplies no lock-free guarantee, allocator, retransmission, or cryptographic protection.

### Demonstration and interview defense

Demonstrate a passing boundary test, a rejected corrupted frame, and successful recovery by the B03 stream receiver. Explain why unsigned shifts need range checks, why C structure padding is unsuitable for a wire format, and who owns each buffer. A truthful portfolio statement is: “Implemented and host-tested a bounded C codec and ring buffer with malformed-input and sanitizer checks.” Add specific counts only from retained evidence.

### Primary references

GCC warning options

GCC instrumentation options

Access date: 27 October 2026. The protocol layout and acceptance tests are original teaching design choices.

## B02 Linux process and resource inspector

Create a command-line inspector that turns a process's Linux /proc records into one JSON report. It reads the name, state, thread count, resident memory, CPU ticks, and number of open file descriptors. Plan 8–10 hours after M01, M02, M05, and M06. This project provides evidence for Linux file I/O, error handling, command-line design, data parsing, and debugging.

### Outcomes and acceptance

Produce valid JSON for a live process and deterministic fixtures, reject malformed PIDs, preserve a process name containing spaces and parentheses, and exit with failure if required process records disappear. Understand that /proc is a kernel-provided view, and its individual files do not form one atomic snapshot. Missing optional resident-memory data is -1; an unreadable descriptor directory is also -1. These values mean unavailable, never zero usage.

### Environment and structure

Use Linux with a mounted /proc, GCC, Make, and Python 3 for tests. No external Python modules or hardware are needed. src/inspect.c is complete; tests/test_inspect.py creates isolated fixtures and exercises real /proc/self. VERSIONS.md and evidence/ record validation. There is no driver, board configuration, or wiring component.

### Architecture and data flow

The CLI validates the PID before constructing paths. It reads named fields in status, then locates the final right parenthesis in stat before numbering fields from the state field. Splitting the entire stat line on whitespace would misparse a command name containing spaces. User and kernel CPU ticks are fields 14 and 15; the report includes _SC_CLK_TCK so a caller can convert ticks to seconds. Linux proc stat documentation.

The file-descriptor count is a separate directory enumeration. For self, opening the enumeration directory itself can affect the observed count; do not use it as an exact historical count. A process may terminate between reads, and access can be restricted by permissions, namespaces, or proc mount policy. The tool returns an explicit error rather than inventing a consistent snapshot.

### Guided run and investigation

make test

./build/inspect self

./build/inspect 1

gdb --args ./build/inspect self

The second invocation prints JSON. The PID 1 invocation can succeed or be denied depending on the host; that environmental result is informative. Compare a successful report to /proc/PID/status immediately, allowing for process changes. Calculate (user_ticks + system_ticks) / ticks_per_second as accumulated CPU seconds. It is not CPU utilization; utilization needs two samples over elapsed time and a stated normalization convention.

In GDB, break after the stat line is read and inspect the point returned by strrchr. Use the test fixture with a command containing ) to justify using the last closing parenthesis. Deliberately remove the fixture stat file between runs and explain the failure path. --root PROC_ROOT PID is explicitly a testing interface; it must point to trusted fixtures when used by privileged code.

Run make clean to remove the build output. Do not stop or modify another process as part of this project.

### Test plan and troubleshooting

The automated suite tests live self-inspection, invalid and overflowing PID strings, JSON escaping of quotes, parenthesized and spaced names, exact CPU counter mapping, malformed records, and absent process directories. A nonzero return during inspection of a short-lived process can be a normal race. A negative RSS field for a kernel thread means unavailable. If JSON parsing fails, inspect escaping and accidental diagnostics on standard output; errors belong on standard error.

The implementation is a bounded educational inspector, not a complete process accounting tool. It does not provide RSS accounting across shared pages, cgroup limits, historical samples, CPU percentages, or process identity stability across PID reuse. An extension should retain a start-time identity and use pidfds where appropriate, without claiming that a pidfd turns all proc reads into an atomic transaction.

### Independent practice and defense

Attempt the separate exercises before reading their solutions. In a five-minute demonstration, show live output, run the tricky-name fixture, explain one permission failure, and calculate CPU seconds. Interview questions: Why is parsing stat with %s unsafe? What changes between two proc reads? Why is RSS different from virtual size? What does -1 mean in this interface? State exactly which fields are required and which are optional.

### Primary references

Linux process stat fields

Linux process status fields

Access date: 27 October 2026.

## B03 UART terminal and framed protocol tool

Implement one bounded request-response transaction over a POSIX terminal device. The tool sends a framed UTF-8/ASCII command, waits for a matching reply, tolerates byte fragmentation and noise, and restores the terminal configuration before exit. Plan 12–16 hours after B01 and M06. It teaches UART configuration, stream framing, partial I/O, deadlines, and protocol tests without requiring hardware initially.

### Scope and measurable outcomes

The minimum tool accepts DEVICE PAYLOAD TIMEOUT_MS, uses 115200 baud with eight data bits, no parity, one stop bit, and no hardware flow control, and sends request type 1 with sequence 1. It accepts response type 2 with that sequence. Payload size is 0–128 bytes. A single monotonic deadline covers both transmission and response; irrelevant traffic cannot restart that deadline. Pass all PTY tests and explain why readable does not mean one full message is available.

### Architecture and interface

The B01 codec is included locally so this folder can be built independently. A nonblocking terminal feeds a bounded accumulation buffer. Invalid candidates consume one byte, preserving possible later sync. A valid but stale sequence is discarded. Incomplete candidates wait for additional bytes. The protocol is specified in docs/PROTOCOL.md; it is a private teaching protocol and does not provide authentication, retransmission, or duplicate suppression.

The terminal library uses tcgetattr, cfmakeraw, explicit speed and stop-bit settings, tcsetattr, and nonblocking poll. Restoring saved settings happens through one cleanup path after success, transaction failure, SIGINT, SIGTERM or SIGHUP. A minimal signal handler records cancellation; ordinary code performs cleanup. SIGPIPE is ignored so a broken stdout pipe becomes a handled write error. SIGKILL and a device that no longer accepts termios operations cannot be repaired by user-space cleanup. POSIX terminal behavior and noncanonical read settings are described in termios. Readiness and hangup details are described in poll.

### Environment and files

Use Linux, GCC 13.3.0 or a compatible C11 compiler, Make, and Python 3. A pseudo-terminal, or PTY, acts as the peer in tests. This exercises software transport behavior, not electrical signaling or UART peripheral timing. Source is in src/, the shared codec contract in include/, and the independent Python peer in tests/. Hardware measurements remain separate from PTY evidence.

### Guided lab

make test

./build/uart_frame /dev/ttyUSB0 hello 1000

gdb --args ./build/uart_frame /dev/ttyUSB0 hello 1000

The second command requires a compatible connected peer and device access. Without a reply it exits with a timeout, which is correct behavior. For an emulator-only start, run the tests first; they create and name a PTY automatically. Read test_pty.py and identify where the peer fragments bytes, injects noise and a stale frame, damages the CRC, or withholds a response.

Set a breakpoint at frame_decode in the receiver. Watch the accumulator grow while the same logical frame arrives over many reads. Then explain why checking only read() == packet_size would discard valid communication. The client does not call an unbounded tcdrain, so deadline handling is not defeated by a stalled device drain operation. make clean removes generated binaries.

### Hardware integration path

Use a USB-to-UART adapter and a documented 3.3 V TTL UART target. Cross adapter TX to target RX, adapter RX to target TX, and connect signal grounds. Board pin numbers depend on the selected board and its alternate-function settings; the hardware worksheet requires those values before connection. Do not substitute RS-232 voltage levels or assume every USB adapter is 3.3 V. No adapter-to-board power connection is required for independently powered devices. The exact reference target is the host PTY; the physical UART path is optional and unverified in this package.

### Acceptance and troubleshooting

The suite covers normal replies, one-byte fragmentation, noise/stale/corrupt-frame recovery, timeout after a corrupt-only reply, zero and maximum payloads, overlength rejection, and terminal attribute restoration after success, errors, SIGINT, SIGTERM, and a broken stdout pipe. The fixed sequence is reused across separate invocations; an old sequence-1 response can therefore pass in a later invocation. This limitation is explicit in the protocol contract. A device-open failure suggests permissions or a wrong path. A consistent CRC failure suggests endian or CRC coverage disagreement. Immediate disconnect errors suggest cable removal or a PTY master closing. A successful PTY test does not validate the board's baud clock, ground integrity, overrun behavior, or framing-error recovery.

### Extensions and interview demonstration

Extend the protocol with negotiated version support or a bounded retry policy only after defining duplicate behavior. Show the PTY test, explain one monotonic deadline, and draw the receive state transitions: accumulate, incomplete, discard, accepted. Defend why a CRC is not a security feature and why volatile is unrelated to robust serial buffering. Practice tasks and worked solutions are in separate folders.

## B04 Multithreaded sensor simulator and logger

Build two simulated sensor producers and a CSV consumer with a bounded queue. The system runs for a selected sample count, drains on a normal termination request, joins every created thread, and exits on an output failure. Plan 12–16 hours after M07. This is evidence for POSIX threads, ownership, condition predicates, resource cleanup, and concurrency debugging.

### Learning outcomes and acceptance

Prove that shared queue fields are accessed under one mutex, that every condition wait rechecks its predicate, and that the finite run writes every produced sample exactly once. Show zero-work behavior, a full queue, normal termination during production, and a failing output stream. Race-tool success is useful evidence and does not prove the absence of every possible race.

### Architecture

Two producers generate (sensor, sequence, value) records. The queue holds eight records and applies backpressure when full. The main thread removes records and performs file I/O outside the queue lock, allowing producers to continue until capacity is reached. Per-sensor sequence is deterministic; global interleaving depends on scheduling and is deliberately not specified.

SIGINT, SIGTERM, and a private shutdown signal are blocked before any worker starts. A dedicated sigwait thread synchronously receives them. It sets the stop flag while holding the queue mutex and broadcasts to waiting producers. This avoids calling condition-variable functions from an asynchronous signal handler and avoids pretending that volatile provides thread synchronization. Producers decrement the active count before exit. The consumer stops when the queue is empty and no producer remains, then joins workers and the signal monitor.

If writing fails, the main thread marks failure, asks producers to stop, and drains remaining queue entries without claiming they reached disk. It returns nonzero. Flush on each record makes the teaching fault test deterministic; it is a throughput tradeoff, not a recommended high-rate logging policy.

### Environment and source navigation

Use Linux, GCC, pthreads, Make, and Python 3. src/logger.c contains the complete implementation; tests/test_logger.py checks ordering, output shape, early termination, and output failure. No hardware or external services are involved. Measured host tests do not establish real sensor sampling precision or hard deadlines. See VERSIONS.md and evidence/VALIDATION.md.

### Guided lab

make test

./build/logger build/samples.csv 100 1000

make tsan

gdb --args ./build/logger build/debug.csv 1000 1000

Arguments are output file, samples per producer, and a delay in microseconds. The delay is an approximate pacing interval and accumulates scheduling and work time; this is not a periodic real-time scheduler. In GDB use info threads, interrupt while running, and inspect a producer waiting at the full queue. Confirm that a thread waiting on a condition does not keep the mutex locked while asleep.

Run a long count from a shell and send SIGTERM from another terminal. Verify that the process returns promptly and each sensor's sequences remain contiguous. The automated test also writes to /dev/full to force a real operating-system I/O error. make clean removes binaries and generated CSV files beneath build/.

### Validation and practical limits

Recorded host tests passed for 1,000 total samples, zero work, early SIGTERM, and failed output. A GCC ThreadSanitizer run passed with 2,000 total samples; retain the exact command and log. Re-run on your environment because sanitizer support can differ. The queue is bounded, but lossless logging still requires either sustainable consumer throughput or a declared policy for dropping data. Blocking sensor acquisition threads may be inappropriate for an actual sensor FIFO; redesign with explicit overrun accounting when hardware is added.

### Troubleshooting and defense

A hang after production ends usually means the active-producer count or wakeup is wrong. Missing records require distinguishing data never produced from data queued but not written. Excess CPU usage suggests polling instead of predicate waits. A benchmark should state storage type, flush policy, sample size, scheduling policy, and queue depth.

Demonstrate a normal run, termination, and /dev/full. Explain the predicate for both waits, why wakeups are advisory, how failed thread creation is unwound, and which thread owns the output stream. A useful extension is monotonic timestamping with absolute periodic deadlines and an explicit deadline-miss metric.

### Primary references

POSIX condition wait specification

Linux pthread condition documentation

GCC sanitizer options

Access date: 27 October 2026. The POSIX web endpoint returned 403 during retrieval; Linux documentation and executed behavior provide the accessible supporting references.

## B05 Cross compilation and QEMU application debugging

Compile the same C program for the host and for AArch64, inspect both ELF files, and run the AArch64 version through QEMU's Linux user-mode emulator. Plan 10–14 hours after M04, M05, and M08. The complete program computes the sum of integers from 1 through a bounded N and identifies its compiled architecture. This deliberately small workload keeps the focus on ABI, toolchains, sysroots, dynamic loaders, and remote debugging.

### Scope and measurable outcomes

Show that the native binary identifies x86_64 on the recorded host, the cross binary identifies aarch64, and both return the same result for boundary values. Explain the ELF machine, interpreter path, pointer size, and the difference between static and dynamic linking. A successful user-mode run does not validate kernel boot, drivers, device tree, interrupts, or board peripherals. I01 introduces a full-system guest.

### Environment and pinned route

The baseline host is x86_64 Ubuntu 24.04-compatible Linux. The verified downloadable toolchain route uses Bootlin aarch64--glibc--stable-2024.05-1 and QEMU user 8.2.2. The exact compiler, sysroot, and emulator versions appear in VERSIONS.md and the recorded log. These are frozen educational inputs, not assertions that older packages contain current security fixes.

For a normal Ubuntu workstation, gcc-aarch64-linux-gnu, qemu-user, and gdb-multiarch provide a convenient alternative, but their installed package versions must be recorded. Do not mix a loader from one sysroot with libraries from another without checking ABI compatibility.

### Build and run

From this project directory, for distribution-installed tools:

make test

make cross

sh scripts/run_qemu.sh

For a relocated Bootlin toolchain, use its actual extracted location:

export CROSS_COMPILE=/absolute/toolchain/bin/aarch64-buildroot-linux-gnu-

make cross

qemu-aarch64 -L "$(${CROSS_COMPILE}gcc -print-sysroot)" ./build/target_sum_aarch64 1000000

qemu-aarch64 ./build/target_sum_aarch64_static 1000000

${CROSS_COMPILE}readelf -h build/target_sum_aarch64

${CROSS_COMPILE}readelf -l build/target_sum_aarch64

The expected sum for N equal to 1,000,000 is 500000500000. It is computed with a 64-bit accumulator. N is capped before the loop so integer limits and unreasonable run times are explicit. The program rejects negative, nonnumeric, overflowing, and above-limit inputs.

### Remote debugging lab

In terminal one, run qemu-aarch64 -g 1234 -L SYSROOT ./build/target_sum_aarch64 10, substituting the actual sysroot path. In terminal two:

gdb-multiarch ./build/target_sum_aarch64

Then enter set sysroot SYSROOT, target remote :1234, break sum_to, continue, info registers, and step. Debugger path expressions are commands, not literal shell variable expansions inside GDB. Replace SYSROOT with the real path. The remote-debug session is an instructed lab unless its own log is supplied; emulator functional tests alone do not establish that this debugger step ran.

QEMU user mode translates guest system calls to the host kernel. A foreign userspace program is running, but there is no separate AArch64 kernel. QEMU user-mode documentation. To learn boot flow and real kernel-driver integration, proceed to I01 and the kernel projects.

### Troubleshooting and acceptance

An Exec format error from executing the cross binary directly means no compatible loader/emulation path was used. A missing /lib/ld-linux-aarch64.so.1 usually indicates a wrong or absent sysroot. file and readelf should resolve architecture and interpreter uncertainty before code changes. Static linking reduces runtime-loader dependencies but does not remove kernel ABI assumptions or make the binary universally portable.

Run the native tests, then compare native and emulated outputs for 0, 1, 10, and 1,000,000. Record command, return code, architecture, and numeric result. Reject the invalid inputs under emulation too. make clean removes only generated binaries.

### Interview demonstration and extension

Show an ELF header, run the AArch64 binary, and explain the sysroot. Defend why a cross compiler's machine tuple does not specify the entire board support package. Add a small file-I/O workload and discuss which behavior is exercised by syscall translation. Keep any measured emulator throughput separate from target hardware performance.

### Primary references

Bootlin toolchain release archive

QEMU Linux user mode

QEMU GDB usage

Access date: 27 October 2026.

## I01 Custom Buildroot appliance

Build a minimal x86_64 Linux appliance containing a custom C package, a startup service, and a separate persistent data disk. Every service iteration safely advances an on-disk counter and prints JSON. The host-testable counter gives a small but complete persistence and recovery problem before a long image build. Plan 20–30 hours after B05 and M14, allowing extra download and build time outside active study.

### Learning outcomes and acceptance

Keep product customization in a br2-external tree, resolve a pinned Buildroot configuration, cross-build an application using the target compiler, boot a guest, and preserve state across a reboot. Demonstrate normal writes, concurrent writers, a crash before rename, a crash after rename, corrupt state, missing storage, and interrupted service operation. Configuration validation is not image-build validation; host filesystem tests are not physical power-loss tests.

### Supported target and contents

Reference: Buildroot 2025.02.1, upstream qemu_x86_64_defconfig, Linux 6.12.9, GCC 13.3.0, binutils 2.43.1, and the release's selected libc and BusyBox. Those values come from the resolved configuration, retained in evidence. The Linux version intentionally differs from I02's separately frozen 6.12.0 API-check tree. These learning pins need updating and revalidation before any real deployment.

No physical board is required. QEMU presents the root filesystem as /dev/vda and a separate 64 MiB ext4 data image as /dev/vdb. The root drive uses a temporary QEMU snapshot; the data drive is persistent. Networking is disabled in the provided launcher. Image creation only formats a project-owned regular file; never substitute a host block-device path.

src/telemetry_once.c is the complete app. br2-external/ contains package metadata and root filesystem overlay. config/learning.fragment records customization. scripts/configure.sh layers the changes over the release's upstream defconfig. tests/test_counter.py exercises the state logic on the host.

### Architecture and recovery behavior

The service mounts its data filesystem before starting. A lock file serializes writers. Each update reads and validates the current versioned counter record, creates a new record, writes a temporary file, calls fsync on the file, renames it in the same directory, and synchronizes the directory. The record includes an XOR check word to detect common accidental changes; it is not a cryptographic integrity check. Invalid existing state is preserved and the service stops for diagnosis.

The previous or new complete file is expected after an interrupted update on a filesystem with the stated rename and synchronization semantics. The host tests inject process exits at specific checkpoints. Storage-controller caches, real power loss, torn sectors, and filesystem damage need separate destructive tests on disposable hardware. The persistent disk is intentionally excluded from root-image replacement; update partition design is developed in A03.

### Guided build and run

First validate the application:

make test

mkdir -p build/state

./build/telemetry-once build/state

Download the official buildroot-2025.02.1.tar.xz release and its signature from the Buildroot archive, verify it using the project's documented release keys, and extract it outside this project. Install the host dependencies documented in that release's manual. Then:

sh scripts/configure.sh /absolute/buildroot-2025.02.1

make -C buildroot-output -j4

sh scripts/run_qemu.sh

The launcher creates the project data image only if it does not already exist. Log in on the serial console of this isolated teaching guest and inspect cat /data/counter. Stop the service using /etc/init.d/S50learn-telemetry stop, run telemetry-once /data, then restart it. Reboot and confirm the counter advances from the previous value. Use QEMU's console exit sequence only after testing normal shutdown first.

The upstream defconfig also enables a host QEMU build. You may use that generated emulator from buildroot-output/host/bin by placing the directory on PATH, or a compatible installed QEMU. Record the version. Run make -C buildroot-output legal-info and retain the generated manifest; supplied package metadata alone is not a complete image license review.

### Tests and evidence

Host acceptance tests cover initial state, regular increments, both crash checkpoints, forty concurrent writers with unique increments, corrupt state preservation, symlink rejection, and missing-directory errors. LEARN_CRASH_AT=after_file_sync exits before the committed rename; after_rename exits before directory synchronization. A process-exit test cannot prove power-failure durability. Configuration resolution was executed; the full toolchain/image build and guest boot are pending unless additional evidence is added.

Debug the C app with gdb --args build/telemetry-once build/state. In the guest, use service logs and mount status before suspecting the parser. A missing /dev/vdb keeps the service stopped. A corrupt record should never be silently overwritten with zero. For cleanup, stop QEMU before removing build output; retain the data image while investigating recovery. Delete that image only when intentionally resetting the lab.

### Project defense

Explain the difference between a Buildroot package and a rootfs overlay, why target compilation uses TARGET_CC, why file fsync alone is insufficient for a new directory entry, and why process-crash tests leave storage power-loss behavior unverified. Demonstrate the counter fault tests before discussing larger product architectures. Extensions include a bounded retention policy, schema migration, and a read-only configuration inspection command.

### Primary references

Buildroot manual; the exact 2025.02.1 manual is also bundled in its archive.

Linux fsync documentation

Buildroot release archive

Access date: 27 October 2026.

## I02 Kernel character driver and user space client

Implement an educational Linux miscellaneous character device backed by a bounded byte FIFO. It supports blocking reads and writes, nonblocking EAGAIN, partial transfers, poll, concurrent access, and normal module lifetime rules. Plan 24–32 hours after M09 and M10. Its purpose is to learn user/kernel boundaries and concurrency; a real sensor should normally use the appropriate Linux subsystem developed in A01.

### Supported environment and acceptance

The API baseline is upstream Linux 6.12.0 with an x86_64 build tree. Use a disposable guest with the same built kernel and module configuration. A host compiler cannot prove a module will load into an arbitrary running kernel. The provided client is Linux userspace C11. Source is complete, but the supplied validation distinguishes object compilation, final module linking, guest loading, and device runtime tests.

Accept a passing zero-length operation, empty/full nonblocking semantics, partial writes, read/write readiness, a sleeping reader awakened by a writer, and a 1 MiB concurrent ordered stream. Repeat stress tests while checking kernel logs. Add the user-copy and unload tests in the separate test plan. A kernel warning, memory fault, blocked teardown, or lost data is a critical failure even if some transfer tests pass.

### Driver architecture

The driver registers /dev/learn_fifo with mode 0600. A single mutex protects the FIFO's data and indices. User copies can fault and sleep, so they occur under a sleepable mutex. An atomic byte-count snapshot provides wait and poll predicates; the actual FIFO remains locked. After moving data, operations update readiness and wake the opposite side. Every wake is followed by a predicate recheck under the mutex because another thread may consume the opportunity first.

Read and write return positive partial counts when some user bytes were transferred, even if the remaining copy faulted. They return an error when no bytes were transferred. Poll registers both wait queues before sampling readiness. The interface is a shared byte stream: it has no per-message atomicity, per-client queue, broadcast semantics, or EOF-on-last-writer behavior. Normal module unloading is blocked while open file references pin .owner = THIS_MODULE. This simple static device does not claim safe hot-unplug support.

Linux 6.12 no longer provides the old no_llseek name used in many tutorials; this source uses stream_open and no seek callback. Consult the selected tree instead of copying API names from unrelated versions. Linux 6.12 misc device lifetime and kernel FIFO APIs support the design choices.

### Build and deploy inside the guest

Build and boot the selected Linux 6.12 kernel following M09, preserving its full output directory and Module.symvers. Then from the project directory:

make client

make module KDIR=/absolute/linux-6.12-built-tree

Copy the project and module into that guest, install Python 3 there for the automated runtime test, and run as guest root:

sh scripts/guest_test.sh

The script loads the module, runs the small client, runs the device tests, displays recent kernel messages, and unloads on exit. Do not run it against the shared host kernel. If building through a separate O= kernel output directory, point KDIR to that output tree. modules_prepare prepares headers and build tools but does not supply a complete Module.symvers; an object-only check is useful but cannot replace a full module build.

### Guided debugging lab

First inspect /sys/class/misc/learn_fifo/dev and the corresponding device node. Use strace in the guest on build/fifo_client /dev/learn_fifo hello. Observe that readiness and transfers are separate calls. Empty the FIFO, start a reader, verify that it sleeps, then write from a second terminal. Fill the FIFO in nonblocking mode and show EAGAIN. Retain the log before and after a concurrent stream test.

For deeper debugging, use the kernel's documented GDB/QEMU workflow with debug symbols, or add temporary tracepoints and remove them after isolating the failure. Do not print one message per byte in a timing experiment. A one-writer/one-reader ordering test does not prove arbitrary many-writer record atomicity, because this interface does not promise record boundaries.

### Validation and limitations

The supplied evidence records GCC 13.3.0 userspace client compilation and a W=1 kernel composite-object build against prepared Linux 6.12.0. Final .ko modpost checks, insertion, all device runtime tests, signal interruption, copy faults, and unload stress are pending in a disposable guest. No runtime success is inferred from object compilation.

If /dev/learn_fifo is absent, inspect registration errors and devtmpfs/udev. If insmod reports a format or symbol error, compare the running release, configuration, compiler options, and module symbol versions. Never bypass that mismatch with forced loading. Before cleanup, close clients and unload normally; make clean KDIR=... then removes local outputs.

### Interview demonstration

Defend why the lock is a mutex, why a wakeup is not ownership of data, how partial user copies are reported, and which references prevent normal unload. Explain why this FIFO needs no ioctl for its minimum functionality. A meaningful extension is a carefully specified statistics interface with stable types, or a dynamically allocated per-device lifetime design using reference counting and explicit removal behavior.

### Primary references

Linux 6.12 external module builds

Linux 6.12 miscellaneous devices

Linux 6.12 kernel FIFO API

Access date: 27 October 2026.

## I03 Device tree GPIO interrupt platform driver

### Purpose and learning outcome

Bind a device from firmware description, request its GPIO as input, count IRQ events, expose one read-only sysfs attribute, and prove teardown ordering. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 18–26 hours; allow remediation time when a prerequisite fails. Relevant learning modules: M08 M09 M10. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Build I02, explain sleepable context, read a device tree phandle, and identify physical versus BCM pin numbers.

### Minimum scope and version boundary

Linux mainline v6.12, GCC matching the target architecture; Raspberry Pi 4 Model B hardware path. This frozen kernel is educational, not a security-maintained production recommendation.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

The device tree describes GPIO17. The platform bus matches the private compatible string and calls probe. Probe acquires state, GPIO, then threaded IRQ, then publishes sysfs. The threaded handler increments an atomic counter. Managed cleanup removes the sysfs group first, quiesces/releases the IRQ, releases GPIO, then frees state. No timer, background worker, or custom character-device lifetime is hidden behind automatic cleanup.

### Build run debug and cleanup

1. Prepare a disposable Raspberry Pi 4 Model B SD-card system using mainline Linux tag v6.12 and a symbol-enabled BCM2711 DTB. Keep the original boot media untouched and a known working recovery card.

2. Build the kernel with ARCH=arm64, a matching AArch64 compiler and CONFIG_PINCTRL_BCM2835, CONFIG_OF, CONFIG_MODULES, CONFIG_SYSFS. Use the kernel source configuration for the exact board; confirm the actual GPIO symbol exists in the base DTB before applying the overlay.

3. Build this module with make KDIR=/absolute/linux-build ARCH=arm64 CROSS_COMPILE=aarch64-linux-gnu- after the kernel build has generated its headers and symbol versions. Do not insert a module compiled for a different running kernel.

4. Compile the overlay with dtc -@ -I dts -O dtb -o learning-edge.dtbo config/learning-edge-overlay.dts. Apply offline with fdtoverlay -i bcm2711-rpi-4-b.dtb -o learning-rpi4.dtb learning-edge.dtbo; boot this DTB together with the matching kernel through the documented boot configuration on your prepared image.

5. Power down, wire the 3.3 V pulse source as documented, then boot. Load learning_edge_counter.ko with sudo insmod learning_edge_counter.ko.

6. Locate the actual attribute using find /sys/bus/platform/devices -path '*/learning_edge/edges' -o -path '*/learning_edge*/edges'. Read it with cat. Device names can contain an address/prefix; use the path actually discovered.

7. Debug with dmesg -w, the device driver symlink, the decoded live device tree, and /proc/interrupts. Stop pulses, unload using sudo rmmod learning_edge_counter, and restore the original DTB to clean up.

### Acceptance criteria and evidence

A clean 10 Hz source must add exactly 100 counts during an externally gated 100-pulse burst. Read repeatedly during pulses; values must never decrease. Unbind and rebind 20 times without kernel warnings, use-after-free reports or stale sysfs access. A missing event-gpios property must fail probe cleanly. Verify that no pin is driven by this driver. Manual switch bounce is not an exact-count test.

Run python3 tests/check_count.py /actual/sysfs/edges --seconds 10 --expected 100 only with the gated pulse source synchronized to the interval. The script is a target acceptance helper, not a host IRQ simulation. Device-tree schema checking requires placing the private binding in a kernel schema tree and resolving its educational vendor prefix; do not call it an upstream accepted binding.

Observed validation: Source, overlay, binding, wiring, and test helper authored. Hardware IRQ, unload stress, device tree application and electrical behavior are pending. Module compile status is recorded separately in evidence/STATUS.md.

### Troubleshooting and limitations

A GPIO event is counted, not debounced, timestamped, queued or interpreted as an industrial safety input. IRQF_ONESHOT prevents reentry into the threaded handler but does not make pulse counting lossless at arbitrary rates. For buttons use the existing gpio-keys subsystem; for timestamped high-rate data choose a suitable hardware/subsystem interface.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Extend the interface with a last-edge monotonic timestamp without exposing a torn pair of count and timestamp. Explain the synchronization and ABI before changing code. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show the DT property and pin table, load the module, send a measured pulse burst, show count change, then unload while a reader is active. Explain why sysfs is acquired last.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

Offline device tree check passed: The overlay compiled with dtc from Linux v6.12 and applied with fdtoverlay to a symbol-enabled BCM2711 Raspberry Pi 4 Model B base DTB. See evidence/device-tree.log. This establishes syntax and phandle resolution, not a booted board or peripheral behavior.

## I04 Custom Yocto telemetry image

### Purpose and learning outcome

Produce an image from an owned layer, install a service, generate an SDK, inspect build metadata and record exact layer revisions. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 22–36 hours plus unattended downloads and builds; allow remediation time when a prerequisite fails. Relevant learning modules: M04 M08 M14 M16. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Build B05/I01, understand recipes and a sysroot, and use a dedicated supported build host.

### Minimum scope and version boundary

Yocto/Poky tag yocto-5.0.9 Scarthgap, qemux86-64, dedicated Ubuntu 22.04 host. Native app checked with GCC 13.3.0 only. Image and SDK builds pending.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

The meta-learning layer supplies an application recipe, a systemd unit and an image recipe. The application prints monotonic simulated samples and handles termination. systemd owns restart and privilege isolation. The recipe passes target compiler and linker flags; no host binary is copied into the target image. The image is intentionally an isolated development image with debug-tweaks enabled.

### Build run debug and cleanup

1. Use a dedicated Ubuntu 22.04 x86-64 VM because this is an explicitly supported host in the frozen Yocto 5.0.9 documentation. The test container running this authoring session is not a verified Yocto build host. Allocate approximately 16–32 GB RAM and 150–250 GB free disk as practical estimates; exact requirements depend on image and parallelism.

2. Install the host prerequisites from the official 5.0.9 system-requirements page. Typical Ubuntu packages include build-essential chrpath cpio debianutils diffstat file gawk gcc git iputils-ping libacl1 locales python3 python3-git python3-jinja2 python3-pexpect python3-pip python3-subunit socat texinfo unzip wget xz-utils zstd lz4. Enable an en_US.UTF-8 locale and read the exact release documentation if a package check fails.

3. From this project directory run sh scripts/setup.sh /absolute/empty/workdir. It checks out tag yocto-5.0.9, adds this layer, configures qemux86-64 and systemd, then builds learning-image. It downloads and executes build tools from their official sources; run as an ordinary user.

4. In another shell cd /absolute/workdir/poky and . oe-init-build-env build; run bitbake-layers show-layers and save the output plus git -C .. rev-parse HEAD.

5. Boot with runqemu qemux86-64 learning-image nographic. In the guest inspect systemctl status telemetry, journalctl -u telemetry, and systemctl stop telemetry.

6. Generate the SDK with bitbake learning-image -c populate_sdk, install the generated script from tmp/deploy/sdk into a separate directory, source its environment-setup file and compile the application again. Inspect the ELF architecture before copying it to the guest.

7. Exit the guest normally and retain conf, layer source, revisions and logs. Remove the dedicated work directory only when downloads and evidence are backed up.

### Acceptance criteria and evidence

A clean image boots in QEMU; telemetry.service is enabled; sequential sample numbers appear with monotonic seconds; SIGTERM stops without a crash; the service runs unprivileged. The SDK emits a target ELF. A rebuild with the same saved configuration and inputs must have its artifact hashes compared, with any differences investigated rather than declared reproducible by assumption.

Run python3 -m unittest discover -s tests -v for a native compile and shutdown test. This checks application behavior only. Run BitBake parsing, image, QEMU boot, service sandbox and SDK tests on the supported host. Inspect bitbake -e learning-telemetry to diagnose recipe paths and inherited values.

Observed validation: Layer, recipes, service, configuration and setup scripts authored. Native application compile and one process integration test passed. BitBake parse, image build, QEMU boot and SDK creation have not been executed in this session.

### Troubleshooting and limitations

Frozen 5.0.9 is a reproducibility baseline, not a recommendation to deploy an outdated image. Before production, update to a maintained release, rebuild, review advisories and licensing, remove debug-tweaks, provision accounts/keys, define update/recovery policy and verify kernel support. The selected Yocto kernel is determined by its own pinned metadata and must not be confused with the standalone Linux 6.12 driver lab.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Package a separate configuration file and prove that updating the application does not overwrite operator-owned persistent settings. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show layer revision, recipe inputs, target ELF, service identity, sample logs and graceful stop; finish with a diagram showing host build, SDK and target runtime as separate environments.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

## I05 RTOS telemetry connected to Linux

### Purpose and learning outcome

Execute a real RTOS scheduler on the supported POSIX port, transfer bounded records between tasks, serialize framed telemetry, and detect a stalled task through supervised progress. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 20–30 hours for simulator path; allow remediation time when a prerequisite fails. Relevant learning modules: M03 M07 M12 M13 M17. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Build B03/B04, explain CRC framing, queues, task priorities and the difference between elapsed time and a deadline guarantee.

### Minimum scope and version boundary

FreeRTOS-Kernel V11.1.0 GCC POSIX port; host GCC 13.3.0; Python 3.12.14; Linux x86-64. STM32 NUCLEO-F446RE physical target source is also supplied and cross-built with GCC ARM 13.2.1; physical execution remains pending.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

A periodic sensor task creates one sample every 50 RTOS ticks at 1 kHz. A four-element FreeRTOS queue transfers ownership by value to a serial task. Only the serial task writes the host transport. A higher-priority supervisor checks both producer and transmitter progress every 20 ticks; after 200 ticks without progress it ends the simulation with failure. The serial task implements partial writes, EAGAIN, EINTR and a bounded transmission timeout. The receiver verifies version, kind and CRC before decoding signed millidegrees Celsius.

### Build run debug and cleanup

1. Use a Linux x86-64 user account with GCC, Make, Python 3.12 and pthread support. No board purchase is needed for this first path.

2. Fetch exact FreeRTOS tag V11.1.0 using sh scripts/fetch.sh. Its license remains with the fetched source; the learning package does not redistribute the kernel.

3. Run make. To use an existing checkout: make FREERTOS=/absolute/FreeRTOS-Kernel. The authoring build used GCC 13.3.0 and the real V11.1.0 source archive.

4. Run python3 -m unittest discover -s tests -v. Tests create temporary FIFOs, start a receiver before opening the transmitter, run the real RTOS executable and decode its binary output.

5. Manual demonstration: in one terminal mkfifo /tmp/learning-uart then cat /tmp/learning-uart | python3 src/receiver.py; in another terminal run ./firmware /tmp/learning-uart. Twenty decoded samples are expected; remove the FIFO afterward.

6. Repeat with ./firmware /tmp/learning-uart stall. The sensor deliberately stalls after five records; supervisor termination must return status 1.

7. Debug with gdb --args ./firmware /tmp/learning-uart. The POSIX port uses threads/signals, so inspect RTOS task state and host threads separately. make clean removes the executable.

### Acceptance criteria and evidence

Normal run transmits sequences 0–19 with correct CRC and no truncated records, exits successfully, and preserves signed decoding. Corrupt packets are rejected and valid packets after noise resynchronize. A sensor stall fails within the supervisory policy in RTOS ticks. A full/blocked transport must not prevent the supervisor from progressing. Hardware watchdog reset timing is not claimed.

Four executed tests cover normal ordered data, stalled-task detection, resynchronization and CRC rejection. The checks run the actual FreeRTOS POSIX port rather than a hand-written scheduler. Read evidence/build.log and evidence/tests.log. Extend the tests with a reader that stops consuming until the FIFO fills; then show that TX timeout/watchdog ends the run.

Observed validation: Code authored, real FreeRTOS kernel build verified, and four host-simulator tests passed. No MCU flash, peripheral, ISR, DMA, electrical or physical-watchdog test was executed. The complete STM32F446RE target is authored and cross-built; its board execution remains pending. See hardware/MCU_PORT.md.

### Troubleshooting and limitations

The tested runtime path uses FreeRTOS POSIX. A second complete target for STM32 NUCLEO-F446RE includes startup vectors, linker script, UART, hardware watchdog and the Cortex M4F port; it compiled and linked, but has not run on a board. A POSIX signal schedule cannot validate an MCU interrupt latency, ISR priority ceiling, stack margin or DMA cache policy. No claim of full hardware readiness follows from these tests.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Add command acknowledgement with sequence matching and a retry budget, then demonstrate that duplicate commands do not execute an action twice. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show task priorities and queue depth, run twenty records, inject the stall, show failure exit, then explain which tests still require a real MCU.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

STM32 target build evidence: Complete firmware cross-built and linked with arm-none-eabi GCC 13.2.1, yielding 6296 bytes text, 4 bytes data and 33084 bytes bss in this build. This is a build-size observation, not measured stack high-water usage. Physical flashing, UART framing, watchdog timeout and task stack margins remain pending. See evidence/mcu-build.log and hardware/MCU_PORT.md.

## A01 Sensor driver using the IIO subsystem

### Purpose and learning outcome

Register a real IIO temperature channel, use correct bus byte order and units, propagate bus errors and restore device configuration on removal. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 24–36 hours; allow remediation time when a prerequisite fails. Relevant learning modules: M09 M10 M11 M15. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Complete I02/I03, decode signed bit fields, understand I2C transactions and managed resource lifetimes.

### Minimum scope and version boundary

Linux mainline v6.12, GCC matching the target architecture; Raspberry Pi 4 Model B hardware path. This frozen kernel is educational, not a security-maintained production recommendation.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

An I2C client probe reads and saves the TMP102 configuration, selects 12-bit continuous mode, waits for conversion, and registers a direct-mode IIO device. Read raw returns the sign-extended 12-bit value. Scale is 62.5 millidegrees Celsius per count. Managed cleanup unregisters IIO before restoring the original sensor configuration. SMBus word-swapped helpers account for the sensor most-significant-byte-first register format.

### Build run debug and cleanup

1. Use Linux v6.12 on Raspberry Pi 4 Model B and a TI TMP102 breakout with documented schematic. Follow hardware/WIRING.md; connect SDA pin 3, SCL pin 5, V+ pin 1 at 3.3 V, GND pin 6 and ADD0 to ground for address 0x48.

2. Enable I2C controller support, CONFIG_IIO and modules in the matching kernel configuration. Remove any existing conflicting TMP102 node or driver binding. The supplied private compatible selects only this educational driver.

3. Build with make KDIR=/absolute/linux-build ARCH=arm64 CROSS_COMPILE=aarch64-linux-gnu-. Build the selected kernel first, including symbol metadata; compile against that same tree.

4. Compile and apply config/learning-tmp102-overlay.dts using dtc -@ and fdtoverlay against a symbol-enabled BCM2711 DTB. Ensure I2C1 pinmux and board boot configuration select the new DTB; the overlay does not magically replace boot firmware configuration.

5. Load learning_tmp102.ko. Locate the IIO directory whose name contains learning_tmp102. Read in_temp_raw and in_temp_scale; multiply to get millidegrees Celsius, then divide by 1000 for Celsius.

6. Debug probe errors through dmesg, the I2C adapter identity, logic-analyzer ACK/address traces and the hardware schematic. Do not repeatedly scan a shared production bus; use this isolated sensor bus.

7. Unload with sudo rmmod learning_tmp102; inspect that the original configuration was restored. Remove the overlay to clean up.

### Acceptance criteria and evidence

At raw word 0x1900 the decode is 25 °C; 0xE700 is −25 °C; 0xFFF0 is −0.0625 °C. All host golden vectors pass. On a board, compare measurements with a reference thermometer within the relevant datasheet accuracy and thermal-settling limits; resolution alone is not accuracy. Disconnecting the sensor must return an I/O error, not a fabricated temperature. Reprobe and concurrent reads must not warn or leak.

Run python3 -m unittest discover -s tests -v. Seven golden-vector checks verify the numeric reference model, including both signed boundaries and wire order. They do not call the kernel driver or emulate an I2C controller. Pending board tests include bus NACK, power loss, unbind/rebind, actual IIO units and configuration restoration.

Observed validation: Complete direct-mode IIO driver, overlay, wiring and decoding tests authored. Seven numeric host tests passed. Kernel-module compile status is recorded in evidence/STATUS.md; hardware execution is pending.

### Troubleshooting and limitations

The upstream TMP102 driver uses hwmon and is normally the suitable existing production driver. This private IIO implementation is for subsystem education. It omits alerts, runtime power management, triggered buffers and calibration. Add those only after selecting a real requirement; this device is a low-rate temperature sensor, not a high-speed inertial sensor.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Add a read-only sampling-frequency description based on configured conversion-rate bits, then design suspend/resume behavior that preserves operator configuration. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show register-to-value arithmetic, compare IIO raw and scale, demonstrate a bus failure, and explain why an existing hwmon driver is usually preferred.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

Offline device tree check passed: The overlay compiled with dtc from Linux v6.12 and applied with fdtoverlay to a symbol-enabled BCM2711 Raspberry Pi 4 Model B base DTB. See evidence/device-tree.log. This establishes syntax and phandle resolution, not a booted board or peripheral behavior.

## A02 Linux latency and performance investigation

### Purpose and learning outcome

Separate accumulated periodic drift from wakeup lateness, collect raw measurements, compare a scheduling change and report uncertainty honestly. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 18–26 hours; allow remediation time when a prerequisite fails. Relevant learning modules: M05 M07 M15. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Understand scheduling and monotonic clocks; finish B04 and basic tracing lessons.

### Minimum scope and version boundary

Host Linux x86-64, Python 3.12.14; recorded kernel 6.18.55. No board required for the delivered host minimum.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

A single Python process runs relative-sleep and absolute-deadline policies using monotonic nanoseconds. An optional separate busy process is pinned to the same allowed CPU to add contention. Each observation records planned and actual times. Percentiles summarize the sample; raw CSVs remain available. An optional trace-cmd wrapper records scheduler wakeup/switch events in a disposable VM.

### Build run debug and cleanup

1. Use Python 3.12 on Linux. Run python3 -m unittest discover -s tests -v.

2. Record kernel, CPU model/governor, power state, affinity and background workload. The script records kernel/Python/affinity, but these are not the entire environment.

3. Run python3 src/latency.py --out evidence/idle --samples 300.

4. Run python3 src/latency.py --out evidence/loaded --samples 300 --load.

5. Read each summary.json alongside its raw CSV. The relative policy accumulates oversleep; the absolute policy aims at a fixed timeline and can catch up. Compare them as different scheduling policies.

6. In a disposable VM with tracing permission, run sudo sh scripts/trace.sh, inspect with trace-cmd report or KernelShark, and correlate long late samples with runnable versus executing intervals.

7. For a tighter C experiment replace Python sleeps with clock_nanosleep(CLOCK_MONOTONIC,TIMER_ABSTIME,...) and record that as a new experiment; do not mix those values with the supplied Python results.

### Acceptance criteria and evidence

Every run saves exactly the requested number of samples, nondecreasing deadlines and environment metadata. Explain max, p99 and cumulative drift correctly. Identify at least one plausible scheduling cause using trace evidence before making a tuning claim. Show that a design improvement preserves correctness and report any throughput/power cost. No latency threshold is universal across hosts.

Two host tests validate sample accounting and evidence recording. Actual idle and loaded experiments each collected 300 samples per policy. No ftrace/perf data was collected here; the tracing command is supplied for the target environment.

Observed validation: Host tests and measurements executed. Actual records are in evidence/idle and evidence/loaded. The delivered results are observations from this shared host, not a PREEMPT_RT benchmark or deadline guarantee.

### Troubleshooting and limitations

Python overhead, host contention, virtualized scheduling, clock behavior, sample size and run order affect results. The loaded case can sometimes have a lower median due to noise/power state; do not select only favorable statistics. Comparing relative cumulative drift with absolute deadline error does not isolate scheduler latency. Use repeated interleaved runs and a C timing harness for a stronger study.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Explain a run where median improves but maximum worsens; design an experiment that distinguishes periodic drift, CPU starvation and lock contention. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show one raw late sample, compute its lateness manually, explain a scheduling trace, then justify the absolute-deadline change and its catch-up behavior.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

## A03 Signed A B update and rollback simulator

### Purpose and learning outcome

Authenticate release metadata and payload, write only an inactive slot, trial boot with a finite retry budget, confirm health, reject old versions and preserve a working active slot across an injected interruption. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 24–36 hours; allow remediation time when a prerequisite fails. Relevant learning modules: M03 M16 M17. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Understand hashes versus signatures, atomic file replacement, persistence and failure-state machines.

### Minimum scope and version boundary

Host Linux x86-64, Python 3.12.14; recorded kernel 6.18.55. No board required for the delivered host minimum. OpenSSL 3.0.13; P-256 ephemeral test signing keys.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

A signed manifest binds compatibility, release version and SHA-256 payload digest. OpenSSL verifies a detached signature against a provisioned public key. The installer snapshots verified bytes, writes the inactive file slot, fsyncs files/directories, then commits pending metadata. Each simulated boot consumes one trial before returning the candidate slot. Two unconfirmed attempts cause rollback. Health confirmation promotes the slot and advances the stored minimum accepted version. A filesystem lock serializes operations.

### Build run debug and cleanup

1. Use Linux, Python 3.12 and OpenSSL 3.0. No block-device access or root privilege is required.

2. Run python3 -m unittest discover -s tests -v. The suite generates ephemeral P-256 test keys inside temporary directories; no private key is shipped.

3. Run sh scripts/demo.sh. This creates signed factory/update bundles, provisions a fresh simulated device, installs B, trial boots B and confirms it.

4. Inspect python3 src/update.py --help for individual bundle/init/install/boot/good/status operations. Store release private keys outside a real target; only a public trust anchor belongs on it.

5. To demonstrate rollback, perform install then boot three times without good. The first two selections are B and the third is A.

6. To demonstrate tamper rejection, modify image.bin after signing and attempt installation; it must fail without changing active or pending state.

7. Delete only the dedicated simulation directory to reset. Never point this program at a real boot partition or existing device state.

### Acceptance criteria and evidence

All nine tests pass. Unauthenticated metadata, payload corruption, premature confirmation and non-increasing versions fail. A fault before pending-state commit leaves A selected. A bad trial image falls back to the verified active slot. Two unconfirmed trials restore A. Confirmation switches active and updates the minimum version. Every claimed durability boundary is tied to a write/fsync/rename step.

The tests execute real OpenSSL signatures and verify simulator state after injected failures. The demo executed successfully and ended with active B, min_version 2, no pending slot. Power loss is simulated at one transaction boundary; actual flash, filesystem, controller and bootloader behavior are not established.

Observed validation: Complete host simulator authored; nine tests and a signing/install/confirm demo passed. Real bootloader integration, storage power-cut testing, secure key storage, monotonic hardware counters and verified boot are pending production work.

### Troubleshooting and limitations

The simulator trusts its local directory, public-key file and state metadata. An attacker who can rewrite those can defeat rollback policy; the counter is not tamper resistant. Signatures authenticate an update package but do not establish secure boot, runtime integrity or trusted recovery. Use a maintained update framework such as RAUC and board-specific boot integration for a deployable system, then test the real interruption matrix.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Add a signed hardware compatibility mismatch test and a second trusted key for planned rotation, with an explicit revocation policy. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show one valid manifest, reject a changed byte, demonstrate two failed trial boots, then confirm a healthy update and explain why secure boot remains a separate requirement.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

Figure 2 Signed update verification and trial health lead to commit or rollback in the file-slot simulator.

## C01 Industrial monitoring and edge gateway

### Purpose and learning outcome

Integrate a bounded telemetry path, persisted validated configuration, freshness/alarm diagnostics, an unprivileged service and update health acceptance with executable fault tests. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 35–50 hours reusing earlier projects; allow remediation time when a prerequisite fails. Relevant learning modules: M06 M07 M13 M14 M16 M17. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Complete B03/I05, one image-builder project, and A03. Know the limits of CRC, sequence checks and durable configuration.

### Minimum scope and version boundary

Host Linux x86-64, Python 3.12.14; recorded kernel 6.18.55. No board required for the delivered host minimum.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

One byte-stream transport carries the I05 TL protocol. The parser rejects corruption and stale/replayed sequence numbers, allows sequence wrap within the half-range window, and retains at most an incomplete frame after processing. Valid data updates a latest-value snapshot and freshness time. The Linux process atomically persists configuration and health; EOF marks transport closed. A service definition connects a configured 115200 serial device. A separate acceptance script runs the capstone tests after an A03 trial boot and marks good only when they pass.

### Build run debug and cleanup

1. Use Linux and Python 3.12; no external Python packages are required. Run python3 -m unittest discover -s tests -v.

2. Run python3 src/simulated_mcu.py | python3 src/gateway.py --state run. Inspect run/config.json and run/health.json. On EOF, healthy becomes false because the transport has closed; this is expected.

3. Inject failures using --corrupt and --replay on the simulated MCU. Compare accepted/rejected counts and verify valid traffic after the bad packet recovers.

4. While running, inspect freshness and alarm behavior with the unit tests or stop the source temporarily. Persistent config defaults to limit_mC 30000 and stale_seconds 0.3; edit while stopped and restart. Invalid values fail startup.

5. Build the I05 firmware and use its frame output as the source. For an actual board, adapt the documented MCU transport, configure the serial device and permissions, and install config/gateway.service only after creating the dedicated gateway user and deploying source under /opt/learning-gateway.

6. In the I04 Yocto 5.0.9 build shell, add this complete layer using bitbake-layers add-layer /absolute/C01_Industrial_Gateway/config/meta-gateway, then bitbake gateway-image and runqemu qemux86-64 gateway-image nographic. The supplied recipe installs the source, creates the gateway user and enables a synthetic-source service. Verify /var/lib/learning-gateway/health.json and journalctl -u gateway-sim. For A/B deployment put that state directory on a persistent data partition; the minimum QEMU image retains state in its writable ext4 root. Image construction and boot were not executed here.

7. With an A03 initialized device and signed bundle, run python3 scripts/accept_update.py DEVICE_ROOT BUNDLE_DIRECTORY. The process trial boots, runs gateway acceptance, then confirms only on success. Clean up dedicated run directories after saving evidence.

### Acceptance criteria and evidence

Requirements C01-R1–R7: accept correct fragmented frames; reject corrupted frames; reject duplicate data without refreshing freshness; mark stale after configured timeout; keep the last accepted value and observable counters; survive restart with the same valid configuration; do not confirm an update when acceptance fails. No network listener or cloud dependency is needed for the minimum.

Eight tests cover counter units, fragmentation, freshness, replay, alarm, sequence wrap, invalid configuration and a real producer/consumer subprocess pipeline with corrupted and repeated frames. Add storage-full and interrupted-configuration fault injection before claiming field reliability. Gateway acceptance is source-level health, not a cryptographic binding between arbitrary installed image bytes and this host process.

Observed validation: Complete host gateway and simulated MCU pipeline authored; eight tests passed. Service units, a complete Yocto layer/image recipe and update acceptance integration authored. Board serial operation, final Linux image build, real update boot and industrial environmental testing remain pending.

### Troubleshooting and limitations

CRC detects accidental corruption and is not authentication. Sequence checks assume a continuous session; an MCU reboot needs an explicit session/handshake design before reconnect can safely reset the sequence baseline. The host snapshot store does not implement a long-term historian. The shell service unit is an integration example for a known ttyACM0 path; production should use stable device identity, controlled permissions and an appropriate persistent data partition.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Design and implement a boot-session identifier without allowing a replayed old hello packet to reset the anti-replay state. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show the system boundary, valid data flow, corruption recovery, stale-state transition, persisted configuration and update health gate. State clearly which code and tests you authored.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

Additional integration executed: The actual FreeRTOS V11.1.0 POSIX firmware was connected through a FIFO to the gateway subprocess. All 20 records were accepted, zero were rejected, and the transport closed cleanly. See evidence/rtos-integration.log and scripts/test_rtos_link.py.

Counter units: accepted counts complete accepted frames; rejected counts a CRC-invalid packet candidate or a complete out-of-range/replayed frame; discarded_bytes counts bytes skipped during resynchronization. Noise bytes are not misreported as rejected frames.

## C02 Linux MCU robot control and diagnostics simulation

### Purpose and learning outcome

Demonstrate Linux command generation, MCU-side validity and freshness checks, independent command expiry, latched emergency-stop semantics, differential-drive simulation and fault-oriented acceptance tests. This is an authored learning reference, not evidence of the learner's proficiency. Estimated effort: 40–60 hours plus optional AI work; allow remediation time when a prerequisite fails. Relevant learning modules: M03 M05 M13 M15 M17 M18. The reference skills framework covers this work; no employer-specific JD was supplied.

Prerequisites: Complete I05 and C01 concepts; understand differential drive kinematics, state machines, bounded command interfaces and timing evidence.

### Minimum scope and version boundary

Host Linux x86-64, Python 3.12.14; recorded kernel 6.18.55. No board required for the delivered host minimum.

The minimum source is complete for the stated target. Hardware-specific claims and optional integrations are separated from executed host checks. See VERSIONS.md and evidence/STATUS.md.

### Architecture and failure handling

The Linux role produces versioned CRC-protected velocity frames every 50 ms. The simulated MCU validates length, version, CRC, speed bounds and sequence freshness, then applies a 200 ms command lifetime. Invalid or repeated commands never refresh the deadline. A local emergency-stop latch forces zero output until deliberate rearm; rearm itself does not enable motion. A 10 ms simulation tick integrates wheel speeds into planar motion. The optional anomaly experiment runs beside this control model and never changes its output.

### Build run debug and cleanup

1. Run python3 -m unittest discover -s tests -v using Python 3.12.

2. Run python3 src/robot.py --output evidence/robot.csv. The default planner stops transmitting at one second. Inspect the final fresh command at 0.95 s and the first stale state at 1.15 s.

3. Plot time, left/right command and x position from the CSV. The deterministic model stops at approximately 0.345 m in this scripted case; the result assumes instantaneous speed response and is not a physical stopping distance.

4. Run python3 src/anomaly.py --out evidence/ai for the optional diagnostic extension. Review the fixed-threshold baseline and learned decision stump on the held-out synthetic test set.

5. Use I05 as the next RTOS implementation target for the MCU role, and C01 for transport diagnostics. Before hardware, add a motor-controller protocol, encoder acquisition, calibrated units and saturation, a real control loop, watchdog reset handling and independently engineered stopping hardware.

6. There are no motor pin assignments in the minimum: it never drives physical motors. Select an actual motor driver and board only after electrical limits, interfaces and safety requirements are known. Do not connect the simulation directly to power hardware.

7. Run config/ci.sh for tests and both evidence generators. Delete only generated evidence/run outputs when resetting experiments.

### Acceptance criteria and evidence

Requirements C02-R1–R9: startup output zero; malformed frame rejected; CRC corruption rejected; command bounds enforced; duplicate does not renew lifetime; sequence wraps correctly; expiry at 200 ms or first subsequent tick forces zero; emergency-stop remains latched across new frames; rearm alone leaves zero. The integrated default scenario must reach STALE at 1.15 s and hold zero afterward.

Nine executed tests cover these invariants plus in-place turning and the full command-loss simulation. They exercise the supplied deterministic software model. No physical braking distance, RTOS scheduling jitter, interrupt latency, electrical fault, functional safety level or industrial compliance is implied.

Observed validation: Simulation and acceptance tests passed; 201 time samples were recorded. The optional AI experiment executed and recorded accuracy, confusion matrices, timing, process CPU time, model bytes and memory observations. Physical robot and Linux–MCU transport integration remain explicitly separate work.

### Troubleshooting and limitations

This is a learning demonstrator. A single Python process does not establish isolation between Linux and MCU, and CRC does not authenticate a remote controller. The emergency-stop object is a software state machine, not an independent safety circuit. Mechanical dynamics, wheel slip, motor torque, communication jitter and hazardous failure handling must be engineered and validated before any production use.

If a command fails, record the command, version, first meaningful error, expected behavior and smallest reproducer before changing dependencies. Fix one hypothesis at a time. Keep build failures separate from runtime failures and unavailable hardware.

### Independent exercise

Introduce delayed and reordered command delivery, then show that stale commands cannot resume motion after an outage. Define a safe reset/session handshake. Attempt it before opening the solution guidance.

### Demonstration and interview defense

Show the responsibility boundary, inject command loss and a duplicate, demonstrate the stop latch, defend the 200 ms policy as a chosen lab parameter, then explain the hardware evidence still needed.

Be ready to answer: What owns each resource? Which failure first changes the visible state? What did the executed tests actually prove? What remains unverified? How would you reproduce a failure on another machine? A credible portfolio description names the selected environment, your contribution, the actual evidence and remaining limitations; do not claim industrial deployment from a lab simulation.

### Files and references

Source and build files implement the minimum behavior; tests define its acceptance gates; configuration holds target integration data; evidence holds recorded checks. Hardware directories are included only where a board path is meaningful. No hidden source archive is required except explicitly fetched official dependencies. Read sources.json for primary-source URLs, access date and claims.

Figure 3 The simulated MCU owns command validity and expiry independently of Linux supervision. Physical control remains a separate integration task.

# 5 Skill assessments

## Skill assessment suite

Begin with Diagnostic. Do not read its answer key until your timed attempt is complete. Scores are not assigned in advance.

Use Topic quizzes, Phase exits, Timed coding, Debugging challenges, Role mocks, and the Final practical. Complete Scoring and evidence rules before interpreting results. Record results in Evidence tracker.

Reference implementations and answers are intentionally separated under answers/. The files in fixtures/ include deliberately broken source and synthetic traces. They are assessment inputs, not project implementations or evidence from real hardware. Do not run broken examples on a production device.

For the 30 portable coding reference routines, compile answers/coding_reference.c with the command in its header. The POSIX Q061 example is answers/posix_reference.c. Actual validation results and sanitizer limitations are recorded in Reference validation.

An independent attempt forbids AI and answer keys. An AI-assisted attempt is separately labeled and must retain prompts, accepted changes, verification, and a later independent explanation. Neither attempt substitutes for real hardware checks.

## Scoring and evidence rules

A score measures the submitted attempt, not the person's general proficiency. Use not assessed until there is an artifact, date, and reviewer or self-review record. Retain source revision, build command, input, output, and an explanation of remaining uncertainty.

For a 100-point practical assessment, score correctness 30, reasoning 15, debugging method 15, resource handling 10, concurrency or state handling 10, tests 10, and communication 10. When a category is inapplicable, its points must be reassigned in the assessment before the attempt; do not change the rubric afterward to improve the result. A default pass is at least 75 with all critical gates satisfied. A 90-minute diagnostic is routing evidence, not a certification or pass/fail employment test.

Critical gates override totals. Fail the relevant gate if the submission knowingly leaves exploitable bounds errors on supplied inputs, use-after-free, unsynchronized conflicting shared access, indefinite required shutdown, fabricated results, acceptance of an unauthenticated update where authentication is required, or motion continuing beyond the specified local timeout. A persuasive explanation does not compensate for those failures. Correct the defect, add a regression, and retake the affected part with changed inputs. Do not average a failed critical gate into a high overall score.

Mark evidence separately as authored, build verified, host tests executed, emulator tests executed, specified hardware tests executed, and pending checks. An emulator success is not a measurement of electrical behavior, interrupt latency, cache coherency, DMA, or sensor accuracy on an absent board.

For oral questions, use the four one-point criteria in the bank. Score 0 for no valid mechanism, 1 for one correct criterion, up to 4 for all four. Require a concise explanation first, then probe one boundary or failure. A critical misconception makes the question fail even if three superficial criteria are present. A question-level score cannot replace the practical gate.

Retakes use a new input seed or changed requirement. Keep the original attempt and a short error analysis; do not overwrite failures. Mark a skill demonstrated independently only after a clean independent practical and a credible explanation. Use assisted practice for AI-supported work. Mastery is never inferred from possession of a generated solution.

## Debugging and review challenges

All supplied traces are synthetic teaching inputs. Do not describe them as measurements from the projects. Allowed tools: editor, compiler, local documentation, debugger, and static/sanitizer tools in a disposable host environment. No AI for independent attempts. For each challenge submit a hypothesis table with supporting and contradicting evidence, a minimal repair, a regression test, and remaining uncertainties. Score each out of 100 using the common rubric; pass at 75 with no critical gate failure.

### DBG01 Packet length review

Prerequisites M02–M03. Time 30 minutes. Review fixtures/broken_parser.c without initially running it. Inputs: empty input; one-byte input 04; 02 41 42; 11 followed by 17 data bytes; and 03 41. Identify each access that requires a precondition, repair the contract so output changes only on success, and add tests. Expected deliverable is a complete C function and executed tests. Critical gate: no out-of-bounds access on any supplied case. Remediation: checked lengths and transactional parsing.

### DBG02 Queue wakeup review

Prerequisites M07. Time 35 minutes. Review fixtures/broken_queue.c. Assume two consumers, one queued byte, spurious wakeups, and shutdown while empty. Explain at least three failing interleavings and repair the predicate. Submit a state-based test design and the corrected function. Critical gate: no empty-queue access and no indefinite required shutdown. Remediation: Q071, Q092 and C90.

### DBG03 Busy-loop trace

Prerequisites M06. Time 20 minutes. Read spin.trace. State the supported fault, the minimum control-flow repair, and tests for hangup with and without buffered data. List one electrical hypothesis the trace does not prove. Deliver a trace annotation and a small event-loop decision table. Critical gate: no endless immediate retry on EOF. Remediation: poll readiness and connection state transitions.

### DBG04 Kernel lifetime report

Prerequisites M09–M10. Time 35 minutes. Read kernel_oops.txt. Explain a possible remove-versus-work race. State the additional exact-source and build evidence needed before claiming root cause. Design teardown ordering that stops requeue, waits for work, and frees only after users quiesce. Include the lock-deadlock risk of synchronous cancellation. Critical gate: no callback can access freed state. Remediation: devres scope, workqueue cancellation and reference ownership.

### DBG05 Firmware watchdog record

Prerequisites M12–M13. Time 30 minutes. Read fault_summary.txt. Draw the supported blocking dependency in prose, calculate which recorded heartbeats are stale at 1400 ms using the stated 100 ms control timeout, and propose an isolation design. Do not infer absent fault-register values. Include how to validate the watchdog's actual hardware behavior later. Critical gate: do not feed solely because the logger still executes. Remediation: bounded queues, useful-progress supervision and priority inversion.

### DBG06 Driver API review

Prerequisites M10–M11. Time 40 minutes. Review this proposed design: hard IRQ handler takes a mutex, reads an I2C register, appends to an unbounded list, calls copy_to_user on a stored user pointer, and schedules work holding a devm pointer; removal only returns. Identify context, trust-boundary, allocation, ownership and teardown defects. Redesign with a threaded or deferred bus read, bounded queue, ordinary read/poll interface, and explicit disconnect state. Deliver a context table and removal sequence. Critical gate: user access and asynchronous state cannot outlive valid ownership. Remediation: Q095 and Q121–Q130.

### DBG07 Performance evidence review

Prerequisites M15. Time 25 minutes. A report claims 99.9th-percentile latency of 2 ms from 100 samples and calls this a hard real-time guarantee. State what the sample supports, what it does not, and a better measurement plan. Include intended/actual wake times, workload, clock, raw samples, distribution, outliers and host interference. Critical gate: do not relabel an observed maximum as a proven bound. Remediation: Q088, Q159 and Q198.

### DBG08 Update failure review

Prerequisites M16. Time 40 minutes. An updater writes the active slot, accepts any file with a matching CRC, changes boot flags before syncing data, and marks healthy when PID 1 starts. Identify four independent defects and propose an interruption matrix. Include shared-data rollback compatibility. Critical gate: reject unauthorized images and preserve a documented recovery path. Remediation: Q100, Q176–Q177 and A03.

## Initial diagnostic

ID D00. Prerequisites: none beyond access to a Linux shell and C compiler. Time: 90 minutes. Allowed tools: local compiler, editor, man pages; no AI, internet answers, or reference solutions. Purpose: find starting gaps. If setup is unavailable, answer on paper and mark every execution item pending rather than inventing results.

Submit one directory with answers.md, C source, command transcript, test output, and a short confidence/evidence table. Each item below is worth the stated points, totaling 100.

1. D01 8 points: explain the lifetime bug in char *f(void) { char b[4]="ok"; return b; }; propose two valid ownership contracts and their tradeoffs.

2. D02 8 points: classify INT_MAX+1, UINT_MAX+1u, and two unsequenced increments of one variable. State assumptions rather than guessing output.

3. D03 12 points: write checked size_t multiplication. Test (0,SIZE_MAX), (SIZE_MAX,1), (SIZE_MAX,2), and null output. Failure must leave output unchanged.

4. D04 8 points: in a temporary directory, create a text file, show permissions, redirect stderr separately, locate lines matching ERROR, and record a Git commit in a temporary repository. Do not use a user's existing repository.

5. D05 8 points: explain why one write of 100 bytes may require several reads, and show how a writer tracks partial progress. Include EINTR, EOF, and EAGAIN distinctions.

6. D06 10 points: repair if (count==0) pthread_cond_wait(&cv,&mu); take_item(); for multiple consumers and shutdown. State the lock and predicate invariants.

7. D07 8 points: describe a representative board boot chain and distinguish Linux application processors from MCU firmware. Name the assumptions needing board documentation.

8. D08 8 points: decide whether a hard-IRQ handler may take a mutex, allocate with GFP_KERNEL, or perform a sleeping bus transaction. Explain a suitable deferred design and configuration caveats.

9. D09 8 points: explain how an RTOS task and ISR exchange a completed DMA buffer without either overwriting data still owned by the other.

10. D10 8 points: using fixtures/spin.trace, identify one supported hypothesis and one unsupported conclusion. Propose a fix and a regression test.

11. D11 7 points: distinguish CRC, signed update verification, and verified boot. Give a failure each one does not solve alone.

12. D12 7 points: explain one real project or a clearly labeled practice task: requirement, your work, a failure, verification, and a remaining limitation. Do not invent experience.

Interpretation: use subtopic evidence. Below 60: prioritize M01–M05 and B01; 60–79: repair failed foundations before track depth; 80 or above: attempt the target track's phase gate. These ranges are planning aids, not a proficiency score for untested topics. Any critical bounds, lifetime, synchronization, trust, or truthfulness failure requires remediation regardless of total. Retake only after explaining and testing the correction.

## Final practical and capstone defense

### FINAL P Four hour practical

Prerequisites: G01–G05 plus one minimum-complete capstone path. Time: four hours active work, excluding documented image-download time. Tools: editor, compiler, debugger, local official documentation and existing project source; no AI or answer keys. The practical can run entirely in host simulation. Actual hardware observations remain a separate evidence layer.

Build or adapt a bounded Linux to MCU telemetry demonstrator. Use one primary simulated serial path, a versioned message with maximum 32-byte payload, explicit CRC parameters, monotonic freshness, persistent configuration and observable health. Choose C01 gateway or C02 simulated robot. Reuse existing components only if you can explain and modify them. C02 must start disarmed and enforce a local stale-command transition; no real motors are needed.

Use this fixed synthetic event sequence, recording actual execution rather than copying expected claims:

1. At start, load a valid configuration with sample interval 100 ms and maximum queue eight. Reject invalid interval zero and queue size beyond the implemented maximum.

2. Send 20 valid numbered records; verify order and accepted/processed counts.

3. Split one frame at every possible byte boundary in separate test runs.

4. Inject one bad CRC, one excessive length, one unknown version and one truncated frame. The parser must stay bounded and expose distinct diagnostics where specified.

5. Pause the consumer long enough to fill the queue. Apply and count the documented overflow policy.

6. Disconnect the peer, then restart it with a new session ID. Recover without accepting a stale command from the previous session.

7. Interrupt a configuration write before replacement using a controllable fault hook. Restart and select a valid old or new configuration according to the documented atomicity contract.

8. For C02, stop command updates beyond the configured local deadline, confirm output permission becomes false, and require explicit re-arm after recovery. For C01, mark data stale and keep health observable.

9. Request shutdown while waiting for input and while queued work exists. Terminate within the test's two-second host deadline under the defined drain/discard policy.

Submit source revision, requirements table, architecture/interface description, test runner, exact commands, raw outputs, a validation matrix and a five-minute demo script. Label each result authored/build/host/emulator/hardware/pending. No benchmark is required unless actually measured; proposed targets are not results.

Scoring out of 100: correctness 25, protocol/state reasoning 15, debugging and failure diagnosis 15, bounded resources 10, concurrency/shutdown 10, tests and independent oracles 15, communication/evidence 10. Pass at 80 with no critical gate failure and at least half in every category. Critical gates: no supplied-input bounds error, indefinite required shutdown, uncontrolled resource growth, stale C02 motion permission, fabricated evidence, or bypass of configured validation. A missing board is not a failure of the host practical, but target validation remains pending.

Remediation: isolate the failed requirement, reproduce it with a minimal input, correct the implementation, add a regression, then retake with a changed event order and new session identifiers. Reading the reference answer alone does not satisfy remediation.

### FINAL D Sixty minute capstone defense

Prerequisites: FINAL P evidence or an honestly documented incomplete practical. Tools: your architecture diagram, requirements matrix and raw evidence; no AI. Spend 10 minutes on requirements and boundaries, 15 on one code path from input to output, 15 on injected failure and recovery, 10 on a live requirement change, and 10 on limitations and deployment gaps.

The reviewer chooses five prompts: where is ownership transferred; which queue can fill; what wakes each blocked component; what survives restart; how does a malicious or corrupt length fail; what happens on peer reset; which measurements are host-only; what would need hardware validation; which AI suggestion was rejected; what changes for a real product?

Score architecture 20, code understanding 20, failure reasoning 20, live change 20 and evidence honesty 20. Pass at 80 with no critical failure. The live change can alter maximum payload from 32 to 24, queue policy from drop-newest to reject-producer, or timeout from 250 to 100 ms; update requirements, tests and explanation consistently. Do not silently patch only one constant.

## Phase exit tests

Pass each gate before relying on its topics in a capstone. These are independent assessments with no AI or answer keys. Local official documentation is allowed, except the oral explanation must be given without notes. Score with the 100-point practical rubric; pass at 75 with every stated critical gate satisfied. Use the actual project versions and preserve evidence.

### G01 Foundations

Prerequisites: M01–M04; B01 and compiler/shell basics. Time: 75 minutes.

Task: Implement Q021 and Q024; explain Q006 and Q016; inspect one ELF with readelf and show a clean Git diff.

Test data and conditions: Inputs: multiplication boundaries; hex 00af, odd length, invalid G, insufficient capacity; structure with char then uint32_t.

Deliverables: C source/tests, ELF inspection transcript, explanation of UB and padding.

Critical gate: Bounds and unchanged-failure behavior are mandatory.

Remediation: Repeat M02 integer/lifetime exercises and B01 with changed capacities.

### G02 Linux systems

Prerequisites: M05–M07; B02–B04. Time: 100 minutes.

Task: Implement a bounded line reader and drain-on-shutdown queue. Diagnose spin.trace and explain a descriptor reuse race.

Test data and conditions: Chunks ab then c\nx\n; overlong 65-byte line for a 64-byte limit; shutdown while empty and full.

Deliverables: Source, PTY/pipe or injected-read tests, thread shutdown transcript, failure table.

Critical gate: No silent truncation, data race or indefinite required shutdown.

Remediation: Repeat partial-I/O lab and condition-variable exercise before retesting.

### G03 Images and drivers

Prerequisites: M08–M11 and M14; B05 and at least one image/driver project. Time: 120 minutes.

Task: Show target build metadata and a boot log if available; review a driver user-copy path and device removal; explain a DT supplier failure.

Test data and conditions: A short user buffer, concurrent readers, remove during blocked read, wrong compatible string.

Deliverables: Build evidence, source review, matching kernel config and an explicitly pending hardware matrix.

Critical gate: No unsafe user pointer or asynchronous use-after-free; never invent a boot log.

Remediation: Repeat I02/I03 lifecycle tests in a disposable VM and verify target headers.

### G04 Firmware and RTOS

Prerequisites: M12–M13; I05 or a host-tested firmware logic core. Time: 100 minutes.

Task: Implement Q163–Q165; draw ISR-to-task buffer ownership; explain startup/linker agreement and priority inversion.

Test data and conditions: Heartbeat unseen/stale/current; FAULT then ARM; stale plausible sensor; wraparound ticks.

Deliverables: Portable tests, state table, stack/timing measurement plan and pending board checks.

Critical gate: Stale command disables permission and fault never clears implicitly.

Remediation: Repeat explicit state machine, watchdog and RTOS ownership lessons.

### G05 Production and diagnosis

Prerequisites: M15–M17; A02/A03 concepts and fault tests. Time: 100 minutes.

Task: Analyze a latency claim, review an update transition matrix, and implement or demonstrate atomic configuration replacement in a disposable directory.

Test data and conditions: Storage full, interrupted temporary write, invalid signature, candidate health timeout, rollback data mismatch.

Deliverables: Failure matrix, raw available measurements, update/configuration evidence, limitations.

Critical gate: No forged benchmark, unauthorized update acceptance or destruction of the only recovery path.

Remediation: Revisit measured-versus-proven timing, persistence sequencing and A03 recovery.

### G06 Capstone integration

Prerequisites: M01–M18 relevant track coverage; One capstone at minimum complete scope. Time: 180 minutes plus 45-minute defense.

Task: Run the chosen capstone from its documented setup; inject disconnect, malformed input, consumer overload and peer restart; explain one independent change.

Test data and conditions: Use fixed seed 2710 and record exact injected event times. For C02 include stale command and re-arm behavior.

Deliverables: Reproducible demo, requirement-to-test table, results, evidence gaps and 5-minute architecture explanation.

Critical gate: No undocumented control continuation, fabricated pass, unbounded queue or unexplained lifecycle fault.

Remediation: Repair the failing requirement, add regression, then retake with changed event order.

## Role mock interviews

Run one role mock per week during revision. Each lasts 75 minutes and is independent: no AI, hidden assistance, or answer key. Local compiler and official documentation are allowed only during the practical section if the interviewer states so before starting. Use a partner or record yourself; a self-review is still labeled self-review. Submit recording or notes, source/test output, scores, and three concrete weaknesses. Pass at 75/100 with all critical gates and at least half the points in every section. Retake with different bank questions after remediation.

### MOCK L Embedded Linux developer

Prerequisites G01–G02 and a system-programming project. First 5 minutes: clarify a telemetry daemon's input rate, maximum record, shutdown and persistence requirements. Next 15 minutes: Q052 partial I/O, Q059 deadlines, Q071 condition predicates. Next 25 minutes: implement strict numeric parsing and a blocking complete-write helper, using inputs from C15 and a pipe. Next 20 minutes: diagnose spin.trace, explain a lost-wakeup scenario and define regression tests. Final 10 minutes: defend B03 or B04 and accurately label host versus hardware evidence.

Score requirements/communication 10, concepts 20, coding 30, debugging 25, project defense 15. Critical gates: no unchecked length/overflow, unsynchronized shared access, endless EOF loop or invented test result. Remediation: M03, M06, M07 and M15 according to the failed section.

### MOCK K Kernel driver and BSP developer

Prerequisites G01–G03 and I02 or I03. First 5 minutes: establish kernel version/configuration, target and hardware enumeration. Next 15 minutes: Q105 bindings, Q114 locking and Q116 managed lifetimes. Next 25 minutes: review a proposed read path that copies to userspace under a spinlock and consumes a FIFO before checking copy result; write a corrected design with blocking/nonblocking and removal behavior. Next 20 minutes: diagnose kernel_oops.txt and a deferred-probe dependency. Final 10 minutes: defend the actual driver build and runtime evidence, including pending target tests.

Score requirements/communication 10, concepts 20, driver review 30, debugging 25, defense 15. Critical gates: no direct user-pointer dereference, sleeping in an incompatible context, callback-after-free or false target validation. Remediation: M08–M11; rerun lifecycle tests in a disposable VM.

### MOCK F MCU firmware and RTOS developer

Prerequisites G01, G04 and I05. First 5 minutes: clarify MCU, clocks, rates, deadlines and reset behavior. Next 15 minutes: Q141 startup, Q153 ISR APIs and Q155 stack evidence. Next 25 minutes: implement Q164 state transitions and Q161 sequence comparison, with wraparound, FAULT then ARM, and explicit clear tests. Next 20 minutes: analyze fault_summary.txt and ISR-to-task buffer ownership. Final 10 minutes: defend firmware architecture, watchdog policy and the difference between host logic tests and board timing evidence.

Score requirements/communication 10, concepts 20, coding 30, debugging 25, defense 15. Critical gates: stale/FAULT state cannot permit motion, ISR cannot block on task mutex, shared buffer ownership cannot overlap, no fabricated hardware results. Remediation: M12–M13 and the matching I05 test.

### AI assisted comparison mock

After one independent mock, repeat a changed problem with AI explicitly permitted. Keep prompts, edits, rejected advice and verification. Score engineering quality using the same rubric but label it assisted. Within 48 hours, explain the result and repair one injected defect without AI. The assisted score must not replace the independent score in the tracker.

## Timed coding tests

Each test is independent and uses C11 on the host unless explicitly stated. Allowed tools are compiler, editor, local man pages, debugger and sanitizers; no AI or answer keys. Submit source, commands, results and a short explanation. Pass at 75/100 with no critical failure. Apply correctness 30, reasoning 15, debugging 15, resource handling 10, state handling 10, tests 10, communication 10.

### C15 Checked conversion in 15 minutes

Prerequisites M02–M03. Implement parse_u32 from Q028 with strict decimal grammar and unchanged output on failure. Inputs: empty, 0, 4294967295, 4294967296, 12x, -1,  1. Record exact expected outputs before running. Extension only after the minimum passes: accept a separate explicit base parameter. Exit evidence is a strict-warning build and boundary tests. Remediation: integer conversion and precondition checks in M02, then repeat with uint16_t.

### C30 Ring buffer in 30 minutes

Prerequisites M03. Implement capacity eight FIFO push/pop with full/empty rejection, no heap, and unchanged state after failed operations. Fill 0–7; reject 8; pop three; push 8–10; drain expected 3–10; reject an empty pop. State whether concurrent use is supported. Do not add unrequested lock-free behavior. Remediation: draw head/tail/count after each step, then repeat at capacity three.

### C60 Fragmented protocol in 60 minutes

Prerequisites M03 and M06. Implement a streaming decoder for one-byte type, one-byte length and up to 16 payload bytes. It must accept arbitrary fragmentation and multiple records per chunk; reject lengths above 16; reset by explicit API after a malformed header. Inputs in fragments.json plus an empty chunk, a 16-byte payload, and length 17. Return consumed length and complete frames without overrunning output. The reset rule deliberately does not claim automatic resynchronization. Tests must exercise every split of one valid frame. Remediation: state diagrams and remaining-length invariants, then add a delimiter-based recovery extension only after the minimum passes.

### C90 Bounded queue in 90 minutes

Prerequisites M07. Write a POSIX producer/consumer queue of capacity four with one producer, one consumer, 100 numbered items, and drain-on-shutdown. No item accepted before shutdown may be lost or duplicated. Stop once while empty and once while full using synchronization barriers rather than sleeps. Join threads before destroying queue storage. Test termination within a specified two-second host test deadline; this is a test bound, not a real-time guarantee. Remediation: condition-variable predicates and cancellation wake paths.

### C60F Firmware state logic in 60 minutes

Prerequisites M12–M13. Implement Q164 robot states with events ARM, DISARM, COMMAND, TIMEOUT, FAULT, CLEAR. Start DISARMED; timeout disarms; FAULT latches; ARM cannot clear it; CLEAR returns to DISARMED. Output permission must be false unless ARMED. Use a table-driven test of every state/event pair and a wraparound deadline test. No real motor output is required. Remediation: explicit transitions and stale-command policy; retake with a new RECOVERY state and documented allowed transitions.

## Topic retrieval quizzes

Each module quiz takes 20 minutes after completing that module. Allowed tools: plain text editor or paper only; no AI, answer bank or external references. For coding/design prompts, supply an algorithm, invariants and test cases; full execution is assessed in the timed practical suite. Submit three answers with assumptions and one uncertainty. Each answer earns 0–4 using its bank criteria; pass at 9/12 with no critical misconception. Failed quizzes route to the linked module lesson and a changed-example retake. These are short-answer retrieval tests, not multiple-choice guessing.

### T01 Module M01

Prerequisite and remediation: Module M01.

1. Q041: Explain preprocessing, compilation, assembly, and linking using one missing-symbol error. Include this boundary: How would C++ name mangling affect linking a C library?

2. Q049: How do you use git bisect without misdiagnosing an intermittent fault? Include this boundary: How would you handle a bisect step that cannot build with the current host toolchain?

3. Q050: What do compiler warnings, static analysis, and sanitizers each contribute? Include this boundary: How would you design input generation to exercise a parser beyond its normal examples?

### T02 Module M02

Prerequisite and remediation: Module M02.

1. Q001: How are an array and a pointer different when passed to a function? Include this boundary: How would you express read-only access and document whether null is valid at length zero?

2. Q006: Is printf("%d %d", i++, i++) guaranteed to print a particular pair? Include this boundary: Give an expression with unspecified evaluation order that does not have undefined behavior.

3. Q018: Why does volatile not fix a shared-counter data race? Include this boundary: When would acquire-release ordering be needed instead of a relaxed counter increment?

### T03 Module M03

Prerequisite and remediation: Module M03.

1. Q023: Implement an eight-byte ring buffer with explicit full and empty states. Include this boundary: How would a single-producer single-consumer atomic implementation publish an element safely?

2. Q028: Parse a decimal uint32_t with no trailing characters or overflow. Include this boundary: How would you support optional base prefixes without silently accepting octal unexpectedly?

3. Q039: Implement hysteresis for a high-temperature alarm. Include this boundary: What should happen when the sensor reading is invalid or stale?

### T04 Module M04

Prerequisite and remediation: Module M04.

1. Q043: What is the difference between an ABI and an API? Include this boundary: Which ABI evidence would you collect when a cross-compiled program crashes immediately?

2. Q044: How does RAII improve resource handling in C++ system code? Include this boundary: How would you expose an explicit finalize operation for a resource whose release can fail?

3. Q047: Why can a static library link fail depending on command-line order? Include this boundary: What do archive groups change, and why can excessive circular dependencies signal a design problem?

### T05 Module M05

Prerequisite and remediation: Module M05.

1. Q051: How do a process, a thread, and a file descriptor differ? Include this boundary: How can close followed by open in another thread create a subtle reuse bug?

2. Q101: How is a Linux application processor different from a typical microcontroller target? Include this boundary: Which portions of a packet parser could be shared unchanged across both targets?

3. Q102: What do an MMU and an MPU each provide? Include this boundary: How would a DMA engine's access differ from CPU accesses checked by an MPU?

### T06 Module M06

Prerequisite and remediation: Module M06.

1. Q052: Why must a read or write loop handle partial progress? Include this boundary: What timeout should cover the whole operation instead of restarting after every short transfer?

2. Q055: How should a daemon shut down cleanly after SIGTERM? Include this boundary: How would you guarantee that a worker blocked on a condition variable notices shutdown?

3. Q059: Why should timeouts normally use a monotonic clock? Include this boundary: How would you define whether a command timeout should elapse while a laptop is suspended?

### T07 Module M07

Prerequisite and remediation: Module M07.

1. Q071: Why must a condition variable be paired with a predicate and mutex? Include this boundary: When should shutdown broadcast rather than signal one waiter?

2. Q074: What is priority inversion, and what does priority inheritance solve? Include this boundary: What would you measure to separate inversion from an intrinsically long critical section?

3. Q077: Why is cancellation harder than adding a stop flag? Include this boundary: How would you guarantee exactly one completion callback for a request canceled at the same time it succeeds?

### T08 Module M08

Prerequisite and remediation: Module M08.

1. Q103: Describe a representative embedded Linux boot path without assuming every board uses U-Boot. Include this boundary: What evidence distinguishes a kernel decompression failure from a missing root filesystem?

2. Q105: What belongs in device tree, and what belongs in a driver? Include this boundary: Why is a binding a compatibility interface that deserves review beyond one driver revision?

3. Q108: What does a cross-compilation sysroot contain, and why is it important? Include this boundary: How can pkg-config accidentally supply host include paths during a cross build?

### T09 Module M09

Prerequisite and remediation: Module M09.

1. Q112: What are process context and interrupt context in kernel code? Include this boundary: Which context would you choose for an I2C transaction triggered by a GPIO interrupt?

2. Q114: How do a mutex and a spinlock differ in the kernel? Include this boundary: Why might holding a spinlock across copy_to_user be invalid?

3. Q116: Why does devm allocation not eliminate the need for a removal design? Include this boundary: How would you handle a user process keeping a character device open during unbind?

### T10 Module M10

Prerequisite and remediation: Module M10.

1. Q123: A read method directly dereferences the user's buffer pointer. What is wrong? Include this boundary: How would you avoid consuming an entire FIFO item when copying it to userspace fails partway?

2. Q125: Poll reports readable but the next read blocks. Is the driver necessarily wrong? Include this boundary: What ordering avoids missing a state change between checking readiness and registering a waiter?

3. Q129: A device is removed while a process is blocked in read. What behavior must the driver define? Include this boundary: How would you make close safe after the underlying hardware has disappeared?

### T11 Module M11

Prerequisite and remediation: Module M11.

1. Q134: Why does IIO expose raw values and scale rather than only a floating-point value? Include this boundary: How would you validate a signed 12-bit sample stored in a 16-bit scan element?

2. Q137: Why is a DMA address different from a CPU virtual or physical address? Include this boundary: How would you test a driver on a system with an IOMMU to expose hidden address assumptions?

3. Q139: How do system suspend and runtime suspend differ? Include this boundary: Which information is lost when a sensor's power rail is removed rather than merely clock-gated?

### T12 Module M12

Prerequisite and remediation: Module M12.

1. Q141: What must startup code establish before calling main on a microcontroller? Include this boundary: How would you diagnose a fault before main using the reset handler and linker map?

2. Q143: How should an interrupt service routine be structured? Include this boundary: How would you handle a burst larger than the ISR-to-task queue capacity?

3. Q147: How should a watchdog be integrated into a multitask firmware system? Include this boundary: What failures cannot a software-supervised watchdog detect if all tasks keep sending stale heartbeats?

### T13 Module M13

Prerequisite and remediation: Module M13.

1. Q153: Why can a mutex be unsuitable in an interrupt handler? Include this boundary: What does a higher-priority-task-woken flag mean for the return-from-interrupt path?

2. Q157: What must a Linux to MCU protocol specify beyond a payload structure? Include this boundary: How would each peer recover when the other reboots and its sequence counter restarts?

3. Q158: How do you make a command timeout meaningful for a robot demonstrator? Include this boundary: Should a recovered link automatically re-arm motion, or require an explicit operator action?

### T14 Module M14

Prerequisite and remediation: Module M14.

1. Q171: How would you choose between Buildroot and Yocto for a learning or product image? Include this boundary: Which evidence would justify switching build systems after an initial prototype?

2. Q172: What do a Yocto layer, recipe, and image each represent? Include this boundary: How would you package one telemetry service in your own layer without editing upstream recipes?

3. Q173: What belongs in a Buildroot external tree for a custom appliance? Include this boundary: How would you prove a clean build includes your application without manual post-build copying?

### T15 Module M15

Prerequisite and remediation: Module M15.

1. Q081: A daemon uses 100 percent CPU after its serial peer disconnects. What do you inspect? Include this boundary: How would you avoid a new busy loop if reconnect itself fails immediately?

2. Q088: A periodic task drifts farther behind each minute. What timing pattern is suspect? Include this boundary: What policy would you choose if one cycle takes three times the nominal period?

3. Q098: A DMA buffer contains stale data on one architecture. What could host testing have missed? Include this boundary: How would an IOMMU make a physical-address assumption fail?

### T16 Module M16

Prerequisite and remediation: Module M16.

1. Q176: How do signed updates differ from secure or verified boot? Include this boundary: How would you rotate an update signing key without stranding deployed devices?

2. Q177: What must an A/B updater do before marking a new slot good? Include this boundary: What health tests should run locally when the network is unavailable?

3. Q180: What makes a reliability test more useful than leaving a device running overnight? Include this boundary: How would you make a storage-full failure test verify both diagnosis and recovery?

### T17 Module M17

Prerequisite and remediation: Module M17.

1. Q181: Design a bounded industrial sensor gateway using one MCU and one Linux computer. Include this boundary: How would you demonstrate that disconnecting the network cannot block MCU acquisition indefinitely?

2. Q186: Design a reliable serial protocol for commands and telemetry with 128-byte payloads. Include this boundary: How would a firmware update preserve compatibility with an older Linux peer?

3. Q190: Design a test strategy for a driver and firmware pair before boards arrive. Include this boundary: Which failure would you expect a PTY test to reveal, and which requires a logic analyzer?

### T18 Module M18

Prerequisite and remediation: Module M18.

1. Q191: How would you explain a project you built with AI assistance honestly? Include this boundary: Which AI suggestion did you reject, and what evidence led you to reject it?

2. Q196: An AI suggests a kernel helper that is absent from your headers. What is your workflow? Include this boundary: How would you distinguish a missing Kconfig dependency from a completely invented API?

3. Q197: How can AI-generated tests give false confidence? Include this boundary: Which single mutation would reveal whether your overflow test actually exercises the overflow branch?

# 6 Interview strategy and portfolio

## Interview day checklist

Before the interview, confirm its format, duration, time zone, coding environment and permitted tools. Test audio, screen sharing and the editor. Keep the actual JD, truthful resume and a short project index available if allowed. Open no answer banks or AI assistance unless explicitly permitted.

During each task, clarify the contract, state assumptions and explain the invariant before implementing. Test boundary and failure behavior. Pause when evidence contradicts a hypothesis. Distinguish knowledge from inference and describe unfinished work accurately. Ask whether the interviewer wants deeper implementation or a broader tradeoff discussion.

For a project demonstration, have a deterministic command and retained logs ready. Show exactly what ran and state hardware or deployment limits. Prepare one real debugging example and one design you changed after evidence.

Afterward, record uncertain topics and missed cases while they are fresh. Repair them with a concrete exercise. Do not infer guaranteed acceptance or rejection from one question or from conversational tone.

## Interview preparation playbook

Use this playbook after demonstrating the foundations. The aim is to explain, implement, debug and defend a small system under stated constraints. Use the questions-only bank before its answer key. No employer JD was supplied; the examples below are preparation methods, not claims about a company's hiring process.

### Prioritize an actual job description

Copy each explicit requirement into the skills matrix and preserve its wording and source. Separate required skills, preferred skills and your own inferred prerequisites. “Linux development” does not automatically mean kernel programming; determine whether the work concerns applications, drivers, BSP integration or firmware.

Select five requirements most central to the described work. For each, write your strongest evidence, one missing capability and the smallest task that could expose that gap. Mark untested skills “not assessed.” Start with required capabilities you cannot yet demonstrate, then deepen relevant strengths. Revisit this order after a diagnostic or mock; do not spend scarce revision time expanding an unrelated stack.

| Primary role | Depth to emphasize | Evidence to prepare |
| --- | --- | --- |
| Embedded Linux applications | M06 M07 M14 M15 M16 | Partial I/O, service lifecycle, concurrency, image integration and C01 |
| Kernel and BSP | M08 M09 M10 M11 | Matching build, driver contexts, lifetimes, device descriptions and tested interfaces |
| MCU firmware | M12 M13 | Startup/map, ISR/task boundaries, queues, watchdog and target measurements |

Keep C, ownership, debugging and honest evidence common to all tracks. A simulated demonstration cannot substantiate a claim of physical board bring-up.

### Answer a conceptual question

Give the definition, the governing rule, a concrete example, and a limit. Aim for an initial answer of about one minute, then expand in response to follow-ups.

For a condition variable: “It lets a thread wait for a shared-state predicate. I hold the associated mutex and recheck the predicate in a loop because another consumer can act before I regain the lock. In my bounded queue, the predicate is nonempty or producers finished. The notification itself does not store an item.” This explains behavior the interviewer can challenge. An unexplained list of API names does not.

State assumptions when the answer depends on C dialect, architecture or kernel release. For undefined-behavior questions, identify the violated rule before discussing an observed compiler output. Practice Q001–Q020 and Q071–Q080, then change the example without consulting the answer.

### Solve a coding problem

For a 60-minute exercise, reserve five minutes for requirements, five for the invariant and tests, thirty for implementation, fifteen for verification and five for explanation. Adjust the allocation when the interviewer changes the scope.

Clarify input bounds, null/empty behavior, ownership, error reporting, concurrency and permitted libraries. State the simplest correct invariant before coding. For a ring buffer, explain how full and empty differ; for a complete-write helper, explain how progress survives a short write. Implement the normal path and error paths together. Test zero, one, maximum, malformed input and an appropriate resource failure. Explain unfinished work precisely instead of concealing it. Use Q021–Q040, Q061–Q070 or Q161–Q170 according to the role.

### Investigate a failure

Describe the symptom and its boundary, list observed facts, rank two plausible causes, and choose one observation that separates them. Do not announce a root cause before collecting evidence.

For a serial daemon spinning after disconnect, inspect syscall results and poll events before changing queue sizes. Repeated immediate hangup/error events can explain a busy loop; a trace should distinguish that from an application that never waits. After correcting the termination behavior, reproduce disconnect and normal traffic and check resources. Preserve a regression that fails on the old code. Practice Q081–Q100 and driver scenarios Q121–Q130 using this evidence sequence.

### Develop a design answer

First agree on rates, latency, memory, persistence, failure assumptions and deployment scope. Draw only the components needed for those requirements. Give every buffer, connection and persistent state an owner. Explain overload, startup, shutdown, restart, corrupted input and update interruption. Define what is measured and what result would fail acceptance.

For a Linux–MCU gateway, say which side controls timing, how message freshness is decided and what happens when Linux stops responding. Avoid claiming a watchdog solves every failure. Compare alternatives using the actual constraint: a queue may simplify ownership; a larger queue may worsen stale-data latency. Use Q181–Q190 and defend one tradeoff under a changed requirement.

### Handle unknowns and explain your contribution

A useful response is: “I have not used that exact API. I would first check its execution-context and lifetime contract in the selected release. My current hypothesis is …; this test would distinguish it from ….” Then reason from known principles. Repeated guessing weakens an otherwise sound answer.

For project discussions, state the problem, your own changes, one difficult failure, the evidence behind the fix and remaining limits. Distinguish reference code, dependencies, teammate contributions and AI assistance. Never turn package test results into personal achievements. For behavioral questions, describe a real situation, your responsibility, actions, observed outcome and what you changed afterward; leave outcomes unquantified when you lack records.

### Run a mock and use its feedback

Use the 30-day calendar, revision sheets and interview-day checklist. Ask the reviewer to record specific incorrect statements and missed test cases. Select at most two weaknesses for the next session, repair them, then retest with different inputs. Fluent recall alone does not pass the readiness gate.

## Concise revision sheets

Use these prompts for retrieval, then check the linked module and source references. Explain an example aloud; merely recognizing the words is insufficient.

### C and ownership

Name the owner, capacity, valid length and lifetime of each pointer. Array-to-pointer conversion loses size information. Check allocation multiplication before allocating. Unsigned wrap can violate an application contract; signed overflow can invalidate language-level reasoning. const restricts modification through a particular access path; volatile supplies neither thread synchronization nor general atomicity. See M02 and Q001–Q040.

### Linux I/O and concurrency

Separate progress, EOF, timeout, interruption and failure. A readiness event is not a message boundary. Use a monotonic deadline and recompute remaining time after interruption. Trace every descriptor through creation, duplication, exec and cleanup. Protect queue invariants with the same synchronization; wait in a predicate loop, publish shutdown and wake relevant waiters. A notification does not own the data. See M06–M07 and Q051–Q100.

### Kernel and platform

Before choosing a helper, identify context, sleepability, lock rules and lifetime. User-copy helpers can fault and report residual bytes; their wrapper's return contract may differ. Managed resources do not automatically preserve state referenced by open files. Device tree describes hardware under binding rules. A matching object build does not prove loadability, correct teardown or physical behavior. See M08–M11 and Q101–Q140.

### Firmware and RTOS

Explain reset vectors, stack placement, initialized-data copying and zeroing. Know the actual device's interrupt-priority convention. Bound ISR work and use the correct ISR-safe API. Queueing a pointer requires a payload ownership rule. Priority inheritance does not prevent deadlock. Feed a watchdog on meaningful application progress. Distinguish measured latency from a proven bound. See M12–M13 and Q141–Q170.

### Images updates and evidence

Separate recipe, layer, image, machine and distribution policy. Pin source inputs and preserve configuration. Atomic replacement and durable persistence need an explicit storage failure model. Signing, secure boot, health acceptance and downgrade protection solve different parts of the trust/recovery problem. Report authored, compiled, host-tested, emulated and hardware-tested separately. See M14–M17 and Q171–Q200.

## Thirty day revision calendar

This is revision for a learner who already passes the C and basic Linux practical gates. It does not compress the full 52-week program into one month. Choose a primary role using an actual JD when available.

Follow Thirty_Day_Revision.csv. Normal days take 90 minutes: 15 minutes of closed-book retrieval, 45 minutes of coding or investigation, 20 minutes of verification and 10 minutes updating the error log. From a question range, select two relevant oral questions or one coding task; the range is a selection pool, not a demand to memorize every entry that day.

Days 7, 14, 21 and 28 are 120-minute mock blocks: 60 minutes interviewed, 30 minutes evidence-based review and 30 minutes of targeted rework. Each seven-day block therefore takes 11 hours. Days 29 and 30 add three hours, giving 47 hours across 30 days. The mocks replace the ordinary daily session.

| Mock | Interview focus | Required output |
| --- | --- | --- |
| Day 7 | Shared C, Linux I/O and one failure | Code, tests and two corrected explanations |
| Day 14 | Primary-track technical depth | Context/lifetime or firmware timing defense with explicit unknowns |
| Day 21 | System design and project defense | Architecture, failure table and acceptance criteria |
| Day 28 | Hiring simulation with a changed requirement | Independent implementation/debugging evidence and a readiness decision |

Keep answers closed during independent work. For an assisted session, record assistance explicitly and perform a later independent retest. When a critical defect remains, replace the next discretionary topic with remediation; do not conceal it behind calendar completion. If an interview is nearer, prioritize the actual role and one defensible project, then state the gaps left open.

## Portfolio and job readiness

Build the portfolio from work you can reproduce and explain. Supplied reference implementations and logs establish what this package contains; they do not establish your contribution or proficiency. Use Readiness_Gate.md before treating a topic as interview ready.

### Project README template

For one strong project, replace each bracketed field with your own evidence. These are intentional personal-evidence fields, not implied achievements.

Project and problem: [specific user or engineering need]

Scope: [implemented behavior and explicit limits]

My contribution: [files, decisions, tests and investigations I performed]

Dependencies: [upstream components, versions and attribution]

Architecture: [components, interfaces, owners and persistent state]

Build and run: [commands from a clean checkout]

Acceptance: [requirement ID, stimulus, expected result, test]

Results: [observed outcome, environment and evidence path]

Failure investigation: [symptom, hypotheses, discriminating test, fix]

Limitations: [pending target, timing, security or durability checks]

Next improvement: [one justified change and its proposed test]

Use one diagram when it clarifies interfaces or failure boundaries. Keep raw logs and measurement data alongside the summarizing explanation. Record whether an execution used a host, user-mode emulator, full guest or physical board; those environments provide different evidence.

### Ten minute demonstration

Spend one minute on the problem and scope, two on architecture and ownership, three on a reproducible normal run, two on an injected failure and recovery, and two on results and limitations. Prepare a fallback log in case the environment is unavailable, and label playback as recorded evidence. The demonstration should show why a decision matters, not just that an executable prints output.

For B03, a useful defense is fragmentation, cancellation cleanup and protocol freshness limits. For I02, explain wait predicates and lifetime, then distinguish object compilation from guest execution. For a capstone, connect each mandatory requirement to its acceptance test. Choose a project aligned with the target role rather than presenting fifteen shallow demos.

### Evidence inventory

Maintain a row per claim: claim, your contribution, source commit, environment, command, input, observed result, raw artifact, reviewer and pending checks. If a number lacks a measurement record, omit it. Keep failed attempts when they explain a decision; they are valuable evidence of engineering judgment.

### Resume and project explanation templates

Use: “Implemented [component] in [language/platform] to [required behavior]; verified [specific cases] using [method].” For debugging: “Investigated [observed failure], identified [mechanism] through [evidence], and added [regression].” For measurements: “Changed [design] and observed [metric] from [baseline] to [result] under [stated workload].” Fill these only after doing the work and retaining evidence.

An honest learning-project bullet may mention a simulation or educational driver. Do not convert it into production deployment, customer impact, years of experience or hardware validation. List AI assistance and reused components accurately when asked about contribution. Prepare to modify a small requirement live and explain why the tests still apply.

## Readiness gate

Apply this gate to one target role and actual JD where available. It is a preparation decision, not a prediction of selection. Use unfamiliar inputs and keep the first attempt independent.

| Dimension | Evidence | Points |
| --- | --- | --- |
| Correctness and contracts | Working coding task with boundary/error cases | 25 |
| Reasoning and debugging | Competing hypotheses, discriminating evidence and regression | 20 |
| Ownership and concurrency | Accurate lifetime, synchronization and shutdown explanation | 20 |
| Role depth | Defensible kernel, Linux-system or MCU task relevant to the JD | 20 |
| Communication and honesty | Clear contribution, assumptions, limits and source attribution | 15 |

Use 80/100 as a local preparation threshold, require at least half the points in every dimension, and retain the assessor's reasons. Any uncorrected out-of-bounds access, dangling lifetime, deadlock, unsafe command-authority behavior or fabricated evidence fails the gate regardless of average score.

Also require one clean-build demonstration, one failed-input recovery, one project defense and one modest change made without copying the reference. A strong oral score cannot replace these practical checks. Hardware-dependent JD requirements remain gaps until the corresponding evidence exists.

After a failure, record the defect and linked module, implement a focused correction, and retest with changed data in a later session. Mark outcomes as ready for the tested scope, needs remediation, or not assessed. Reassess when the target JD or project environment changes.

# 7 AI integration

## AI workflows and reusable prompts

Use these prompts after an independent attempt. Replace bracketed fields with verified context. Record assistance in the study log, and follow the allowed-tool rules of each assessment.

### Workflow 1 Socratic concept tutor

Context to supply: Your current explanation, a small example, module and target architecture.

Prompt:

Act as a Socratic tutor for [concept]. Here is my explanation: [text]. Ask one question that reveals the most consequential misunderstanding. Do not give a full solution yet. After my answer, use one concrete counterexample and ask me to predict the result. Distinguish C language rules, compiler behavior and hardware behavior.

Verification: Explain a changed example without the conversation; check normative claims in the named official reference.

Common failure: A fluent analogy can hide incorrect ordering, ownership or version assumptions.

### Workflow 2 Read unfamiliar source

Context to supply: Repository commit, exact files, entry point and the behavior you want to trace.

Prompt:

Trace [event] through these supplied source files at commit [commit]. Name the entry point, key state changes, resource owners, locks and error paths. Cite file and function names that actually exist in the supplied code. Mark any unavailable dependency as unknown. Finish with three observations I can collect to check your trace.

Verification: Search the source for every named function; step through or instrument one end-to-end transaction.

Common failure: Invented helper functions and unseen dependencies can make a false architecture look plausible.

### Workflow 3 Find the correct versioned API

Context to supply: Kernel/tool version, declaration you found, required context and compiler diagnostics.

Prompt:

For [exact version], identify the official contract for [API]. State whether it can sleep, its return-value convention, ownership and cleanup requirements. Give the official URL or exact source path and quote only the minimal declaration needed. If you cannot inspect that version, say so and provide a search plan instead of inventing a signature.

Verification: Open the versioned source/header yourself and compile a minimal use under the actual configuration.

Common failure: An API can exist on a newer branch but have a different signature or behavior on the target.

### Workflow 4 Review a design before coding

Context to supply: Requirements, architecture, data ownership, execution contexts and target limits.

Prompt:

Review this design against the stated requirements. Identify the three most consequential correctness risks, each with a failure sequence and a test that would expose it. Check buffer ownership, concurrency, blocking, restart and teardown. Separate required changes from optional complexity. Do not rewrite the system until the interface contracts are clear.

Verification: Walk a normal operation and a failure through every resource owner; implement a minimal risk-focused test.

Common failure: A review can recommend sophisticated mechanisms that solve no requirement and introduce new lifetimes.

### Workflow 5 Adversarial code review

Context to supply: Complete changed function plus callers, tests, protocol and build flags.

Prompt:

Review only this change and its relevant callers. Look for bounds errors, integer overflow, partial I/O, stale state, deadlock, races and incomplete cleanup. For each finding give a concrete input or interleaving and the affected requirement. Do not report style preferences as bugs. State which context is missing.

Verification: Reproduce each claimed defect; reject findings that rely on an impossible precondition.

Common failure: False positives waste time, while isolated snippets hide caller guarantees or real shared-state hazards.

### Workflow 6 Generate independent tests

Context to supply: External interface contract and allowed/invalid input classes before implementation details.

Prompt:

Design tests for this interface from its contract. Include boundaries, malformed inputs, repeated calls, allocation/I/O failure and state transitions. Define expected outcomes and explain which requirement each test checks. Keep a separate list of properties not covered by these tests. Do not derive expected results by calling the implementation under test.

Verification: Make one meaningful mutation or reintroduce the known bug and confirm the relevant test fails.

Common failure: Tests can mirror the same mistaken algorithm or merely check that a function returns.

### Workflow 7 Investigate build errors

Context to supply: Exact command, first meaningful error, tool versions, relevant configuration and recent diff.

Prompt:

Analyze this build failure. Separate the first causal diagnostic from downstream messages. Rank at most three hypotheses and give a small check to distinguish them. Explain whether the issue is source, API version, configuration, toolchain or link/runtime mismatch. Do not suppress warnings or add casts without proving the type contract.

Verification: Run the smallest discriminating check and rebuild from a clean affected target.

Common failure: Changing random flags can hide an ABI mismatch or run an old executable after a failed build.

### Workflow 8 Analyze logs and crashes

Context to supply: Original timestamped logs, matching build ID, expected behavior and actions before failure.

Prompt:

Create a timeline using only observed events. Label hypotheses separately. Identify the earliest divergence from expected behavior and what the evidence cannot establish. Propose the next two observations that would distinguish likely causes. Preserve raw error values and units. Do not invent missing log lines.

Verification: Correlate timestamps/clocks and reproduce with the same build; verify symbol files before trusting a backtrace.

Common failure: Merged clocks, truncated logs and mismatched symbols can produce a persuasive but false causal story.

### Workflow 9 Conduct a mock interview

Context to supply: Chosen track, level, covered modules and whether hints are allowed.

Prompt:

Interview me for [track and level]. Ask one question at a time, wait for my answer, then probe assumptions with a changed case. Score correctness, reasoning, edge cases and communication using the package rubric. Do not reveal the full answer before I attempt it. Record concrete gaps and one practical retest for each.

Verification: Repeat a different variant with no assistant or reference material and compare the rubric evidence.

Common failure: Memorizing the assistant answer can inflate apparent readiness without implementation ability.

### Workflow 10 Write truthful project documentation

Context to supply: Your actual changes, test logs, limitations and measured values.

Prompt:

Draft an architecture and demonstration note from the supplied evidence. Separate code authored, build verified, host execution, emulator execution and hardware execution. Do not invent speedups, reliability claims or personal contributions. Turn each test result into a bounded statement with its input and environment. List missing checks plainly.

Verification: Compare every claim and numeric value with an existing log or source artifact.

Common failure: Polished language can exaggerate an object compile into a working driver or a simulation into hardware proof.

### Independent learning checkpoint

After assistance, close the answer and explain the code from memory. Change one requirement and implement the change without copying. Run a fresh failure case. If you cannot do this, keep the learner status as implemented with assistance and plan a retest. AI can accelerate feedback; it cannot provide your evidence of independent competence.

### Handling code and credentials

Supply minimal reproducible examples. Remove secrets, credentials, customer identifiers and proprietary details unless your organization's approved environment and policy allow them. Do not paste private keys to ask whether signing works. Use disposable lab keys and share only the required public verification material. Keep an audit of accepted changes and their tests.

### Embedded inference extension

C02 contains a small optional anomaly pipeline after the deterministic robot core. Treat its synthetic dataset as a software-pipeline fixture. It does not establish detection quality on an AMR or industrial plant. Begin with a threshold or statistical baseline, split data by acquisition run/device/time, tune only on training/validation and evaluate the final model once on held-out data. Count false alarms and missed faults and report the sample population.

Measure model file bytes, working memory, latency distribution and CPU cost on the target that will run inference. A host latency is not an MCU latency. Keep sampling/control and hard limits deterministic, and define fallback for invalid input, model failure, excessive latency and distribution shift. Linux user-space inference or a supported MCU runtime are options; select based on actual operators and resource limits, not model marketing. [LITERT] [ONNX]

The extension passes only if the core acceptance tests still pass, the evaluation split is documented, the baseline comparison is reported honestly and failure never extends command freshness or actuator permission.

# 8 Interview question and answer bank

## Interview question bank

This bank contains 200 distinct questions for shared foundations, Embedded Linux, kernel and BSP development, and MCU firmware. Questions are original teaching and assessment material. Primary references are supplied for verification; no company-specific interview claims are made.

Use Questions_Only.md for a closed-book attempt before opening the answers. Each question has four one-point scoring criteria. A score of 3/4 is a useful retrieval-practice target, but a dangerous incorrect claim about bounds, concurrency, lifetime, update trust, or control authority fails that question regardless of fluent delivery. Complete the practical assessments separately; oral scores do not demonstrate implementation competence.

Assumptions are C11 for C questions, C++17 where stated, Linux/POSIX for system programming, and Linux 6.12 documentation for kernel concepts. Project VERSIONS.md controls actual build targets. Conceptual references to current project documentation do not silently upgrade a project's pinned toolchain. All source access dates are 27 October 2026.

Coding tasks Q021–Q040 and Q161–Q170 have executable reference routines in ../04_Skill_Assessments/answers/coding_reference.c; Q061 has a complete POSIX helper in ../04_Skill_Assessments/answers/posix_reference.c. Other Linux coding tasks include precise solution contracts and can be developed against the associated system-programming projects. Full project implementations remain in 07_Projects.

### C objects and behavior

#### Q001 How are an array and a pointer different when passed to a function?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Object representation, parameter adjustment, and bounded API design.

Spoken answer: An array owns a sequence of elements; a pointer holds an address. An array parameter is adjusted to a pointer, so pass its length separately.

Deeper explanation: For int a[8], sizeof a in its declaring scope measures the array. In void f(int a[8]), sizeof a measures a pointer. An interface such as f(int *a, size_t n) must still validate n against the caller's actual storage contract. A one-past pointer may be formed, but dereferencing it is invalid.

Assumptions: Assume ISO C11 and a real array, not a flexible array member.

Common incorrect answer: Claiming the callee recovers element count from the pointer or bracket annotation.

Follow-up: How would you express read-only access and document whether null is valid at length zero?

Scoring out of 4: 1 point each: object distinction; parameter adjustment; length contract; correct one-past rule.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q002 Explain storage duration, scope, and linkage using a static local variable.

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Separating three language concepts that affect firmware state.

Spoken answer: Scope controls where a name is visible, storage duration controls lifetime, and linkage connects declarations. A static local persists across calls with block scope.

Deeper explanation: In int next(void) { static unsigned n; return ++n; }, n has static storage duration and no linkage. It is initialized once, before normal execution, and retains state. This is not automatically reentrant or thread safe. A file-scope static object instead has internal linkage. Hidden persistent state complicates reset, unit testing, and multiple device instances.

Assumptions: Assume C11; integer conversion of the returned unsigned counter needs a separate contract.

Common incorrect answer: Saying static always means global visibility or a thread-safe singleton.

Follow-up: How would you refactor this into an explicit per-instance context with a reset operation?

Scoring out of 4: 1 point each: scope; duration; linkage; reentrancy and explicit-state consequence.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q003 What do const placement and volatile placement actually promise?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Type qualifiers and the boundary between language semantics and hardware access.

Spoken answer: const int *p prevents modification through p; int *const p fixes the pointer. volatile preserves required accesses to a volatile object under implementation rules.

Deeper explanation: A const view does not prove the underlying object never changes through another legal alias. Conversely, casting away const and modifying an originally const-defined object is invalid. volatile is useful for implementation-defined hardware access patterns, but it supplies neither mutual exclusion nor inter-thread ordering. For Linux MMIO, use the kernel's accessor APIs instead of substituting a volatile pointer.

Assumptions: Assume C11; device access width and ordering require target documentation.

Common incorrect answer: Treating volatile as atomic, or assuming const universally puts data in ROM.

Follow-up: Why might reading a status register twice be incorrect even when both reads are volatile?

Scoring out of 4: 1 point each: qualifier placement; alias caveat; volatile limits; hardware API distinction.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q004 Classify signed overflow and unsigned wraparound.

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Integer contracts and avoiding optimization-sensitive bugs.

Spoken answer: Unsigned arithmetic wraps modulo its type's range. Signed overflow is undefined behavior; a compiler may optimize on the assumption that it does not happen.

Deeper explanation: int x = INT_MAX; x + 1 has no portable numeric result. Check before the addition, for example positive b requires a <= INT_MAX-b; handle negative b symmetrically. Unsigned wrap can implement a deliberate counter, but it is a bug when it defeats a bounds or allocation check. Sanitizers help expose exercised violations and do not prove absence.

Assumptions: Assume standard integer arithmetic without compiler flags defining alternate overflow behavior.

Common incorrect answer: Reporting signed overflow always wraps because one observed processor does so.

Follow-up: How would you safely check count * element_size before allocating?

Scoring out of 4: 1 point each: both classifications; precondition check; optimization implication; allocation connection.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q005 Why can comparing a negative int with an unsigned length surprise you?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Usual arithmetic conversions and length validation.

Spoken answer: The operands may be converted to an unsigned type, making a negative value a large positive number. Validate signs before crossing signed and unsigned domains.

Deeper explanation: For typical equal-rank int and unsigned int, -1 < 1u is false after conversion. The exact conversion path depends on ranks and representable ranges, so avoid memorizing one result for every type combination. Store object sizes as size_t, keep negative error codes separate, and only convert a validated nonnegative count. Review mixed comparisons in parsers and syscall result handling.

Assumptions: Assume the stated type ranks; ssize_t is a POSIX signed result type.

Common incorrect answer: Casting every value to unsigned to silence warnings without validating its meaning.

Follow-up: How do you handle read() returning -1 before comparing its result with a buffer size?

Scoring out of 4: 1 point each: conversion mechanism; rank qualification; validated conversion; syscall example.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q006 Is printf("%d %d", i++, i++) guaranteed to print a particular pair?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Recognizing undefined behavior instead of guessing evaluation order.

Spoken answer: No. The two modifications of the same scalar are unsequenced in C11, so the expression has undefined behavior and no valid universal output.

Deeper explanation: Changing optimization level or compiler may change the observation without making either result authoritative. Rewrite as separate full expressions when an order matters: save the first value, increment, save the second, increment, then print the saved values. Distinguish this from unspecified argument order when independent arguments do not create a conflicting access. A code-output interview answer should name the violated rule first.

Assumptions: Assume ISO C11; do not transfer C++ sequencing rules without specifying its version.

Common incorrect answer: Calling the result merely compiler-dependent or selecting a likely left-to-right order.

Follow-up: Give an expression with unspecified evaluation order that does not have undefined behavior.

Scoring out of 4: 1 point each: UB classification; unsequenced conflict; valid rewrite; distinction from unspecified order.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q007 What makes returning a pointer to a local array unsafe?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Object lifetime and ownership at function boundaries.

Spoken answer: The automatic array's lifetime ends when the function returns. A returned pointer does not extend that lifetime, and dereferencing it is invalid.

Deeper explanation: Choose an explicit ownership model: the caller supplies a buffer and capacity; the function allocates and transfers ownership; or it returns a documented immutable static object. Static storage avoids the lifetime bug but may introduce shared-state races and overwrite earlier results. A caller-owned buffer often suits firmware because memory and concurrency are visible. Return a status and written length to distinguish truncation from success.

Assumptions: Assume ordinary automatic storage, not an object intentionally allocated on the heap.

Common incorrect answer: Believing the pointer is safe until another function happens to reuse the stack.

Follow-up: How would the API report required size when the supplied buffer is too small?

Scoring out of 4: 1 point each: lifetime; caller-buffer alternative; ownership transfer; static-state tradeoff.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q008 Why is memcpy not a safe replacement for memmove on overlapping regions?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Library preconditions and reasoning about aliasing.

Spoken answer: memcpy requires nonoverlapping source and destination regions; violating that precondition is undefined behavior. memmove supports overlap with an as-if temporary copy.

Deeper explanation: Consider shifting a byte buffer one position right: memmove(buf+1, buf, n) preserves the original bytes. A naive forward loop overwrites future input; a naive backward loop fails for the opposite overlap direction. Do not compare unrelated object pointers to invent portable overlap detection. Choose the correct contract at the call site and independently verify destination capacity.

Assumptions: Assume both source and destination designate valid regions of the requested length.

Common incorrect answer: Assuming a particular libc implementation happens to handle overlap, so the code is portable.

Follow-up: Does using memmove also solve a destination buffer overflow?

Scoring out of 4: 1 point each: overlap precondition; direction example; capacity independence; portable contract.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q009 Can a received byte buffer be cast directly to a C packet structure?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Alignment, object representation, endianness, and defensive decoding.

Spoken answer: Usually that is not a portable parser. Structure layout, alignment, padding, byte order, and available input length need explicit handling.

Deeper explanation: Decode fixed-width fields from bytes after checking the complete field is present. For a big-endian 16-bit field, combine ((uint16_t)p[0] << 8) | p[1]. A packed structure may remove padding for a particular compiler, but it can create unaligned accesses and does not resolve endian or lifetime assumptions. Validate lengths before constructing views and avoid trusting a packet's declared size.

Assumptions: Assume an octet-based wire format and available uint16_t.

Common incorrect answer: Using a packed attribute as a complete cross-platform serialization strategy.

Follow-up: How would you validate a length field without overflowing header_size + payload_size?

Scoring out of 4: 1 point each: layout; alignment; explicit decoding; checked bounds.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q010 Why is bit-field layout unsuitable for a portable wire protocol?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Distinguishing convenient source notation from an interchange representation.

Spoken answer: C bit-field allocation and ordering are implementation-defined in important respects. Use explicit masks and shifts over well-defined integer or byte representations for a wire format.

Deeper explanation: Define bit positions in the protocol specification, for example bit 0 means sensor-valid and bits 1–3 contain a mode. Validate the field value before shifting and preserve reserved bits according to the contract. A register description may also require write-one-to-clear behavior, making a read-modify-write unsafe. Test encoding against fixed vectors instead of comparing only an encoder to its own decoder.

Assumptions: Assume the protocol specifies byte order and bit numbering independently of compiler layout.

Common incorrect answer: Expecting a bit-field structure to have identical layout across compilers and endian targets.

Follow-up: What additional precautions apply when the destination is a hardware status register?

Scoring out of 4: 1 point each: implementation dependence; explicit masks; reserved-bit rule; register side effects.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q011 When is a shift expression undefined or implementation-defined?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Boundary conditions in bit operations.

Spoken answer: A negative shift count or a count at least the promoted left operand width is undefined. Signed shifts have additional restrictions; prefer unsigned operands for bit masks.

Deeper explanation: For 1u << n, validate n against the width of unsigned, not an assumed 32. In C11, right shifting a negative signed value is implementation-defined. Left shifting signed values is not a general multiplication substitute because representability restrictions apply. Integer promotions also mean a uint8_t operand may be promoted before shifting. Use width-specific constants and document the supported width.

Assumptions: Assume C11 and no implementation extension that changes these semantics.

Common incorrect answer: Claiming the processor masks the count, so shifting by the width is valid C.

Follow-up: How would you construct a full-width mask without evaluating 1u << width?

Scoring out of 4: 1 point each: count limits; unsigned choice; negative-right-shift qualification; promotions.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q012 Explain strict aliasing and a safe way to inspect a float's bytes.

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Access type rules and representation inspection.

Spoken answer: Accessing an object through an incompatible lvalue type can violate C's effective-type rules. Character-type access may inspect its representation; memcpy also avoids an incompatible pointer dereference.

Deeper explanation: Copy a float into an unsigned-character array with memcpy(bytes, &f, sizeof f) and inspect the bytes as representation, not as a promised IEEE-754 encoding. Copying into an equal-sized integer can be useful only with an explicit size and representation assumption. Pointer-casting a float to uint32_t * risks both aliasing and alignment problems. Compiler optimization can expose bugs hidden in debug builds.

Assumptions: Assume ISO C11; floating representation and byte order are target properties.

Common incorrect answer: Saying equal object sizes make all pointer casts legal.

Follow-up: How would you serialize a floating measurement for heterogeneous machines?

Scoring out of 4: 1 point each: incompatible access; character exception; memcpy pattern; representation caveat.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q013 What is the difference between a null pointer and an uninitialized pointer?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Initialization and error-resistant API contracts.

Spoken answer: A null pointer is a defined value that designates no object or function. An uninitialized automatic pointer has an indeterminate value and is not a valid substitute.

Deeper explanation: Initializing an owner to null makes cleanup paths simpler because free(NULL) is defined. It does not make every null dereference or every function call with null acceptable. Document whether an API accepts null with zero length. After freeing, clear the owner when useful, but other aliases remain dangling; clearing one variable does not resolve shared ownership or use-after-free elsewhere.

Assumptions: Assume a hosted C implementation; an all-zero byte pattern is not a universal pointer representation.

Common incorrect answer: Treating an uninitialized pointer as probably null or clearing one alias as a lifetime solution.

Follow-up: Why is memset(&p, 0, sizeof p) a weaker portable initialization than p = NULL?

Scoring out of 4: 1 point each: defined null; indeterminate pointer; API contract; dangling aliases.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q014 How should realloc failure be handled without losing the original allocation?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Ownership-preserving allocation and size contracts.

Spoken answer: Assign the result to a temporary pointer. On failure for a nonzero requested size, retain the original allocation and decide whether to continue or free it.

Deeper explanation: Use void *tmp = realloc(p, new_size); if (!tmp) { /* preserve p */ } else p = tmp;. Validate multiplication before calculating new_size; define zero-size behavior explicitly rather than relying on version-sensitive rules. A successful move invalidates the old pointer and interior aliases. Embedded software may instead use fixed-capacity buffers when bounded memory usage and deterministic failure behavior matter.

Assumptions: Assume nonzero size and a standard hosted allocator; ISR allocation is a separate policy question.

Common incorrect answer: Overwriting the sole owning pointer directly and then being unable to free on failure.

Follow-up: What happens to a pointer into the old allocation after a successful relocation?

Scoring out of 4: 1 point each: temporary owner; size validation; aliases; zero-size qualification.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q015 What can a function pointer safely represent in a driver-like C interface?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Typed callbacks, context ownership, and dispatch.

Spoken answer: A function pointer can select behavior through a compatible function type. Pair it with explicit context and a lifetime contract for callback registration and removal.

Deeper explanation: For example, struct sink { int (*write)(void *, const unsigned char *, size_t); void *ctx; }; makes state ownership visible. Define whether callbacks may block, whether they can be called concurrently, and who owns the input during the call. Calling through an incompatible function-pointer type is not repaired by a cast. Unregistering requires preventing new calls and waiting for in-flight calls before freeing context.

Assumptions: Assume C11; converting object pointers to function pointers is not generally portable ISO C.

Common incorrect answer: Casting any callback signature to the expected type and trusting matching register usage.

Follow-up: How would cancellation work when a callback may already be running?

Scoring out of 4: 1 point each: compatible type; explicit context; execution contract; unregister lifetime.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q016 How do alignment and structure padding affect an embedded ABI?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Memory layout versus source-field intuition.

Spoken answer: Compilers may insert padding to satisfy alignment, so a structure's size is not necessarily the sum of its member sizes. ABI rules determine the concrete layout.

Deeper explanation: Reordering private members can reduce padding, but changing a public structure may break binary compatibility. Use sizeof, _Alignof, offsetof, and compile-time assertions for an explicitly supported target. Do not compare whole structures with memcmp to test semantic equality because padding may differ. DMA buffers, shared memory, and wire data each have distinct layout and alignment contracts that should be specified separately.

Assumptions: Assume C11 and the selected compiler ABI; exact offsets must be measured or documented.

Common incorrect answer: Assuming packing is a cost-free way to save memory on every architecture.

Follow-up: Which problems can appear when a packed member is passed to a function expecting aligned storage?

Scoring out of 4: 1 point each: padding; ABI contract; semantic comparison; packed-access consequence.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q017 What makes a C API reentrant, and how is that different from thread safe?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: State isolation and nested-call reasoning.

Spoken answer: A reentrant function can be entered again before an earlier invocation finishes without corrupting state. Thread safety concerns concurrent calls under a specified threading model.

Deeper explanation: A function protected by a nonrecursive mutex may be thread safe yet unsafe to call recursively from a signal path or callback that interrupts its execution. Per-call local state and caller-owned contexts improve reentrancy, but callees and allocator behavior still matter. An interrupt handler has stricter allowed-operation rules than ordinary threads. State precisely which callers, contexts, and shared resources an API supports.

Assumptions: Assume the terms are evaluated against an explicit execution model rather than as unconditional labels.

Common incorrect answer: Treating a mutex as proof that an API is safe in a signal handler or ISR.

Follow-up: Can a function with only local variables still be non-reentrant because of a callee?

Scoring out of 4: 1 point each: nested entry; thread distinction; callee effects; context restrictions.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q018 Why does volatile not fix a shared-counter data race?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: The C memory model and atomic read-modify-write.

Spoken answer: Volatile does not make an increment atomic or establish synchronization between threads. Use a mutex or a suitable C atomic operation, depending on the required invariant.

Deeper explanation: counter++ includes a read and write that can overlap another increment. Even when machine word accesses are naturally atomic, unsynchronized conflicting C accesses can be undefined behavior. _Atomic unsigned with atomic_fetch_add makes the update atomic; relaxed ordering is enough only when the counter does not publish other state. A multi-field invariant usually needs a larger synchronization design.

Assumptions: Assume C11 threads and objects shared by threads; MMIO is a different problem.

Common incorrect answer: Arguing that a single-core CPU or optimization level makes the data race acceptable.

Follow-up: When would acquire-release ordering be needed instead of a relaxed counter increment?

Scoring out of 4: 1 point each: race; atomic RMW; relaxed limit; multi-field invariant.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q019 What does undefined behavior imply for a code-output interview question?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Disciplined reasoning under missing language guarantees.

Spoken answer: State that the standard gives no guaranteed behavior, identify the first violated precondition, and propose a valid rewrite. Do not manufacture one universal output.

Deeper explanation: Examples include using a freed object, out-of-bounds access, signed overflow, and conflicting unsequenced accesses. An experiment can show one implementation's observation but cannot define all valid implementations. Separate undefined behavior from unspecified choices and implementation-defined behavior that must be documented. Ask for the language version, data model, and compiler extensions only when those can legitimately resolve the question.

Assumptions: Assume ISO C semantics unless the interviewer explicitly requests a documented extension.

Common incorrect answer: Calling every surprising result undefined or every observed result implementation-defined.

Follow-up: How would you explain an optimization-sensitive crash to a colleague without blaming the compiler prematurely?

Scoring out of 4: 1 point each: precise classification; first violation; valid rewrite; experiment limitation.

Learning connection: Module M02. Verification references: C11, GCC.

#### Q020 How do you choose between fixed allocation, a pool, and general heap allocation?

Track: Shared | Difficulty: Foundation | Round: Technical screen | Tags: conceptual;code-analysis

Assesses: Resource budgeting and failure behavior in embedded software.

Spoken answer: Choose from measured lifetime patterns and timing requirements. Fixed storage is simple and bounded; pools bound object counts; a general allocator supports flexibility with more failure modes.

Deeper explanation: A telemetry queue can use a fixed ring when overload policy is explicit. A pool can serve equal-sized request objects without general external fragmentation. Heap use may be reasonable during initialization or noncritical work, but handle exhaustion and document ownership. Avoid claiming all heaps are nondeterministic or all static allocation is safe: static sizing can still overflow or silently drop important work.

Assumptions: Assume the target's allocator, memory budget, and scheduling constraints are known before final selection.

Common incorrect answer: Banning all dynamic allocation without explaining the actual lifetime or timing requirement.

Follow-up: How will the system behave when the pool is empty, and how will that be observed?

Scoring out of 4: 1 point each: lifetime analysis; bounded capacity; failure policy; measured tradeoff.

Learning connection: Module M02. Verification references: C11, GCC.

### C coding exercises

#### Q021 Implement checked multiplication for two size_t values before allocation.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Overflow prevention without evaluating an already-overflowed result.

Spoken answer: Reject multiplication when the second operand is nonzero and the first exceeds SIZE_MAX / second. Otherwise store the product through a valid output pointer.

Deeper explanation: The reference function checked_mul returns false on overflow and leaves the output unchanged. Cases include (0, SIZE_MAX), (SIZE_MAX, 1), and (SIZE_MAX, 2). Do not check the wrapped product afterward. Also distinguish representability from an application's smaller memory limit: a mathematically valid allocation can still fail or be unacceptable for the target.

Assumptions: Assume a nonnull output pointer; the API explicitly rejects null in the reference implementation.

Common incorrect answer: Testing only a*b < a, which misses some overflow combinations and obscures the contract.

Follow-up: How would you add a header size to the multiplication result safely?

Scoring out of 4: 1 point each: division precheck; zero handling; unchanged failure output; boundary tests.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q022 Write a bounds-checked big-endian 16-bit decoder from bytes.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Portable byte decoding and a clear error interface.

Spoken answer: Require at least two bytes, widen each byte before shifting, combine them, and return status separately from the decoded value.

Deeper explanation: decode_be16 accepts a byte pointer, length, and output pointer; invalid pointers or short input fail without modifying output. Vectors 00 00, 12 34, and ff ff must yield 0, 4660, and 65535. Casting the input to uint16_t * is unnecessary and can introduce alignment, endian, and aliasing assumptions. Larger parsers must retain a cursor and remaining-length invariant.

Assumptions: Assume 8-bit bytes and availability of uint16_t; the wire format is big-endian.

Common incorrect answer: Returning zero on error, making an invalid packet indistinguishable from a valid zero reading.

Follow-up: How would the interface advance a cursor only when decoding succeeds?

Scoring out of 4: 1 point each: length check; explicit endian conversion; status separation; unmodified failure state.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q023 Implement an eight-byte ring buffer with explicit full and empty states.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Capacity invariants, wraparound, and overload policy.

Spoken answer: Track head, tail, and count. Push only when count is below capacity; pop only when count is nonzero. Advance indices modulo the fixed capacity.

Deeper explanation: The reference ring_push and ring_pop reject full and empty operations without altering state. Test filling all eight slots, a ninth push, partial draining, wraparound writes, then FIFO order. Maintaining count allows all eight elements to be usable. This simple implementation is single-threaded; synchronization cannot be added by merely qualifying fields volatile. Define whether a future application drops, blocks, or replaces data when full.

Assumptions: Assume a caller-initialized zeroed ring and no concurrent access.

Common incorrect answer: Allowing head equal to tail to ambiguously mean both full and empty.

Follow-up: How would a single-producer single-consumer atomic implementation publish an element safely?

Scoring out of 4: 1 point each: invariant; wraparound; no mutation on failure; concurrency qualification.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q024 Convert an even-length ASCII hexadecimal string into a bounded byte buffer.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Validation, capacity checks, and transactional parsing.

Spoken answer: Reject odd length, invalid digits, and insufficient output capacity. Validate all characters before modifying output if the API promises failure leaves output unchanged.

Deeper explanation: hex_decode accepts upper- and lowercase hex without separators. "00aF" becomes two bytes; "0", "0G", and a two-byte result with capacity one fail. The reference uses explicit digit classification rather than a locale-sensitive permissive parser. Record output length only on success. Two-pass validation costs another linear scan but makes the failure contract simple for callers.

Assumptions: Assume a NUL-terminated input and output storage that does not overlap it.

Common incorrect answer: Using a conversion routine that silently stops at the first invalid character.

Follow-up: When would a streaming decoder with partial progress be a better contract?

Scoring out of 4: 1 point each: all validation; bounded output; transactional failure; case and malformed tests.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q025 Reverse a byte array in place without accessing beyond either end.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Index invariants and zero-length handling.

Spoken answer: Swap symmetric elements while the left index is below half the length. Compute the matching index as n-1-i only inside that loop.

Deeper explanation: reverse_bytes treats null with zero length as valid but rejects null with positive length. The half-length loop avoids computing n-1 for an empty array, so there is no unsigned underflow before a guard. Test lengths zero, one, two, and five, plus applying reverse twice. This is linear time and constant extra space; the contract does not promise safe concurrent mutation.

Assumptions: Assume ordinary writable byte storage for every index in the stated length.

Common incorrect answer: Starting an unsigned right index at n-1 before checking n == 0.

Follow-up: How would reversing UTF-8 bytes differ from reversing user-visible characters?

Scoring out of 4: 1 point each: empty input; valid indices; odd middle element; stated byte-level scope.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q026 Set or clear a selected bit in a uint32_t without undefined shifts.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Bit-width validation and preserving unrelated bits.

Spoken answer: Reject bit indices 32 or above. Build a UINT32_C(1) mask, then OR it to set or AND its complement to clear.

Deeper explanation: bit_assign receives an output pointer, index, and boolean value. Tests must preserve all other bits, exercise indices zero and 31, and reject 32. A zero initial value helps test set operations; all-ones tests clear operations. This function operates on an ordinary in-memory object. Applying an equivalent read-modify-write to a hardware register may be wrong when bits are write-one-to-clear or concurrently updated.

Assumptions: Assume uint32_t exists and the object is not concurrently modified.

Common incorrect answer: Using signed 1 << 31 or accepting an unchecked shift count.

Follow-up: How would an atomic shared flags word change the implementation?

Scoring out of 4: 1 point each: index bound; unsigned mask; preservation; MMIO and concurrency caveat.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q027 Implement CRC-16/CCITT-FALSE and verify a known check value.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Protocol interoperability and complete CRC parameterization.

Spoken answer: Use polynomial 0x1021, initial value 0xffff, no input/output reflection, and final XOR zero. The ASCII bytes of 123456789 produce 0x29b1.

Deeper explanation: crc16_ccitt processes each byte into the high end of the register and advances eight polynomial steps. CRC names alone can be ambiguous, so put all parameters and byte order in the protocol document. The function returns 0xffff for empty input. CRC detects many accidental transmission errors but supplies no authentication; an attacker can recompute it after modifying a message.

Assumptions: Assume eight-bit bytes; null is allowed only with zero input length.

Common incorrect answer: Using another CRC-16 variant and assuming all peers will agree because the width matches.

Follow-up: What wire byte order will you choose for the two CRC bytes, and why specify it separately?

Scoring out of 4: 1 point each: parameters; check vector; empty input; integrity versus authenticity.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q028 Parse a decimal uint32_t with no trailing characters or overflow.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Defensive conversion of external text.

Spoken answer: Require at least one decimal digit and reject signs, whitespace, and trailing characters under the stated grammar. Before each step, ensure the next multiply-add fits.

Deeper explanation: parse_u32 checks value > (UINT32_MAX-digit)/10 before updating. Test zero, 4294967295, 4294967296, empty input, 12x, and -1. The output stays unchanged on failure. This narrow grammar suits machine configuration; a command-line UI may choose a more permissive grammar, but should do so explicitly. Avoid conflating parse failure with numeric zero.

Assumptions: Assume NUL-terminated ASCII input and a nonnull output pointer.

Common incorrect answer: Using atoi, which cannot express the necessary error and range distinctions.

Follow-up: How would you support optional base prefixes without silently accepting octal unexpectedly?

Scoring out of 4: 1 point each: grammar; precheck; status contract; maximum and malformed cases.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q029 Write a bounded string copy that reports truncation explicitly.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: C string termination and useful caller-visible status.

Spoken answer: If capacity is positive, copy at most capacity minus one characters and terminate. Return success only when the complete source fits; report the needed length separately.

Deeper explanation: copy_text computes the source length and handles zero capacity without writing. Test an exact fit including the terminator, one-byte destination, empty source, and insufficient space. A terminated partial string is still truncated, so callers must not treat termination as complete success. Document that the source and destination cannot overlap. Avoid using a silent truncation helper for security-sensitive identifiers.

Assumptions: Assume a valid NUL-terminated source; source length excludes its terminator.

Common incorrect answer: Claiming strncpy always terminates or that a truncated pathname is an acceptable success.

Follow-up: When would rejecting the entire operation be preferable to writing a truncated prefix?

Scoring out of 4: 1 point each: terminator; zero capacity; explicit truncation; overlap and semantic contract.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q030 Implement a saturating addition of two uint32_t values.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Deliberate arithmetic policy instead of accidental wraparound.

Spoken answer: Return UINT32_MAX when a > UINT32_MAX-b; otherwise return a+b. Saturation is an explicit application policy, distinct from modular arithmetic.

Deeper explanation: saturating_add is useful for cumulative diagnostics where wraparound would hide repeated errors. Test zero, values adding exactly to the maximum, and an overflowing pair. The operation sacrifices information once saturated, so expose a saturation indicator if the application needs to distinguish a true maximum from overflow. Do not transfer this unsigned check to signed arithmetic without considering both overflow directions.

Assumptions: Assume a 32-bit unsigned result and no requirement to count beyond saturation.

Common incorrect answer: Computing the sum first and casting it to a larger type afterward.

Follow-up: How would you extend the interface to return a separate overflow flag?

Scoring out of 4: 1 point each: precondition; exact boundary; policy explanation; loss-of-information consequence.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q031 Implement a debounced boolean input state machine using sample counts.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: State transitions, noisy input handling, and bounded state.

Spoken answer: Keep the stable value, a candidate value, and consecutive candidate count. Commit a change only after the candidate persists for the configured number of samples.

Deeper explanation: debounce_step resets the candidate count when a sample disagrees with the pending candidate. Use threshold three and the trace 0,1,0,1,1,1 from stable zero: only the last sample changes the stable output. Saturate the count rather than allowing it to wrap. Sample-count debounce depends on sample cadence; missed samples alter wall-clock behavior and must be considered when specifying latency.

Assumptions: Assume periodic sampling and a threshold of at least one.

Common incorrect answer: Delaying in an ISR for the whole debounce interval or counting nonconsecutive samples as stable.

Follow-up: How would the design change to debounce by monotonic timestamps instead of sample count?

Scoring out of 4: 1 point each: candidate state; consecutive rule; bounded counter; timing interpretation.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q032 Implement an integer moving average over the most recent four samples.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Window maintenance, startup behavior, and accumulator range.

Spoken answer: Keep a four-element ring, running sum, current count, and next replacement index. Subtract the outgoing sample only after the window fills, then add the new sample.

Deeper explanation: average_push uses a 64-bit sum for 32-bit samples and divides by the current count during startup. For inputs 4, 8, 12, 16, 20, outputs are 4, 6, 8, 10, 14 with integer truncation. A running sum gives constant work per sample. Define rounding, signedness, and missing-sample behavior rather than assuming every averaging application shares one interpretation.

Assumptions: Assume unsigned 32-bit samples and a four-sample count-based window.

Common incorrect answer: Dividing by four before four samples exist without stating that zero-padding is intended.

Follow-up: How would you detect the maximum required accumulator width for a configurable window?

Scoring out of 4: 1 point each: replace/subtract rule; startup count; wide accumulator; known sequence.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q033 Insert a uint32_t into a sorted fixed-capacity array.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Bounds, shifting, and a precise duplicate policy.

Spoken answer: Reject a full array, find the insertion position, shift larger elements right, insert the value, and increment count only after success.

Deeper explanation: sorted_insert places a duplicate after existing equal values. Test insertion at the beginning, middle, and end; duplicates; empty storage; and full capacity with unchanged contents. Moving elements backward prevents overwriting values that still need to move. The operation is linear time, which is reasonable for small bounded embedded collections but different from a logarithmic lookup requirement.

Assumptions: Assume the first count elements are already sorted and count does not exceed capacity.

Common incorrect answer: Shifting forward into the destination or updating count before validating capacity.

Follow-up: When would a heap or bitmap be a better data structure than a sorted array?

Scoring out of 4: 1 point each: capacity; backward shift; duplicate contract; boundary and complexity analysis.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q034 Compare a wrapping 32-bit tick counter against a deadline safely.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Modular-time assumptions and half-range ambiguity.

Spoken answer: Use unsigned subtraction and a documented half-range rule. A deadline is reached when (uint32_t)(now-deadline) < 0x80000000u, provided relevant distances stay below half the range.

Deeper explanation: tick_due is portable unsigned arithmetic and avoids relying on out-of-range unsigned-to-signed conversion. Test a deadline of 0xfffffff0 against times just before it, equal to it, and 0x00000010 after wrap. Exactly half a cycle is ambiguous and excluded by the contract. Expired deadlines must be consumed or refreshed before a full ambiguity window passes.

Assumptions: Assume no scheduled interval or observation gap reaches 2^31 ticks.

Common incorrect answer: Comparing now >= deadline or claiming wrap-safe arithmetic works for arbitrarily long intervals.

Follow-up: How would you handle a timer that may be suspended longer than half a counter cycle?

Scoring out of 4: 1 point each: unsigned difference; half-range condition; wrap example; observation-gap limit.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q035 Detect a cycle in a singly linked list using constant extra space.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Pointer traversal invariants and algorithmic reasoning.

Spoken answer: Advance a slow pointer one link and a fast pointer two links, checking fast and its next pointer before advancing. A meeting indicates a cycle.

Deeper explanation: list_has_cycle covers an empty list, a one-node noncycle, a self-loop, and a cycle starting after a prefix. The method is linear in the reachable nodes before detection and constant in additional storage. It presumes every traversed link points to a live node or null; it cannot safely validate arbitrary corrupted addresses. Concurrency would require a separate lifetime strategy.

Assumptions: Assume an immutable list during the check and valid node storage.

Common incorrect answer: Dereferencing fast->next->next before verifying the necessary pointers.

Follow-up: How would you find the entry point of the cycle after detecting a meeting?

Scoring out of 4: 1 point each: two speeds; null guards; cycle cases; valid-memory assumption.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q036 Count the set bits in a uint32_t without a 32-iteration loop.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Bit identities and complexity that depends on input.

Spoken answer: Repeatedly clear the lowest set bit with x &= x-1 and increment a counter until x becomes zero. Each iteration removes exactly one set bit.

Deeper explanation: popcount32 returns zero for zero and 32 for all ones. Explain why subtraction flips the lowest one and lower zero bits, making the AND clear only that one. The algorithm executes as many iterations as set bits; it is not constant-time with respect to the value. A compiler intrinsic may map to an efficient instruction but requires a documented portability fallback.

Assumptions: Assume unsigned arithmetic; timing-sensitive cryptographic code needs separate analysis.

Common incorrect answer: Claiming the algorithm always executes exactly 32 steps or using signed overflow-prone intermediates.

Follow-up: When would a table, intrinsic, or constant-step method be preferable?

Scoring out of 4: 1 point each: identity; zero/all-ones cases; input-dependent work; portability/timing tradeoff.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q037 Round a size_t up to a power-of-two alignment without overflow.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Alignment preconditions and checked arithmetic.

Spoken answer: Reject zero or non-power-of-two alignment. Check size <= SIZE_MAX-(alignment-1) before computing (size+alignment-1) & ~(alignment-1).

Deeper explanation: round_up keeps the output unchanged on failure. Test already aligned sizes, zero, one below a boundary, alignment one, invalid alignment three, and overflow near SIZE_MAX. The result describes a numeric size; it does not itself allocate memory with that alignment. Also respect a target allocator's supported alignment values and minimum requirements.

Assumptions: Assume alignment fits size_t and only power-of-two alignment is supported by this API.

Common incorrect answer: Using the mask formula for arbitrary alignment or ignoring the addition overflow.

Follow-up: How would you round up to a non-power-of-two block size safely?

Scoring out of 4: 1 point each: power-of-two check; overflow check; exact mask; allocation distinction.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q038 Validate a sequence of one-byte-type one-byte-length TLV records.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Length-driven parsing and progress guarantees.

Spoken answer: Require a complete two-byte header, then ensure the declared value length fits the remaining input. Advance by header plus value and require exact exhaustion.

Deeper explanation: validate_tlvs accepts unknown types structurally and counts records; semantic policy is separate. Test empty input, a zero-length value, multiple records, a truncated header, and an oversized final value. Comparing length with remaining bytes avoids overflow-prone absolute-end calculations. Every successful iteration consumes at least the two-byte header, so zero-length values cannot cause an infinite loop.

Assumptions: Assume the outer buffer length is trusted and values contain arbitrary bytes.

Common incorrect answer: Advancing by value length alone or accepting a trailing orphan byte as harmless padding.

Follow-up: How would you cap nesting depth if a type contains another TLV sequence?

Scoring out of 4: 1 point each: header bound; value bound; forward progress; semantic/structural separation.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q039 Implement hysteresis for a high-temperature alarm.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Stateful threshold behavior and boundary precision.

Spoken answer: Turn the alarm on at or above the high threshold and off at or below the low threshold. Between thresholds, preserve the prior alarm state.

Deeper explanation: alarm_step rejects invalid threshold ordering and returns the next state through an output pointer. With low 60 and high 70, the trace 65, 71, 68, 60 yields off, on, on, off from an initially off state. Hysteresis limits chatter but does not filter measurement noise in every circumstance; sampling, debounce, and sensor validity may still need separate policies.

Assumptions: Assume integer temperatures in one documented unit and low strictly below high.

Common incorrect answer: Recomputing alarm as temperature >= high on every sample and losing the hold state.

Follow-up: What should happen when the sensor reading is invalid or stale?

Scoring out of 4: 1 point each: two transitions; hold band; boundary tests; invalid/stale-data policy.

Learning connection: Module M03. Verification references: C11, GCC.

#### Q040 Replace a bounded bit field in a uint32_t while preserving other bits.

Track: Shared | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Bit-range validation, mask construction, and overflow-free shifts.

Spoken answer: Validate width and offset, ensure the field fits within 32 bits, and reject a value too large for the field. Clear the field mask then insert the shifted value.

Deeper explanation: field_replace handles a full-width field separately so it never computes 1u << 32. Test a four-bit field at offset eight, a one-bit field at bit 31, a full-width replacement, and out-of-range values. Width zero is rejected by this contract. This routine updates ordinary memory; hardware registers may require different write semantics or atomic access.

Assumptions: Assume nonconcurrent uint32_t storage and explicitly numbered least-significant-bit offsets.

Common incorrect answer: Allowing the value to spill into adjacent bits or constructing a full-width mask with an invalid shift.

Follow-up: How would you adapt the interface to extract the same field and verify encode-decode vectors?

Scoring out of 4: 1 point each: range; value bound; full-width case; unrelated-bit preservation.

Learning connection: Module M03. Verification references: C11, GCC.

### Tools linking and C++

#### Q041 Explain preprocessing, compilation, assembly, and linking using one missing-symbol error.

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Locating a build failure at the correct stage.

Spoken answer: Preprocessing expands includes and macros, compilation produces assembly, assembly produces objects, and linking resolves references and creates an executable or library.

Deeper explanation: An undefined reference at link time can mean the implementation was not compiled, the object or library was omitted, the archive order is wrong, or symbol naming differs. Inspect commands with verbose builds, then use nm and readelf on the actual objects. A header declaration alone does not provide an implementation. Avoid changing source randomly when the failing stage is the link command.

Assumptions: Assume GCC-style ELF tooling; exact options and link-order behavior depend on the linker.

Common incorrect answer: Adding more include directories to fix an omitted function definition at link time.

Follow-up: How would C++ name mangling affect linking a C library?

Scoring out of 4: 1 point each: stages; plausible symbol causes; object inspection; declaration/definition distinction.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q042 What information can readelf and a linker map reveal that source code cannot?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Binary-level verification of a firmware build.

Spoken answer: They reveal section placement, symbols, relocations, entry points, and loadable segments. A map connects memory usage to input objects and linker decisions.

Deeper explanation: For a microcontroller image, check vector placement, flash and RAM regions, data load versus run addresses, and stack/heap reservations. For an ELF Linux binary, inspect interpreter path and required architecture. Source may look correct while the linker places a buffer in the wrong memory region. Compare actual section sizes against the linker script's limits and preserve the map with the build evidence.

Assumptions: Assume the selected ELF toolchain; some bare-metal artifacts are later converted to binary or Intel HEX.

Common incorrect answer: Equating the executable file size with runtime RAM usage.

Follow-up: Why might .bss consume RAM while contributing little payload to the image file?

Scoring out of 4: 1 point each: sections/segments; symbol ownership; memory budgeting; load/run distinction.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q043 What is the difference between an ABI and an API?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Compatibility across compilation and runtime boundaries.

Spoken answer: An API defines source-level interfaces and behavior. An ABI defines binary conventions such as calling convention, data layout, symbol naming, and object format.

Deeper explanation: Two builds may expose the same C function declaration but disagree on floating-point calling convention, structure packing, or architecture. That can cause runtime corruption even when names link. Pin compiler target options and compatible libraries, and rebuild dependent components when ABI changes. Linux user-space interfaces and internal kernel interfaces have different stability expectations; out-of-tree modules must match their target build.

Assumptions: Assume the same source language does not imply the same target ABI.

Common incorrect answer: Saying recompilation is never necessary because the header text is unchanged.

Follow-up: Which ABI evidence would you collect when a cross-compiled program crashes immediately?

Scoring out of 4: 1 point each: API/ABI distinction; concrete conventions; compatibility checks; kernel qualification.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q044 How does RAII improve resource handling in C++ system code?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Lifetime-bound cleanup and ownership transfer.

Spoken answer: Resource Acquisition Is Initialization ties resource ownership to an object's lifetime so destructors release resources on every normal scope-exit path, including exceptions.

Deeper explanation: Wrap a file descriptor in a move-only type that closes only when it owns a valid descriptor. Delete copying, transfer ownership during move, and make destruction nonthrowing. RAII applies to locks, mappings, and transactions, not just heap memory. It does not by itself prevent deadlocks, ambiguous shared ownership, or failure during a close-like operation that needs explicit reporting.

Assumptions: Assume C++17 or later for examples; exception policy depends on the embedded project.

Common incorrect answer: Treating a raw pointer member as ownership without implementing safe copy/move behavior.

Follow-up: How would you expose an explicit finalize operation for a resource whose release can fail?

Scoring out of 4: 1 point each: lifetime cleanup; move-only ownership; nonthrowing destructor; explicit error tradeoff.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q045 When would you use unique_ptr, shared_ptr, or a nonowning pointer?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Ownership communication and avoiding unnecessary shared lifetime.

Spoken answer: Use unique ownership by default when one component controls lifetime. Use shared ownership only for genuinely shared lifetime; use nonowning references or pointers with a clear validity contract.

Deeper explanation: shared_ptr reference counting has overhead and does not make the pointed-to object thread safe. Cycles require redesign or weak references. In a driver-like application, a manager can own a device object while short-lived operations borrow it under an explicit shutdown protocol. Ownership type and concurrency control solve different problems. Firmware may impose allocation policies that constrain standard-library use.

Assumptions: Assume C++17 and a toolchain/runtime supporting the selected library features.

Common incorrect answer: Choosing shared_ptr everywhere to avoid designing ownership or assuming it prevents data races.

Follow-up: How do you prevent a callback from using a borrowed object after shutdown?

Scoring out of 4: 1 point each: ownership choices; shared cost/cycles; data race distinction; borrower lifetime.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q046 How would you make a Make or CMake build reproducible enough to investigate a regression?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Controlled inputs and traceable build configuration.

Spoken answer: Record source revision, toolchain, options, dependency versions, configuration, and build environment. Rebuild from a clean or well-defined incremental state and compare artifacts appropriately.

Deeper explanation: Avoid embedding uncontrolled timestamps and host-specific absolute paths when bit-identical output matters. A successful rebuild is weaker than a reproducible binary. Keep generated files deterministic and include their generators. When a regression appears, compare the actual compile/link commands before assuming the source change alone caused it. A container helps capture environment but does not pin external downloads by itself.

Assumptions: Assume reproducibility goals are stated as repeatable behavior or byte-identical artifacts.

Common incorrect answer: Claiming a container tag or a committed Makefile proves every dependency is fixed.

Follow-up: How would you identify why two nominally identical ELF files differ?

Scoring out of 4: 1 point each: input manifest; environment control; goal distinction; artifact comparison.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q047 Why can a static library link fail depending on command-line order?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Archive extraction and dependency direction.

Spoken answer: Many traditional linkers scan archives to satisfy unresolved symbols known at that point, so objects that require symbols commonly precede their providing archives.

Deeper explanation: For an object using a utility library that itself uses a math library, an order such as app.o -lutil -lm may be required. Circular archive dependencies may need grouping or architectural cleanup. Shared-library behavior and linker options can alter details, so inspect the selected linker rather than applying a universal slogan. The useful debugging question is which unresolved symbol existed when an archive was scanned.

Assumptions: Assume a conventional GNU-style archive-linking model; confirm the active linker and flags.

Common incorrect answer: Rearranging include directives or adding the same library repeatedly without understanding the dependency graph.

Follow-up: What do archive groups change, and why can excessive circular dependencies signal a design problem?

Scoring out of 4: 1 point each: unresolved-symbol scan; example order; linker qualification; dependency reasoning.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q048 What should a useful embedded code review examine beyond formatting?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Risk-focused review of interfaces and failure paths.

Spoken answer: Review ownership, bounds, execution context, concurrency, timeout behavior, error recovery, tests, and observability against requirements. Style is useful but does not establish correctness.

Deeper explanation: Trace a resource from acquisition through normal completion, error unwinding, cancellation, and shutdown. For a parser, inspect invalid inputs and arithmetic before accepting a happy-path demo. For an ISR, inspect bounded work and the allowed API context. Ask for a test that would fail before the fix. Separate blocking correctness issues from optional preferences so reviews remain actionable.

Assumptions: Assume the reviewer has the relevant requirements and pinned target API documentation.

Common incorrect answer: Approving code because it compiles or because an AI review found no issues.

Follow-up: What evidence would justify accepting a known limitation in a learning demonstrator?

Scoring out of 4: 1 point each: risk categories; lifecycle trace; regression test; actionable prioritization.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q049 How do you use git bisect without misdiagnosing an intermittent fault?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Controlled experimentation over source history.

Spoken answer: Define a reliable good/bad test, identify known boundary revisions, and bisect while controlling environment. Mark untestable commits explicitly rather than labeling them good.

Deeper explanation: An intermittent failure needs repeated trials or a statistically justified classification rule; one clean run is insufficient. Preserve failing seeds, configuration, and logs. A commit may expose an older latent bug, so the first bad revision is evidence about behavior, not automatically proof of root cause. Build failures can require a skip or a separately documented compatibility fix.

Assumptions: Assume the regression is reproducible enough to classify revisions under the chosen test.

Common incorrect answer: Treating the first revision found as proof that every line in its patch is wrong.

Follow-up: How would you handle a bisect step that cannot build with the current host toolchain?

Scoring out of 4: 1 point each: classifier; environment; intermittent handling; exposure versus root cause.

Learning connection: Module M04. Verification references: GCC, LLVM.

#### Q050 What do compiler warnings, static analysis, and sanitizers each contribute?

Track: Shared | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Understanding complementary verification tools and their limits.

Spoken answer: Warnings catch many suspicious constructs, static analysis reasons about paths without normal execution, and sanitizers instrument executed paths to detect selected runtime violations.

Deeper explanation: Use strict warnings for code you own, inspect static findings rather than suppressing them wholesale, and run meaningful tests under supported sanitizers. AddressSanitizer and ThreadSanitizer are normally separate builds; target/runtime support matters. A clean run does not prove all paths or all races are safe. Record exact compiler, flags, test inputs, and exclusions so the evidence is reproducible.

Assumptions: Assume tool support is confirmed for the selected host and target; do not infer MCU support from host success.

Common incorrect answer: Claiming a sanitizer proves a program memory-safe or treating every analyzer warning as a confirmed bug.

Follow-up: How would you design input generation to exercise a parser beyond its normal examples?

Scoring out of 4: 1 point each: tool roles; separate builds/support; coverage limitation; recorded evidence.

Learning connection: Module M04. Verification references: GCC, LLVM.

### Linux process and I O fundamentals

#### Q051 How do a process, a thread, and a file descriptor differ?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Basic Linux resource and execution models.

Spoken answer: A process provides a resource context and address space; threads execute within it and share many resources. A file descriptor is a per-process integer handle to an open file description.

Deeper explanation: Duplicated descriptors can refer to the same open file description and therefore share offset or status flags. Threads generally share the descriptor table, so one thread closing a descriptor can affect another and lead to descriptor reuse races. A process boundary can isolate memory faults but introduces IPC and lifecycle complexity. Do not equate a descriptor number with permanent object identity.

Assumptions: Assume conventional Linux processes and POSIX threads; clone flags can construct other sharing arrangements.

Common incorrect answer: Thinking every thread has an independent descriptor table or that descriptor 5 always names one device.

Follow-up: How can close followed by open in another thread create a subtle reuse bug?

Scoring out of 4: 1 point each: process; thread sharing; open description; descriptor reuse consequence.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q052 Why must a read or write loop handle partial progress?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Correct byte-stream I/O and error semantics.

Spoken answer: A successful read or write may transfer fewer bytes than requested. Track progress, handle interruptions and nonblocking conditions, and treat EOF according to the operation's contract.

Deeper explanation: For writes, advance the pointer only by the positive byte count returned. Retry EINTR according to documented semantics, wait for readiness on EAGAIN when appropriate, and do not spin forever on no progress. Reads may split one protocol frame across calls or combine multiple frames. A negative return must be checked before converting to an unsigned length.

Assumptions: Assume POSIX byte-stream descriptors; datagram semantics and special devices need separate treatment.

Common incorrect answer: Assuming one write corresponds to one read or discarding the unwritten suffix.

Follow-up: What timeout should cover the whole operation instead of restarting after every short transfer?

Scoring out of 4: 1 point each: partial progress; errors; stream framing; absolute deadline.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q053 What is the difference between blocking mode and readiness notification?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Avoiding event-loop stalls and race assumptions.

Spoken answer: Blocking mode controls what an I/O operation does when it cannot progress. Readiness notification reports that an operation may progress; it does not reserve data for your thread.

Deeper explanation: Another consumer can drain a descriptor after poll reports it ready. Use nonblocking descriptors in multiplexed loops and handle EAGAIN even after readiness. A readable stream can signal EOF, not just payload. Error and hangup flags may coexist with readable buffered data, so drain according to the device contract before final closure. Readiness is not equivalent to a complete application message.

Assumptions: Assume POSIX poll-style readiness on Linux and potentially concurrent consumers.

Common incorrect answer: Calling a blocking read for a full expected message after one readiness event.

Follow-up: How should the loop handle a socket that is readable because its peer closed cleanly?

Scoring out of 4: 1 point each: mode versus event; race; EOF; message-boundary distinction.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q054 When should you choose poll instead of epoll, or epoll instead of poll?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Scale-sensitive design without premature complexity.

Spoken answer: Poll is straightforward for a small descriptor set. Epoll maintains an interest set and scales differently for many mostly idle descriptors, but introduces registration and event-lifecycle details.

Deeper explanation: Choose from measured workload, portability, and complexity. Level-triggered epoll is easier to reason about initially; edge-triggered mode normally requires nonblocking I/O and draining until EAGAIN. A large number of events still requires work regardless of the API. Handle descriptor closure and object lifetime carefully, because an event can outlive application metadata if teardown is poorly designed.

Assumptions: Assume Linux for epoll; regular files and device implementations have readiness-specific limitations.

Common incorrect answer: Saying epoll is always faster or edge-triggered mode automatically provides one event per message.

Follow-up: What would you measure to justify migrating a 12-descriptor daemon from poll?

Scoring out of 4: 1 point each: tradeoff; workload; edge-trigger drain; lifecycle risk.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q055 How should a daemon shut down cleanly after SIGTERM?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Signal safety and coordinated resource lifetime.

Spoken answer: Convert the signal into normal control flow, stop accepting work, notify blocked workers, drain or discard by policy, join them, then release shared resources.

Deeper explanation: A minimal signal handler can set a volatile sig_atomic_t flag or write to a suitable self-pipe using permitted operations. Linux signalfd with a correctly managed signal mask is another design. Avoid printf, mutex locking, or general allocator calls in a handler. A shutdown deadline prevents a stuck worker from hanging service termination forever, but forced termination must acknowledge possible data loss.

Assumptions: Assume documented POSIX async-signal-safety rules; the flag alone may not wake a blocked poll.

Common incorrect answer: Calling complex cleanup directly inside the signal handler or freeing state before workers exit.

Follow-up: How would you guarantee that a worker blocked on a condition variable notices shutdown?

Scoring out of 4: 1 point each: safe notification; blocked-worker wake; join-before-free; deadline policy.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q056 What are /proc, /sys, and /dev used for?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Kernel interfaces and choosing a stable observation method.

Spoken answer: Procfs exposes process and other kernel information, sysfs exposes device-model attributes, and device nodes under /dev provide entry points to device operations.

Deeper explanation: Treat each file as an interface with its own documented format rather than a normal disk file. A process can exit while its /proc/PID data is read, and files can change between reads. A sysfs attribute generally has a narrow representation; a character device can support a stream or control ABI. Prefer an established interface and document which fields your tool consumes.

Assumptions: Assume Linux and mounted virtual filesystems; permissions and namespaces affect visible data.

Common incorrect answer: Treating every virtual file as seekable, static, or safe to parse with fixed column assumptions.

Follow-up: Why is parsing /proc/PID/stat by simply splitting on spaces unsafe for the command field?

Scoring out of 4: 1 point each: three roles; dynamic data; per-interface contract; robust parsing example.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q057 How do fork and exec interact with descriptors and memory?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Process creation and deliberate inheritance.

Spoken answer: Fork creates a child with logically copied process state, commonly using copy-on-write memory. Exec replaces the process image while preserving selected state, including descriptors not marked close-on-exec.

Deeper explanation: Set close-on-exec atomically during descriptor creation where supported to avoid races in multithreaded programs. After fork in a multithreaded process, only the calling thread remains in the child, while copied locks may reflect vanished owners; follow restricted operations until exec. Use waitpid or another deliberate reaping strategy so child exits do not accumulate zombies.

Assumptions: Assume POSIX/Linux semantics; posix_spawn may simplify some launch cases.

Common incorrect answer: Believing exec creates a second process or assuming all descriptors close automatically.

Follow-up: Why can logging with a library mutex in the post-fork child deadlock?

Scoring out of 4: 1 point each: fork/exec distinction; inheritance; multithreaded caveat; reaping.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q058 What does fsync add beyond a successful write?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Durability and failure-aware persistence.

Spoken answer: A successful write usually means data was accepted by the kernel, not necessarily made durable. Fsync requests persistence for the file under the filesystem and device's guarantees.

Deeper explanation: For replacement of a configuration file, write a temporary file in the same filesystem, check writes, fsync the file, rename it into place, then fsync the containing directory when durability of the name change matters. Atomic rename and power-loss durability are different properties. Validate behavior on the actual filesystem and storage stack; hardware caches and failures can affect the guarantee.

Assumptions: Assume a filesystem supporting the operations; exact crash guarantees are target-specific.

Common incorrect answer: Calling rename alone a universal power-loss-safe transaction or ignoring writeback errors.

Follow-up: How would you test interrupted configuration updates without damaging real user data?

Scoring out of 4: 1 point each: acceptance versus durability; file sync; rename/directory; target validation.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q059 Why should timeouts normally use a monotonic clock?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Deadline correctness under wall-clock adjustments.

Spoken answer: A monotonic clock is intended for measuring elapsed time without civil-time jumps. Use a single deadline and recompute remaining time after interruptions or partial progress.

Deeper explanation: A wall clock may step when synchronized or manually adjusted, causing a timeout to expire early or late. Clock choice also determines whether suspend time is counted; Linux CLOCK_BOOTTIME and CLOCK_MONOTONIC differ in that respect. Convert units carefully and clamp expired deadlines to zero. Repeated relative sleeps accumulate work and scheduling delays, so periodic tasks benefit from absolute deadlines.

Assumptions: Assume the selected API supports the chosen clock; verify condition-variable clock configuration separately.

Common incorrect answer: Restarting a full timeout after every EINTR or assuming all monotonic-like clocks include suspend.

Follow-up: How would you define whether a command timeout should elapse while a laptop is suspended?

Scoring out of 4: 1 point each: elapsed-time clock; absolute deadline; suspend semantics; conversion/expiry.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

#### Q060 What does least privilege mean for an embedded Linux service?

Track: Embedded Linux | Difficulty: Practical | Round: Technical screen | Tags: conceptual

Assesses: Reducing the consequences of parser or network failures.

Spoken answer: Run with only the identity, device access, filesystem access, and capabilities required for the service's job. Separate privileged setup from ongoing processing when practical.

Deeper explanation: A serial telemetry service may need a specific device group and writable state directory, not unrestricted root. Service-manager restrictions and namespaces can reduce exposure, but must be tested against actual I/O and recovery behavior. Opening access by chmod 777 or globally disabling protections hides the permission model. Log actionable denial errors without exposing credentials or secret payloads.

Assumptions: Assume a defined deployment environment; capabilities and sandbox settings are Linux-specific and version-sensitive.

Common incorrect answer: Using root as a permanent fix for any device-permission failure.

Follow-up: How would you verify that an update helper has narrowly scoped authority?

Scoring out of 4: 1 point each: required privileges; concrete reduction; tested restrictions; operational diagnostics.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, EPOLL, SIGNAL, PROC, FSYNC, CLOCK.

### Linux coding exercises

#### Q061 Write a complete-write helper for a blocking descriptor with EINTR handling.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Progress tracking and syscall error handling.

Spoken answer: Loop until all bytes are written, advancing by each positive result. Retry EINTR, return a clear error for other failures, and reject a zero-progress write to prevent an infinite loop.

Deeper explanation: The reference write_all_blocking returns zero on success and -1 with errno on failure. Its contract excludes nonblocking descriptors and leaves timeout/cancellation to a higher layer. Test through a pipe with a reading peer; a large payload may produce partial progress. Preserve the original buffer and requested length for diagnostics rather than overwriting them with a syscall result.

Assumptions: Assume length is representable for each write call and the caller handles SIGPIPE policy.

Common incorrect answer: Retrying from the original buffer after a short write and duplicating the prefix.

Follow-up: How would you add cancellation and an absolute deadline for a nonblocking socket?

Scoring out of 4: 1 point each: suffix progress; EINTR; zero-progress/error; explicit blocking contract.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q062 Read a text file whose length is unknown without trusting stat size.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Dynamic input handling and growth limits.

Spoken answer: Read chunks until EOF, grow a buffer with overflow checks, and impose a maximum accepted size. Reserve space for a terminator only if the result is intended as text.

Deeper explanation: Procfs files may report an unhelpful size, so their contents must be read rather than allocated solely from stat. Handle EINTR and preserve the original allocation if growth fails. The exercise contract caps input at 64 KiB and rejects embedded NUL when parsing text. Tests should include empty input, exact cap, over-cap, and short reads through a pipe.

Assumptions: Assume a blocking descriptor and a caller-selected maximum; binary input requires a different parsing policy.

Common incorrect answer: Using one read sized from stat and assuming the returned data is complete and NUL-terminated.

Follow-up: How would you distinguish allocation failure from exceeding the configured maximum?

Scoring out of 4: 1 point each: EOF loop; bounded growth; realloc safety; dynamic-file cases.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q063 Build a poll loop that reads a pipe and exits when a control pipe signals shutdown.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Multiplexing, readiness, and shutdown ownership.

Spoken answer: Register both descriptors, process all returned flags, read available bytes, and leave the loop when the control channel requests shutdown or closes by policy.

Deeper explanation: Use nonblocking descriptors to avoid races after readiness. Handle EINTR by retrying with the remaining deadline, and handle data-descriptor hangup after draining buffered bytes. Keep shutdown notification separate from payload framing. Tests write payload in fragments, then send one control byte; another test closes the payload writer. Ensure descriptors are closed exactly once by their owner after the loop exits.

Assumptions: Assume a single loop owns these descriptors and the control byte protocol is local.

Common incorrect answer: Checking only POLLIN and spinning forever after POLLHUP or POLLERR.

Follow-up: What if the control pipe fills before a shutdown request is written?

Scoring out of 4: 1 point each: both sources; flag handling; nonblocking reads; ownership and wake failure.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q064 Implement a bounded newline decoder for fragmented stream input.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Streaming parser state and overflow recovery.

Spoken answer: Append bytes until newline, emit a complete line, and retain a partial line across reads. When capacity is exceeded, enter discard-until-newline mode and report one overlong record.

Deeper explanation: The decoder must handle "ab" then "c\nx\n" as two lines, abc and x. Define whether CRLF is normalized and whether empty lines are allowed. Do not assume each read contains a whole line. Once overflow occurs, consuming to a delimiter restores framing; resetting immediately on every byte can accidentally accept the tail of a malformed command.

Assumptions: Assume a byte-oriented newline protocol with a 64-byte maximum excluding the delimiter.

Common incorrect answer: NUL-terminating each read and treating it as one complete command.

Follow-up: How would binary framing differ when the payload may legitimately contain newline?

Scoring out of 4: 1 point each: retained partial state; multiple lines; overlong recovery; explicit grammar.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q065 Implement a monotonic deadline helper returning milliseconds remaining.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Time arithmetic, rounding, and poll integration.

Spoken answer: Subtract current monotonic time from an absolute deadline, return zero if expired, and round a positive fractional millisecond upward without overflowing the API's int limit.

Deeper explanation: Rounding down can create an immediate busy loop just before expiry; rounding up avoids declaring time remains while repeatedly polling with zero. Clamp very distant deadlines to INT_MAX and recompute after wakeup. Store nanoseconds in a sufficiently wide representation or normalize seconds/nanoseconds carefully. Unit tests should use injected timestamps rather than sleeping, so boundary behavior is deterministic.

Assumptions: Assume CLOCK_MONOTONIC semantics and poll's millisecond integer timeout.

Common incorrect answer: Subtracting timespec fields independently without borrowing or restarting the deadline after every event.

Follow-up: How would you expose an infinite timeout distinctly from an expired timeout?

Scoring out of 4: 1 point each: absolute subtraction; upward rounding; clamping; deterministic tests.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q066 Write a subprocess launcher that captures exit status without leaking descriptors.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Process lifecycle and error-channel design.

Spoken answer: Create needed pipes with close-on-exec, fork or use posix_spawn, close unused ends in each process, execute the child, and reap it with EINTR-aware waitpid handling.

Deeper explanation: Distinguish normal exit from signal termination with the wait-status macros. An exec failure should be communicated separately from the child's eventual application exit code, for example through a close-on-exec error pipe. For multithreaded parents, restrict child-side operations before exec or prefer posix_spawn. Test missing executable, exit zero, exit seven, and signal termination using controlled local commands.

Assumptions: Assume trusted executable paths and arguments passed as an argv vector, not concatenated shell text.

Common incorrect answer: Passing untrusted arguments through a shell or interpreting raw wait status as the exit code.

Follow-up: How would you enforce a subprocess timeout and avoid orphaning grandchildren?

Scoring out of 4: 1 point each: descriptor ends; exec failure; status macros; reaping and trusted arguments.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q067 Write a TCP request parser that accepts a 16-bit length followed by payload.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Message framing over a stream and memory limits.

Spoken answer: Maintain header and payload states, accumulate partial input, reject lengths above the configured maximum, and emit only after the exact declared payload has arrived.

Deeper explanation: Zero-length messages need a deliberate policy. One recv may contain part of a header, a whole message, or several messages, so return consumed-byte counts and continue decoding. A disconnect mid-payload is a truncated-message error. Pair a total frame deadline with a maximum size to prevent a peer from holding memory indefinitely by sending slowly. Exercise every possible split of a small valid frame.

Assumptions: Assume network-order length, maximum payload 1024 bytes, and a byte-stream socket.

Common incorrect answer: Allocating directly from an unchecked length or equating a recv call with one request.

Follow-up: How do you prevent queued complete messages from exhausting memory when the consumer is slow?

Scoring out of 4: 1 point each: parser states; size cap; split/coalesced frames; deadline/backpressure.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q068 Implement atomic replacement of a small configuration file with error checking.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Persistence sequencing and recovery from partial failure.

Spoken answer: Create a unique temporary file in the destination directory, write the complete validated content, sync it, rename it over the destination, and sync the directory when required.

Deeper explanation: Check every operation and clean up the temporary path on pre-rename failures. Preserve the old configuration until replacement succeeds. Set ownership and permissions deliberately; avoid predictable temporary names and symlink races. Failure after rename may mean the new version is visible but durability is uncertain, so report that state rather than pretending nothing changed. Test with injected failure at each stage in a disposable directory.

Assumptions: Assume local filesystem support for the chosen operations and a defined crash-consistency contract.

Common incorrect answer: Writing directly into the live file or treating an unchecked fflush as durable storage.

Follow-up: How would you handle concurrent writers so a stale update does not overwrite a newer configuration?

Scoring out of 4: 1 point each: sequence; secure temp; failure states; concurrency/version consideration.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q069 Implement a bounded producer-consumer queue with clean shutdown.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Condition-variable predicates and lifetime-safe concurrency.

Spoken answer: Protect queue state with a mutex; wait in predicate loops for space or data; publish state before notification; and include a shutdown flag in every blocking predicate.

Deeper explanation: Define whether shutdown drains queued items or discards them. Broadcast to wake both producer and consumer waiters, then join all threads before destroying synchronization objects. Spurious wakeups require rechecking conditions. A test should stop while empty, stop while full, run multiple producers, and verify each accepted item is consumed once under the chosen drain policy.

Assumptions: Assume POSIX threads and a fixed-capacity queue; cancellation behavior is explicitly out of scope unless implemented.

Common incorrect answer: Using if instead of while around condition waits or destroying a mutex while a worker can still access it.

Follow-up: How would you report that a producer's item was rejected because shutdown began?

Scoring out of 4: 1 point each: protected predicates; loops; shutdown policy; join-before-destroy.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

#### Q070 Parse a /proc PID stat record robustly enough to extract state and PID.

Track: Embedded Linux | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Parsing a kernel text interface with nontrivial delimiters.

Spoken answer: Parse the numeric PID before the opening parenthesis, locate the command field's closing delimiter under the documented format, then parse the following state field with bounds checks.

Deeper explanation: The command field may contain spaces and parentheses, so a naive whitespace split is invalid. For this exercise, validate the trailing ) state  layout and reject malformed records. Include fixtures with worker name, a command containing ), truncated data, and PID overflow. Reading /proc is racy: a missing process is an expected operational outcome, not necessarily a program defect.

Assumptions: Assume Linux proc_pid_stat format; extraction requirements are deliberately limited to PID and state.

Common incorrect answer: Assuming the command is always a single token or assigning a failure to PID zero silently.

Follow-up: How would you avoid accidentally associating observations from a reused PID with the old process?

Scoring out of 4: 1 point each: delimiter awareness; numeric bounds; malformed cases; process-lifetime race.

Learning connection: Module M06. Verification references: READ, WRITE, POLL, PROC, FSYNC, CLOCK, PTHREAD.

### Concurrency and IPC

#### Q071 Why must a condition variable be paired with a predicate and mutex?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Lost-wakeup prevention and state-based waiting.

Spoken answer: The condition variable is a notification mechanism; shared state is the truth. Protect the predicate with a mutex and wait in a loop that rechecks it after waking.

Deeper explanation: The wait operation releases the mutex and blocks as one coordinated operation, then reacquires it before returning. This avoids a gap in which a producer changes state and sends a notification just before the consumer actually sleeps. Spurious wakeups and competing consumers still require a loop. Update the predicate while holding its lock, then signal or broadcast according to who may now proceed.

Assumptions: Assume POSIX condition variables and consistent use of the same mutex for the predicate.

Common incorrect answer: Treating a signal as a stored event token that a later waiter is guaranteed to consume.

Follow-up: When should shutdown broadcast rather than signal one waiter?

Scoring out of 4: 1 point each: predicate; atomic wait transition; while loop; correct notification scope.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q072 How does deadlock differ from starvation and livelock?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Classifying lack of progress and selecting evidence.

Spoken answer: Deadlock is a cycle of waits that cannot resolve internally. Starvation denies one participant progress while others run. Livelock performs activity without useful completion.

Deeper explanation: Two threads acquiring A then B and B then A can deadlock. A low-priority task may starve under continuous higher-priority work. Two peers repeatedly backing off in synchrony can livelock. Capture stacks, ownership, scheduling traces, and progress counters to distinguish them. A timeout may restore service but does not automatically eliminate the underlying resource-order or fairness defect.

Assumptions: Assume a defined notion of useful progress and observable task state.

Common incorrect answer: Calling every hung application a deadlock or using sleep calls as a permanent locking fix.

Follow-up: How would a global lock ordering prevent the two-lock example?

Scoring out of 4: 1 point each: three distinctions; concrete examples; diagnostic evidence; structural remedy.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q073 When is an atomic counter insufficient to protect a data structure?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Atomicity versus compound invariants.

Spoken answer: An atomic counter protects that counter's operations, not an invariant involving several fields or an object's lifetime. Choose synchronization covering the complete state transition.

Deeper explanation: A queue's head, tail, slot contents, and storage lifetime must agree. Publishing a new head before the item is visible can let a consumer read uninitialized data. Acquire-release may support a carefully designed single-producer single-consumer queue, but does not make an arbitrary multi-producer design correct. A mutex is often simpler when the operation spans several related fields.

Assumptions: Assume the C memory model for user space; Linux kernel atomics have their own documented model.

Common incorrect answer: Replacing every shared integer with an atomic and declaring the whole algorithm lock-free and correct.

Follow-up: What linearization point would you choose for a successful queue push?

Scoring out of 4: 1 point each: invariant scope; publication; producer assumptions; simpler alternative.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q074 What is priority inversion, and what does priority inheritance solve?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Scheduling interactions through shared resources.

Spoken answer: A high-priority task can wait for a lower-priority lock owner while unrelated medium-priority work delays that owner. Priority inheritance temporarily raises the owner's effective priority.

Deeper explanation: Inheritance reduces this specific scheduling interference; it does not shorten the critical section, prevent deadlock, or bound arbitrary I/O performed while holding the lock. Multiple locks and nested dependencies need careful analysis. On Linux, use documented priority-inheritance primitives when required; on an RTOS, inspect its actual implementation and restrictions. Keep shared critical sections short and avoid blocking operations inside them when deadlines matter.

Assumptions: Assume fixed-priority scheduling for the illustrative high/medium/low example.

Common incorrect answer: Claiming priority inheritance guarantees no blocking or makes a poor lock hierarchy safe.

Follow-up: What would you measure to separate inversion from an intrinsically long critical section?

Scoring out of 4: 1 point each: three-task mechanism; temporary boost; limits; measurement.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q075 How should shared memory between processes be synchronized?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Cross-process lifetime, initialization, and publication.

Spoken answer: Define a binary layout, initialize it exactly once, and use synchronization explicitly supported for interprocess sharing. Ordinary in-process assumptions do not automatically apply.

Deeper explanation: POSIX process-shared mutexes require appropriate attributes and placement in shared storage. Robust mutexes can report owner death, but the next owner must repair or reject inconsistent data before declaring the state usable. Include version, size, and initialization state in the shared ABI. A process crash can leave a half-updated record even though shared memory itself persists.

Assumptions: Assume a Linux/POSIX implementation supporting the selected process-shared or robust primitive.

Common incorrect answer: Placing a default mutex in shared memory and assuming it works correctly across processes.

Follow-up: How would you recover if a writer dies halfway through updating a multi-field record?

Scoring out of 4: 1 point each: shared attributes; initialization; owner-death recovery; versioned layout.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q076 What is backpressure, and where should it appear in telemetry systems?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Bounded resource usage under overload.

Spoken answer: Backpressure makes producers slow, reject, or adapt when consumers cannot keep up. Every queue needs an explicit bound and overload policy.

Deeper explanation: For critical alarms, dropping silently may be unacceptable; for high-rate trend samples, coalescing or dropping oldest data may be appropriate. A serial input cannot always be paused, so count losses and reserve capacity for important events. Applying backpressure only at the final network queue can still let earlier queues grow without bound. Test sustained overload, not just short bursts.

Assumptions: Assume requirements distinguish command, alarm, and best-effort telemetry semantics.

Common incorrect answer: Using an unbounded queue as the solution to a slow consumer.

Follow-up: How would you expose overload in health metrics without producing even more unbounded logs?

Scoring out of 4: 1 point each: bounded queues; chosen policy; end-to-end path; observable loss.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q077 Why is cancellation harder than adding a stop flag?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Interrupting waits and coordinating object lifetime.

Spoken answer: A flag expresses intent but does not wake blocked I/O, condition waits, or sleeping threads. Cancellation must reach each blocking point and define ownership of unfinished work.

Deeper explanation: Use a wake descriptor, condition broadcast, timeout, or API-specific cancellation mechanism. Define who completes a request when stop races with normal completion. Join or otherwise establish quiescence before freeing shared context. A stop flag must itself be synchronized in a multithreaded C program. Tests should trigger shutdown while empty, full, reconnecting, and processing a partial message.

Assumptions: Assume cooperative cancellation; asynchronous thread cancellation has additional cleanup hazards.

Common incorrect answer: Setting a volatile boolean and immediately freeing the worker's context.

Follow-up: How would you guarantee exactly one completion callback for a request canceled at the same time it succeeds?

Scoring out of 4: 1 point each: wake paths; synchronized state; completion race; lifetime barrier.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q078 What is false sharing, and how would you establish that it matters?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Cache effects versus correctness failures.

Spoken answer: False sharing occurs when independent writable objects occupy a cache line that moves between cores. It can reduce performance despite logically independent accesses.

Deeper explanation: Separate per-thread hot counters or redesign aggregation after measuring cache-coherence behavior and throughput under controlled CPU placement. Padding may help but increases memory footprint and depends on target cache-line size. Do not confuse false sharing with a data race: each thread may own its own variable correctly. A benchmark in a noisy virtual machine may not represent the embedded target's memory hierarchy.

Assumptions: Assume a coherent multicore system and a workload with frequent writes.

Common incorrect answer: Adding arbitrary padding everywhere before measuring or calling false sharing undefined behavior.

Follow-up: How would you compare padded and unpadded layouts without changing unrelated benchmark conditions?

Scoring out of 4: 1 point each: cache-line mechanism; correctness distinction; measurement; target-aware remedy.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q079 What does lock-free mean, and what does it not guarantee?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Progress guarantees and realistic use of atomics.

Spoken answer: Lock-free algorithms guarantee system-wide progress under their formal assumptions; they do not guarantee every thread completes promptly. Wait-free is a stronger per-operation progress property.

Deeper explanation: An atomic variable alone does not establish an algorithm's progress guarantee. Compare-and-swap loops may starve one participant, and memory reclamation can be the hardest part. On a platform, a language atomic type may internally use a lock unless lock-free support is confirmed. For small embedded queues, a clear mutex design may outperform a complicated atomic design in maintainability and predictable behavior.

Assumptions: Assume formal progress terms are used with the required scheduler and atomic-operation assumptions.

Common incorrect answer: Equating lock-free with faster, bounded latency, or simply no explicit mutex in the source.

Follow-up: How would you reclaim a node while another thread may still hold its address?

Scoring out of 4: 1 point each: system progress; starvation distinction; implementation support; reclamation concern.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

#### Q080 When would you choose a pipe, Unix socket, shared memory, or message queue?

Track: Embedded Linux and Kernel | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Selecting IPC from semantics instead of preference.

Spoken answer: Choose by message boundaries, transfer volume, topology, permissions, failure handling, and lifecycle. Pipes suit simple streams; Unix sockets support richer local communication; shared memory needs explicit coordination.

Deeper explanation: A local command service can use a Unix-domain socket with bounded framed messages and peer credentials. A high-volume shared-memory ring can reduce copying but adds initialization, synchronization, and crash-recovery complexity. Message queues may preserve records but impose size and capacity constraints. Measure before optimizing copies, and define behavior when a peer exits or restarts.

Assumptions: Assume local Linux processes; portability requirements may narrow the available APIs.

Common incorrect answer: Selecting shared memory automatically because it is presumed fastest without considering recovery and ownership.

Follow-up: How would you version messages so independently updated peers can reject incompatible requests safely?

Scoring out of 4: 1 point each: selection factors; concrete choice; failure/restart; versioning.

Learning connection: Module M07. Verification references: PTHREAD, LOCKS, FUTEX, C11.

### Linux debugging scenarios

#### Q081 A daemon uses 100 percent CPU after its serial peer disconnects. What do you inspect?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Evidence-led diagnosis of an event-loop spin.

Spoken answer: Capture a short syscall trace and loop counters. Look for repeated readiness or hangup returns followed by zero-byte reads, unhandled errors, or a retry path with no wait.

Deeper explanation: The provided spin.trace fixture repeatedly shows poll returning POLLHUP and read returning zero. That points to EOF/hangup handling, not an expensive computation. Close or transition the connection into a bounded reconnect state after draining any valid buffered data. Verify the fix with disconnect, reconnect, and a silent peer, while measuring CPU over an agreed interval.

Assumptions: Assume the fixture is illustrative teaching data; actual devices may report different error combinations.

Common incorrect answer: Adding an arbitrary sleep without explaining why the loop remains active.

Follow-up: How would you avoid a new busy loop if reconnect itself fails immediately?

Scoring out of 4: 1 point each: trace hypothesis; EOF interpretation; state transition; regression/CPU check.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q082 A parser passes whole-frame tests but fails on real UART traffic. What is likely missing?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Transport-independent framing and fragmentation tests.

Spoken answer: UART supplies a byte stream, so frames may be split or combined across reads. Inspect retained parser state, length checks, and recovery after malformed bytes.

Deeper explanation: Replay a known frame through every split point, then concatenate two frames and inject junk between them. The provided fragments.json includes a header split across reads and a truncated declared payload. Ensure the parser consumes bytes monotonically and has a bounded resynchronization strategy. Real electrical errors remain a separate validation concern even after host fragmentation tests pass.

Assumptions: Assume baud rate and framing configuration are otherwise correct; verify those rather than presuming them.

Common incorrect answer: Increasing the read buffer until one read usually contains a frame.

Follow-up: How would a partial frame time out without discarding a valid following frame?

Scoring out of 4: 1 point each: stream model; exhaustive splits; malformed recovery; timeout/electrical separation.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q083 A process reports ENOENT when executing a file that visibly exists. How can that happen?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: ELF interpreter and script-launch dependencies.

Spoken answer: The missing object may be the executable's interpreter rather than the file itself. Inspect the ELF program headers or script shebang and the target filesystem.

Deeper explanation: A dynamically linked ELF can name a loader absent from a minimal root filesystem. A script can reference an unavailable interpreter or contain a malformed shebang with an unintended carriage return. Compare file, readelf -l, architecture, and interpreter path using the exact deployed binary. Do not treat chmod as the universal fix; permission failures have different evidence.

Assumptions: Assume Linux exec behavior and a trusted local binary under inspection.

Common incorrect answer: Recreating the file repeatedly or claiming ENOENT proves the pathname lookup alone failed.

Follow-up: How would a static binary change the deployment requirements, and what would it not solve?

Scoring out of 4: 1 point each: interpreter cause; inspection commands; architecture/rootfs; error distinction.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q084 A service works as root but fails under its intended account. How do you fix it?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Permission analysis without weakening the entire system.

Spoken answer: Identify the exact failing operation and errno, then compare identity, groups, device-node permissions, directory traversal rights, and service sandbox settings.

Deeper explanation: Use a controlled trace or structured error log to distinguish opening /dev/ttyX from writing the state directory. Grant the narrow device or directory access actually required and ensure the service sees supplementary-group changes. If a systemd restriction blocks an operation, modify only the relevant policy and retest. Preserve the intended account and record why each privilege is needed.

Assumptions: Assume the target uses documented Linux permissions and possibly a service manager.

Common incorrect answer: Running permanently as root or setting device nodes world-writable to make the test pass.

Follow-up: How would a udev rule make the device permission persistent across reboot and reconnect?

Scoring out of 4: 1 point each: failing syscall; identity/path checks; narrow fix; persistence test.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q085 Memory usage grows slowly while the process stays responsive. How do you separate a leak from caching?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Resource accounting and experiment design.

Spoken answer: Measure memory over a repeatable workload, distinguish live allocations from retained allocator arenas or intentional caches, and track object counts and ownership paths.

Deeper explanation: Run repeated connect-disconnect or request cycles, then observe whether live application objects return to baseline. A heap profiler or sanitizer can identify leaks in exercised paths, while RSS alone cannot prove them. Check unbounded queues and log buffers as well as missing free calls. Set explicit cache limits and eviction policies; a designed cache can still violate a memory budget.

Assumptions: Assume host tooling is supported and the workload can be replayed with stable input volume.

Common incorrect answer: Declaring a leak from one rising RSS sample or calling all retained memory harmless caching.

Follow-up: What evidence would show that cancellation paths leak request objects?

Scoring out of 4: 1 point each: repeatable workload; live-versus-resident distinction; ownership counts; bounded policy.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q086 A multithreaded application hangs only during shutdown. What is your first investigation?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Wait dependencies and destruction ordering.

Spoken answer: Capture all thread stacks and the shared shutdown state. Look for a worker blocked on a predicate that never includes shutdown, an unwoken wait, or a join holding a needed lock.

Deeper explanation: A common failure is the main thread holding the queue mutex while joining a worker that needs that mutex to exit. Another is signaling one waiter while several remain asleep. Draw a wait-for relationship from evidence, then repair wakeup and ownership order. Test shutdown from empty, full, processing, and error states rather than relying on a normal-exit demo.

Assumptions: Assume cooperative shutdown and a reproducible or captured hang.

Common incorrect answer: Detaching all threads to hide the hang while their shared memory is freed.

Follow-up: How would you make a shutdown test deterministic instead of using arbitrary sleeps?

Scoring out of 4: 1 point each: thread stacks; wait graph; join/lock ordering; state-based regression.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q087 A network request sometimes contains duplicated bytes. What write-loop bug would you check?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Partial-write bookkeeping and reproducible transport tests.

Spoken answer: Check whether the loop advances its pointer and remaining count after each positive write. Retrying the entire buffer after a short write duplicates the already accepted prefix.

Deeper explanation: Instrument requested length, returned count, and cumulative offset without logging sensitive payloads. Use an injected writer that deliberately returns small counts and EINTR to make the bug deterministic. Distinguish this from application-level retries after an uncertain acknowledgment, which require request IDs and idempotency. One is byte-stream bookkeeping; the other is delivery semantics across a reconnect.

Assumptions: Assume the receiver's parser is independently tested and transport ordering is preserved.

Common incorrect answer: Blaming TCP retransmissions for duplicate bytes delivered to one reliable stream.

Follow-up: How would you safely retry a command when the connection drops after the server may have applied it?

Scoring out of 4: 1 point each: partial offset; injected writes; transport distinction; idempotency.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q088 A periodic task drifts farther behind each minute. What timing pattern is suspect?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Periodic scheduling with absolute versus relative deadlines.

Spoken answer: A loop that does work and then sleeps a fixed period adds its work and scheduling delay to every cycle. Schedule against successive absolute deadlines instead.

Deeper explanation: Record intended wake time, actual wake time, execution duration, and missed-deadline count separately. Decide whether an overrun skips missed releases, catches up, or enters a degraded state; blindly executing many overdue cycles can worsen overload. Use a monotonic clock and appropriate absolute-sleep APIs. Averages alone can hide rare deadline misses that matter more than small steady drift.

Assumptions: Assume the period is an application requirement and clock suspend semantics are defined.

Common incorrect answer: Reducing the sleep duration by one observed average work time and assuming drift is solved.

Follow-up: What policy would you choose if one cycle takes three times the nominal period?

Scoring out of 4: 1 point each: relative-drift mechanism; absolute schedule; overrun policy; latency measurements.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q089 A cross-compiled program crashes on one board but passes on the host. What do you compare?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Architecture-sensitive faults and environment differences.

Spoken answer: Compare architecture, ABI, endianness, alignment assumptions, integer widths, libraries, and inputs. Reproduce with the exact target binary and capture a backtrace or core when possible.

Deeper explanation: A parser that casts bytes to a structure may tolerate unaligned access on one host and fault or behave differently elsewhere. A missing synchronization barrier may appear only under a different memory model. Use compiler warnings, sanitizers where supported, and portable byte decoding to isolate assumptions. Host success validates host behavior, not board timing or peripheral correctness.

Assumptions: Assume the target executable and debug symbols correspond to the same build.

Common incorrect answer: Rebuilding with optimization disabled and calling the problem fixed without understanding the cause.

Follow-up: How would you verify that gdbserver and the debugger agree on the target's symbols and architecture?

Scoring out of 4: 1 point each: environment comparison; concrete portability bug; matching symbols; validation limits.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

#### Q090 A configuration file becomes empty after power loss. What is wrong with a direct overwrite?

Track: Embedded Linux | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Crash-consistent updates and evidence-based persistence.

Spoken answer: Truncating the live file before writing exposes an empty or partial version if power fails. Use a validated temporary file, syncing and atomic replacement under a tested filesystem contract.

Deeper explanation: The fix must handle data and directory durability, writeback errors, and permissions. Keep a known-good fallback or versioned copy if the product requires recovery from corruption. Inject failures at each operation in a disposable image, then power-cut test the actual storage stack when available. Passing a host rename test is not evidence of board-level power-failure robustness.

Assumptions: Assume the filesystem offers the required rename and sync semantics; verify rather than generalize.

Common incorrect answer: Adding fflush alone or claiming atomic visibility guarantees durable persistence.

Follow-up: How would the application choose between two versioned configuration records after a torn write?

Scoring out of 4: 1 point each: truncate window; safe sequence; recovery selection; target power-cut evidence.

Learning connection: Module M15. Verification references: READ, WRITE, POLL, FSYNC, CLOCK, PROC, PTHREAD.

### Concurrency and fault analysis scenarios

#### Q091 A counter occasionally loses increments despite being volatile. How do you demonstrate the defect?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Data-race diagnosis and independent verification.

Spoken answer: Identify concurrent read-modify-write operations without synchronization. Replace the mechanism with an atomic operation or mutex, then test the required invariant under concurrent load.

Deeper explanation: A stress test can compare the final count with known accepted work, and ThreadSanitizer may report exercised races on a supported host. Neither a failed run nor a clean run alone defines the language semantics: the unsynchronized C accesses are already invalid. Choose relaxed atomics only if the counter is independent of other published state. Record sanitizer support and actual flags.

Assumptions: Assume user-space C11 threads; volatile MMIO is not the object under test.

Common incorrect answer: Claiming the fix is to force optimization off or make the variable wider.

Follow-up: What if the counter also indicates that a corresponding data buffer is ready?

Scoring out of 4: 1 point each: RMW race; proper primitive; invariant test; ordering qualification.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q092 A consumer sleeps although the queue contains data. What lost-wakeup pattern do you inspect?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Atomic relationship between predicate checks and sleeping.

Spoken answer: Check whether the consumer tests the predicate outside its mutex or releases the mutex before starting a separate wait. The state check and wait transition must use the correct condition-variable pattern.

Deeper explanation: A producer can enqueue and signal during the gap, leaving no later notification to wake the consumer. The fix is a mutex-protected predicate loop around pthread_cond_wait, not an extra signal placed randomly. Also inspect whether producer and consumer use different mutexes or inconsistent predicates. Build a test barrier that forces the suspect interleaving instead of hoping stress reproduces it.

Assumptions: Assume POSIX condition variables and shared in-process queue state.

Common incorrect answer: Treating condition notifications as a durable count of pending items.

Follow-up: How can multiple consumers turn a single if check into an empty-queue access after wakeup?

Scoring out of 4: 1 point each: gap identification; correct wait loop; same-lock predicate; forced interleaving.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q093 A two-thread program locks up after each thread takes a different mutex. What is the repair?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Lock-order deadlock and design-level prevention.

Spoken answer: Confirm the circular wait with thread stacks and lock ownership. Establish a consistent global lock order or redesign the operation so both locks are unnecessary.

Deeper explanation: If one path takes A then B and another B then A, both can wait forever. A try-lock-and-backoff approach needs a progress analysis and can livelock; it is not automatically better. Document the order near interfaces and test opposing operations concurrently. Avoid joining a thread or calling an unknown callback while holding a lock the other path may need.

Assumptions: Assume nonrecursive mutexes and no external mechanism deliberately breaking the wait.

Common incorrect answer: Using recursive mutexes as a cure for a cycle involving two different threads.

Follow-up: How would lockdep or an application lock-order assertion help catch future inversions?

Scoring out of 4: 1 point each: wait cycle; consistent order; callback/join risk; regression prevention.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q094 A timer callback accesses freed state after device removal. What lifetime ordering is missing?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Asynchronous activity teardown and use-after-free prevention.

Spoken answer: Removal must prevent new scheduling, stop or cancel the timer synchronously as appropriate, and wait for in-flight callbacks before releasing state they can access.

Deeper explanation: A flag alone is insufficient if the callback reads freed memory before checking it. A callback may also rearm itself, so close that path before synchronization. The exact kernel timer/work APIs and allowed calling context matter. Exercise repeated load-use-unload or bind-unbind cycles in a disposable VM with supported memory debugging, then inspect retained evidence rather than assuming devm allocation handles asynchronous activity.

Assumptions: Assume a Linux driver and the selected kernel's timer API contract.

Common incorrect answer: Freeing memory first and hoping the callback sees a stop flag.

Follow-up: How can calling a synchronous cancellation routine while holding a callback-needed lock deadlock?

Scoring out of 4: 1 point each: prevent rearm; synchronous quiescence; free-last; lock/context caveat.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q095 A driver fails with a sleeping-in-atomic-context warning. What do you inspect?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Execution context and API preconditions.

Spoken answer: Identify the current context and held locks, then inspect the call chain for sleeping operations such as mutex acquisition, GFP_KERNEL allocation, or faulting user copies.

Deeper explanation: On a conventional non-RT kernel, spinlocked and hard-IRQ paths cannot sleep. Move substantial work into an appropriate threaded context or redesign resource acquisition before the atomic section. Do not blindly substitute GFP_ATOMIC everywhere: allocation can fail and may still be unsuitable. PREEMPT_RT changes some lock semantics, so reason from the pinned configuration and API rules rather than a slogan.

Assumptions: Assume the warning trace is available and kernel version/configuration are known.

Common incorrect answer: Disabling the warning or changing one allocator flag without addressing the execution-context design.

Follow-up: How would you divide a hardware interrupt handler from deferred processing?

Scoring out of 4: 1 point each: context; sleeping call; structural move; version/config caveat.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q096 A low-priority logger causes a high-priority control task to miss deadlines. How can this occur?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Priority inversion through a shared resource.

Spoken answer: The control task may wait for a mutex held by the logger while medium-priority tasks prevent the logger from running. Capture ownership and scheduling history to test that hypothesis.

Deeper explanation: Shorten the shared critical section, avoid slow I/O under the lock, and use appropriate priority inheritance where supported. Consider transferring log records through a bounded queue so control never waits for storage. Inheritance does not fix a filesystem write that itself stalls for a long time. Measure worst observed blocking under representative interference, and state that observed maxima are not formal worst-case bounds.

Assumptions: Assume fixed-priority tasks and a shared lock identified in the trace.

Common incorrect answer: Simply raising every task to the highest priority or calling average latency a deadline guarantee.

Follow-up: What information must be logged to separate inversion from CPU overload?

Scoring out of 4: 1 point each: ownership chain; scheduling evidence; bounded redesign; measured-versus-proven bound.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q097 A device works until runtime suspend is enabled. Where do you start?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Power-state-dependent resources and access lifetimes.

Spoken answer: Correlate failures with suspend/resume events and inspect whether each access holds the required runtime-PM reference and whether registers, clocks, regulators, and interrupts are restored correctly.

Deeper explanation: A cached software state can disagree with reset hardware after resume. Ensure deferred work cannot access a powered-down device, and balance references on error paths. Test repeated idle-resume cycles, requests racing with suspend, and removal during suspended state. Disable autosuspend only as a diagnostic experiment; it does not establish the final fix. Use the selected subsystem's PM conventions.

Assumptions: Assume the hardware and driver advertise runtime power management support.

Common incorrect answer: Adding a long delay after every request without checking the device power state.

Follow-up: What state must be replayed after a power domain loses register contents?

Scoring out of 4: 1 point each: correlation; PM references; restore/quiesce; race tests.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q098 A DMA buffer contains stale data on one architecture. What could host testing have missed?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: DMA ownership, cache coherency, and address domains.

Spoken answer: Inspect use of the DMA API, transfer direction, map/unmap or sync transitions, and ownership between CPU and device. CPU virtual addresses are not interchangeable with DMA addresses.

Deeper explanation: A coherent x86 host can hide cache-maintenance bugs that appear on another target. Even coherent allocations do not remove every ordering requirement. Do not access a streaming DMA buffer from the CPU while the device owns it unless the documented synchronization protocol permits it. Check mapping errors and alignment restrictions, then validate on the actual bus/SoC with known patterns.

Assumptions: Assume a Linux driver using a real DMA-capable device and the selected DMA API contract.

Common incorrect answer: Calling cache flush functions ad hoc or passing a kmalloc pointer directly to hardware as an address.

Follow-up: How would an IOMMU make a physical-address assumption fail?

Scoring out of 4: 1 point each: ownership; API mapping; address domains; target coherency test.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q099 A firmware task resets under heavy logging but never in the debugger. What hypotheses fit?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: Timing perturbation, stack usage, and watchdog diagnosis.

Spoken answer: Collect reset cause, fault registers, stack high-water evidence, and watchdog timing. Heavy logging can increase stack use, block critical work, or change interrupt timing.

Deeper explanation: A debugger can halt watchdogs or slow execution, so its absence of failure is not decisive. Use bounded, low-overhead event records and reproduce at full speed. Inspect formatting functions, queue capacity, interrupt priorities, and whether the watchdog is fed from a path that masks system failure. Preserve fault data in a mechanism that survives the relevant reset without relying on unsafe fault-time printing.

Assumptions: Assume MCU-specific reset and fault information is available from its reference manual.

Common incorrect answer: Increasing every task's stack or watchdog period without identifying the violated budget.

Follow-up: How would you distinguish stack overflow from a watchdog timeout that occurs while interrupts are disabled?

Scoring out of 4: 1 point each: reset evidence; timing perturbation; resource budgets; targeted reproduction.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

#### Q100 An update passes in normal testing but bricks the device after power interruption. What was missing?

Track: Shared | Difficulty: Intermediate | Round: Debugging round | Tags: debugging;scenario

Assesses: State-transition fault injection and recovery guarantees.

Spoken answer: Test interruption at every persistent transition: download, verification, inactive-slot write, boot-selection update, first boot, health confirmation, and rollback. Validate an always-available recovery path.

Deeper explanation: A/B storage alone is insufficient if metadata updates are not crash safe or if both slots depend on a shared incompatible data schema. Signed payload verification must precede activation, while health confirmation must reflect useful operation. Simulate interruption in disposable images, then test the actual bootloader and storage. Record exact boot attempts and slot states; do not invent successful power-cut coverage.

Assumptions: Assume a defined bootloader/update framework and a documented partition layout.

Common incorrect answer: Declaring rollback works because a manual reboot successfully starts the new image once.

Follow-up: How should persistent application data remain compatible with rollback to the previous software?

Scoring out of 4: 1 point each: transition matrix; metadata safety; health/rollback; shared-data compatibility.

Learning connection: Module M15. Verification references: PTHREAD, LOCKS, DEVRES, DMA, PM, FREERTOS, RAUC.

### Architecture boot and device tree

#### Q101 How is a Linux application processor different from a typical microcontroller target?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Platform assumptions and selecting the correct software model.

Spoken answer: An application processor commonly runs a rich OS with virtual memory and external memory; a microcontroller commonly emphasizes integrated peripherals, bounded resources, and direct or RTOS-controlled execution.

Deeper explanation: These are practical categories, not absolute definitions. Check the actual core, MMU or MPU, memory map, boot chain, and board support. A Cortex-M firmware example cannot simply be compiled as a Linux driver, and a Linux userspace program relies on services absent from bare metal. Shared C modules can still be portable if hardware, time, and concurrency are abstracted behind explicit interfaces.

Assumptions: Assume representative ARM platforms; exact capabilities come from the selected part documentation.

Common incorrect answer: Equating every ARM device with Raspberry Pi or expecting POSIX processes on any MCU.

Follow-up: Which portions of a packet parser could be shared unchanged across both targets?

Scoring out of 4: 1 point each: practical distinction; hardware verification; software services; portable boundary.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q102 What do an MMU and an MPU each provide?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Memory protection and address-translation concepts.

Spoken answer: An MMU can translate virtual addresses and enforce permissions through mappings. An MPU typically applies access permissions and attributes to regions without providing the same general virtual-memory model.

Deeper explanation: The exact features depend on architecture. Linux processes normally rely on virtual-memory isolation and page tables on supported MMU platforms, while MCU firmware can use an MPU to separate task stacks or protect code and peripheral regions. Neither unit automatically fixes a buffer overflow in permitted memory. Region alignment, count, and cache attributes constrain an MPU design.

Assumptions: Assume a documented architecture; Linux also has specialized no-MMU configurations that need separate treatment.

Common incorrect answer: Claiming an MPU is just a smaller page table or that memory protection eliminates all memory corruption.

Follow-up: How would a DMA engine's access differ from CPU accesses checked by an MPU?

Scoring out of 4: 1 point each: translation; region protection; limitations; non-CPU access caveat.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q103 Describe a representative embedded Linux boot path without assuming every board uses U-Boot.

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Boot-stage responsibilities and platform-specific qualification.

Spoken answer: A platform may execute immutable ROM, early firmware, a bootloader, the kernel, and init or a service manager. Each stage initializes enough state and verifies or loads the next as configured.

Deeper explanation: Some boards use U-Boot SPL and U-Boot proper; others use different firmware or direct kernel loading. Record the board's actual chain, console, image formats, device tree, and root filesystem source. A kernel boot message does not prove userspace started. Debug by finding the last known-good stage and the expected handoff inputs, rather than reflashing unrelated components.

Assumptions: Assume a chosen board or QEMU machine; the sequence is a representative model, not a universal prescription.

Common incorrect answer: Stating all ARM boards use the identical ROM-to-U-Boot sequence.

Follow-up: What evidence distinguishes a kernel decompression failure from a missing root filesystem?

Scoring out of 4: 1 point each: stages; responsibilities; board qualification; stage-specific evidence.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q104 How do initramfs and a persistent root filesystem differ?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Early userspace and root transition.

Spoken answer: An initramfs is an early in-memory filesystem that can provide initial userspace and prepare access to the final root. A persistent root filesystem resides on a storage or network source.

Deeper explanation: A minimal appliance can run entirely from initramfs, while another uses it to unlock storage, load drivers, or select a root slot before switching. Its contents and init program must match the target architecture. A kernel configured without the needed storage driver available at the required stage cannot mount the final root merely because the driver exists somewhere on that inaccessible filesystem.

Assumptions: Assume Linux boot configuration explicitly identifies the root and initial userspace behavior.

Common incorrect answer: Treating initramfs as just a compressed disk partition or assuming it is mandatory for every system.

Follow-up: Why might a storage driver need to be built into the kernel rather than available only as a module?

Scoring out of 4: 1 point each: early-memory role; persistent-root role; transition; driver availability.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q105 What belongs in device tree, and what belongs in a driver?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Hardware description versus implementation policy.

Spoken answer: Device tree describes hardware connections and properties according to bindings; the driver implements the device's behavior using those resources and subsystem interfaces.

Deeper explanation: A sensor node can specify compatibility, bus address, interrupt, supply, and clock references. Do not place arbitrary Linux implementation details into bindings when they are not hardware properties. Matching can succeed while probe still fails because a referenced supplier is unavailable. Validate the tree against binding schemas and inspect the live tree and logs on the running target.

Assumptions: Assume a platform using device tree; ACPI or other enumeration paths may apply elsewhere.

Common incorrect answer: Treating a device-tree node as executable driver code or adding undocumented properties until probe works.

Follow-up: Why is a binding a compatibility interface that deserves review beyond one driver revision?

Scoring out of 4: 1 point each: description; binding contract; resource dependencies; validation.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q106 What are pin control and GPIO control responsible for?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Avoiding confusion between pad configuration and logical line access.

Spoken answer: Pin control configures pin multiplexing and electrical attributes such as pull or drive settings. GPIO APIs acquire and operate logical GPIO lines with polarity and ownership rules.

Deeper explanation: A pin may need the correct mux before a GPIO function reaches the physical pad. The GPIO descriptor's active-low property lets a driver use logical asserted/deasserted values. Some GPIO controllers are reached through sleeping buses, so choose APIs and context accordingly. The schematic, SoC pin map, and board device tree must agree; a Linux line number alone is not a wiring specification.

Assumptions: Assume a selected board and descriptor-based Linux GPIO APIs.

Common incorrect answer: Hard-coding a legacy global GPIO number or treating physical header pin numbers as GPIO offsets.

Follow-up: What happens if a pin is still muxed to UART while a GPIO consumer toggles it?

Scoring out of 4: 1 point each: mux/electrical; logical GPIO; polarity/context; board mapping.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q107 Why are clocks and regulators explicit driver resources?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Dependency management and power-aware initialization.

Spoken answer: Devices need correct clocks and supplies before register access. Drivers obtain and sequence these resources through their frameworks so ownership, sharing, and power transitions are coordinated.

Deeper explanation: A regulator may be shared by several consumers, and a clock may require a rate suitable for the bus or peripheral. Probe failures must unwind acquired state, while suspend and runtime idle need a compatible sequence. A missing supplier can require deferred probing rather than permanent failure. Always follow the hardware's timing and sequencing requirements and the subsystem's resource conventions.

Assumptions: Assume the board bindings describe the relevant supplies and clocks correctly.

Common incorrect answer: Accessing registers first and adding arbitrary delays when reads return all ones.

Follow-up: How would you distinguish an absent regulator driver from a wrong voltage or enable pin in the board description?

Scoring out of 4: 1 point each: prerequisites; shared frameworks; sequencing/unwind; supplier diagnosis.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q108 What does a cross-compilation sysroot contain, and why is it important?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Target headers and libraries versus host contamination.

Spoken answer: A sysroot provides the target-facing headers and libraries used by a cross toolchain. It prevents accidental use of incompatible host interfaces when building target applications.

Deeper explanation: The compiler target triple, libc, ABI, dynamic loader, and linked libraries must agree with the deployed root filesystem. A successful compile can still produce a binary requesting a loader absent on the board. Use the SDK's environment and inspect verbose commands and ELF metadata. Do not copy random host shared libraries into the target image to resolve one error.

Assumptions: Assume a toolchain and SDK intended for the selected target distribution or image.

Common incorrect answer: Believing setting CC alone ensures every dependency is built for the target.

Follow-up: How can pkg-config accidentally supply host include paths during a cross build?

Scoring out of 4: 1 point each: sysroot role; ABI/libc match; ELF check; dependency isolation.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q109 How does an interrupt differ from polling, and when can polling be reasonable?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Latency, load, and execution-context tradeoffs.

Spoken answer: An interrupt notifies software of an event asynchronously; polling checks state at chosen intervals. Choose from event rate, latency, CPU budget, power, and hardware behavior.

Deeper explanation: For a low-rate sensor, interrupts can avoid repeated checks. Under very high sustained load, bounded polling can reduce interrupt overhead; some subsystems combine both approaches. An interrupt handler still needs to acknowledge the correct condition and defer inappropriate work. A polling period bounds observation delay only under the assumed scheduling behavior, not merely because a sleep duration was configured.

Assumptions: Assume the hardware documents event-latching and acknowledge semantics.

Common incorrect answer: Claiming interrupts are always faster or polling inherently guarantees deterministic latency.

Follow-up: How would you prevent a level-triggered interrupt storm caused by an uncleared source?

Scoring out of 4: 1 point each: mechanism; workload tradeoff; acknowledgment; scheduling qualification.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

#### Q110 What is memory ordering, and why can it matter even with coherent caches?

Track: Kernel BSP and Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Visibility order across cores or devices.

Spoken answer: Coherency concerns agreement about a location's value; ordering concerns when accesses to different locations become observable. Use the relevant synchronization or I/O primitives to establish required order.

Deeper explanation: A producer must initialize data before publishing a ready flag in the chosen memory model. A DMA descriptor may need its fields visible before a valid bit is exposed, even with coherent DMA memory. User-space C atomics, kernel barriers, and MMIO accessors serve different domains and are not interchangeable incantations. Start from the ownership and publication protocol, then choose documented primitives.

Assumptions: Assume a specified CPU, device, and software memory model; barriers are not substitutes for all synchronization.

Common incorrect answer: Using volatile or one generic barrier everywhere without identifying the ordering relation.

Follow-up: Why does a cache flush not by itself establish a complete concurrent object-lifetime protocol?

Scoring out of 4: 1 point each: coherence/order distinction; publication example; domains; ownership-first design.

Learning connection: Module M08. Verification references: DT, GPIO, DMA, UBOOT, ARM, QEMU.

### Kernel internals and lifetime

#### Q111 Why should most application logic remain in user space?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Privilege boundaries and choosing the narrowest kernel responsibility.

Spoken answer: User space provides isolation, mature libraries, easier deployment, and safer debugging. Kernel code is appropriate when hardware or core subsystem integration requires that privilege and execution model.

Deeper explanation: A sensor driver can expose measurements through IIO while filtering, storage, networking, and inference run in user space. Moving a daemon into a module does not automatically reduce latency meaningfully, and it enlarges the failure impact of bugs. Measure the actual bottleneck and use existing interfaces before designing a new kernel ABI. Keep a clear boundary between mechanism and product policy.

Assumptions: Assume the required device functionality can be represented by a suitable subsystem interface.

Common incorrect answer: Writing a kernel module merely because a product is embedded or needs a service to start at boot.

Follow-up: What evidence would justify moving part of a data path closer to the kernel?

Scoring out of 4: 1 point each: isolation; legitimate kernel role; subsystem boundary; measured justification.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q112 What are process context and interrupt context in kernel code?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Allowed operations and who may sleep.

Spoken answer: Process context runs on behalf of a schedulable task, although held locks or disabled preemption can restrict it. Hard interrupt context cannot perform operations that may sleep.

Deeper explanation: A file operation is generally in task context but may enter atomic sections internally. A hard IRQ handler should do bounded urgent work and arrange suitable deferred processing. Threaded IRQ handlers and workqueues provide different contexts with their own rules. Never infer permission to sleep from the function name alone; inspect call paths, locks, and the selected kernel configuration.

Assumptions: Assume Linux 6.12 documentation; PREEMPT_RT affects some locking and interrupt behavior.

Common incorrect answer: Saying all code called by a process may sleep or all deferred work has identical restrictions.

Follow-up: Which context would you choose for an I2C transaction triggered by a GPIO interrupt?

Scoring out of 4: 1 point each: task context; IRQ restriction; held-lock caveat; deferred-context selection.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q113 How do kmalloc, vmalloc, and DMA allocation serve different needs?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Address-space, contiguity, and device access requirements.

Spoken answer: Kmalloc provides physically contiguous kernel allocations subject to allocator limits; vmalloc provides virtually contiguous mappings; DMA allocations or mappings meet a device's DMA contract.

Deeper explanation: Do not choose a buffer merely by whether the CPU can dereference it. A large vmalloc area is not a single physically contiguous block suitable for an arbitrary device. DMA APIs handle device-visible addresses, masks, and coherency requirements. Allocation flags must match context, and every allocation can fail. Bound buffers and avoid large allocations in hot or atomic paths.

Assumptions: Assume ordinary Linux allocation APIs; precise sizes and architecture constraints depend on the target.

Common incorrect answer: Passing a virtual pointer to a device or expecting GFP_ATOMIC to make unlimited memory available.

Follow-up: How would a scatter-gather-capable device change the buffer design?

Scoring out of 4: 1 point each: allocation distinction; DMA domain; context/failure; device capability.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q114 How do a mutex and a spinlock differ in the kernel?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Lock selection based on context and critical-section behavior.

Spoken answer: A mutex is a sleeping lock for eligible task contexts. A conventional spinlock protects short non-sleeping sections; PREEMPT_RT changes spinlock_t semantics, so configuration matters.

Deeper explanation: On a non-RT kernel, do not call sleeping functions while holding a spinlock. Shared state accessed from an interrupt may require interrupt-safe locking to avoid local deadlock, but the choice depends on all access paths. Raw spinlocks retain stricter behavior and are not a general optimization substitute. Document lock ownership, ordering, and which fields each lock protects.

Assumptions: Assume Linux 6.12 and explicitly distinguish non-RT from PREEMPT_RT behavior.

Common incorrect answer: Saying a spinlock is always faster or that disabling local interrupts serializes other CPUs.

Follow-up: Why might holding a spinlock across copy_to_user be invalid?

Scoring out of 4: 1 point each: sleeping distinction; context; RT qualification; per-state locking contract.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q115 What does a wait queue solve in a blocking character driver?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: State-based blocking and wakeup races.

Spoken answer: A wait queue lets a task sleep until a device-specific predicate becomes true, such as data available or device removed. The predicate and state synchronization remain the driver's responsibility.

Deeper explanation: A read path must handle interruption, nonblocking mode, removal, and competition with other readers. Wake after changing state so a woken task can observe progress. Recheck the condition because another reader may consume data first. Poll support must register the same wait relationship and report readiness from the same protected state. A wakeup is not a data item.

Assumptions: Assume the selected wait-event APIs and file-operation error conventions for Linux 6.12.

Common incorrect answer: Sleeping after a separate unprotected empty check or assuming each wake guarantees a readable byte.

Follow-up: What should a blocked read return if the device is disconnected?

Scoring out of 4: 1 point each: predicate; races; interruption/nonblocking; removal/poll consistency.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q116 Why does devm allocation not eliminate the need for a removal design?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Managed resource scope versus asynchronous lifetime.

Spoken answer: Managed resources simplify release when a device detaches or probe fails. They do not automatically stop work, timers, open-file users, or callbacks that still reference those resources.

Deeper explanation: Before detach completes, prevent new activity, quiesce hardware, and synchronize outstanding users using appropriate subsystem mechanisms. Registering cleanup actions in a deliberate order can help, but only if each action satisfies its context and lifetime contract. Open descriptors may outlive unbind, requiring separate state ownership or a disconnected state. A devm pointer is not a universal reference-counted object.

Assumptions: Assume Linux managed-device resources for the selected driver model.

Common incorrect answer: Removing manual cleanup blindly and allowing a workqueue callback to run after managed memory is released.

Follow-up: How would you handle a user process keeping a character device open during unbind?

Scoring out of 4: 1 point each: managed scope; asynchronous users; teardown order; open-file lifetime.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q117 What is the core idea of Read Copy Update?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Read-mostly synchronization and delayed reclamation.

Spoken answer: RCU lets readers access appropriately published objects while updates replace or unlink them. Old objects are reclaimed only after a suitable grace period ensures relevant preexisting readers are done.

Deeper explanation: RCU separates removal from reclamation and does not make arbitrary in-place updates safe. Writers still need a coordination strategy, and pointers must use the documented publish/dereference operations. The reader's allowed operations depend on the RCU flavor. A grace period is a lifetime guarantee for the specified readers, not a generic delay chosen in milliseconds.

Assumptions: Assume Linux 6.12 RCU APIs; SRCU and other flavors have distinct constraints.

Common incorrect answer: Calling sleep for a fixed time before freeing or claiming RCU removes every writer lock.

Follow-up: When would a reference count be needed in addition to an RCU read-side critical section?

Scoring out of 4: 1 point each: replace/unlink; grace-period reclamation; writer coordination; flavor/lifetime limits.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q118 How do workqueues differ from doing everything in an interrupt handler?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Deferred processing and cancellation.

Spoken answer: Workqueues run deferred callbacks in worker-thread context, allowing eligible sleeping operations. The interrupt path can acknowledge the event and schedule bounded follow-up work.

Deeper explanation: Coalescing behavior matters: queuing the same work item is not necessarily a count of every hardware event, so retain event data or counters separately. Protect shared state and choose ordering when multiple devices or events interact. During removal, prevent new queuing and use the correct cancellation or flushing protocol before releasing context. Do not hold a lock needed by the worker while waiting for it.

Assumptions: Assume ordinary workqueues in the selected Linux release; specialized queues have additional properties.

Common incorrect answer: Expecting one callback invocation for every attempted queue operation without checking the API semantics.

Follow-up: How would you preserve ten rapid events when one work item is already pending?

Scoring out of 4: 1 point each: worker context; event accounting; shared state; teardown/lock ordering.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q119 What is a kernel oops, and how should its evidence be preserved?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Crash interpretation without inventing a root cause.

Spoken answer: An oops reports a serious kernel fault with context such as instruction location and stack trace; it may or may not escalate to panic under configuration. Preserve the complete report and matching build identity.

Deeper explanation: Record kernel version, configuration, taint state, loaded modules, symbols, and reproduction steps. Decode addresses against the exact binary and inspect the first fault, because later failures may be consequences. A null-looking fault address suggests a class of access but does not prove the original source-level cause. Use a disposable VM for intentional fault experiments.

Assumptions: Assume access to matching unstripped symbols and a logging path that survives the failure where possible.

Common incorrect answer: Decoding against a different build or treating one stack frame as definitive proof of blame.

Follow-up: How would you distinguish a use-after-free from a simple null dereference using additional evidence?

Scoring out of 4: 1 point each: report preservation; exact symbols; first-fault reasoning; qualified hypothesis.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

#### Q120 Why must an out-of-tree module match the target kernel build?

Track: Kernel BSP | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Internal API and configuration compatibility.

Spoken answer: Kernel internal interfaces and structure layouts are not a stable external ABI. Build the module against the exact target kernel's prepared build configuration and exported interfaces.

Deeper explanation: Version strings alone may be insufficient when configuration, symbol versions, compiler assumptions, or vendor patches differ. Inspect module metadata and load errors; do not force-load incompatible modules to make a demonstration pass. A module building successfully proves compiler acceptance for that build, not correct runtime behavior or safe teardown. Keep source, config, build logs, and runtime evidence together.

Assumptions: Assume Linux loadable modules; distribution signing and policy may impose additional requirements.

Common incorrect answer: Copying a random headers directory with the same major version or treating force-load as a compatibility fix.

Follow-up: What evidence would you collect for an unknown-symbol error at insertion time?

Scoring out of 4: 1 point each: unstable internals; exact build; load diagnostics; build/runtime distinction.

Learning connection: Module M09. Verification references: LOCKS, DEVRES, RCU, WORKQUEUE, DMA, KERNEL.

### Driver review scenarios

#### Q121 A platform driver's probe never runs. What matching evidence do you inspect?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Enumeration, binding, and build configuration.

Spoken answer: Check that the device exists, the driver is built or loaded, the compatible or ID table matches, and the bus can bind them. Inspect the live description and logs.

Deeper explanation: A correct source DTS is not enough if a different DTB was deployed. A module may need a device table for alias-based autoloading. A disabled node, missing parent bus, absent Kconfig selection, or earlier resource dependency can prevent expected behavior. Distinguish no matching attempt from probe returning an error by adding appropriate diagnostic evidence rather than unconditional noisy prints.

Assumptions: Assume a device-tree-backed platform device and the selected Linux driver model.

Common incorrect answer: Renaming probe repeatedly or assuming source-code presence means the module is in the running image.

Follow-up: How would you verify the compatible string in the live device tree?

Scoring out of 4: 1 point each: enumeration; build/load; match table; deployed/live evidence.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q122 Probe returns EPROBE_DEFER repeatedly. Is this always a bug in the driver?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Supplier dependencies and correct deferral.

Spoken answer: Not necessarily. A required supplier such as a regulator, clock, GPIO controller, or interrupt domain may not yet be available. Trace the unresolved dependency and its eventual availability.

Deeper explanation: Deferral is useful when a supplier will appear later, but a misspelled reference or absent provider can make it permanent. Use framework error helpers and deferred-probe diagnostics appropriate to the pinned kernel. Do not convert every acquisition error into deferral; a malformed property or unsupported device should remain distinguishable. Confirm that successful reprobe does not leak resources from prior attempts.

Assumptions: Assume APIs report deferral through documented error codes and resource-acquisition patterns.

Common incorrect answer: Replacing EPROBE_DEFER with success while leaving essential resources uninitialized.

Follow-up: What happens when the supplier's driver is built as a module but never packaged in the image?

Scoring out of 4: 1 point each: supplier cause; permanent versus temporary; error distinctions; retry cleanup.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q123 A read method directly dereferences the user's buffer pointer. What is wrong?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: User/kernel trust boundary and fault handling.

Spoken answer: A user pointer must be accessed through the appropriate user-copy APIs, with validated sizes and error handling. Direct dereference can fault or violate the kernel's access rules.

Deeper explanation: copy_to_user and copy_from_user report bytes not copied rather than a conventional negative errno. Define how partial transfer affects device state and the read return value. Copying can fault and sleep in ordinary contexts, so do not place it blindly under a spinlock. Validate lengths and initialize any returned padding to avoid disclosing uninitialized kernel data.

Assumptions: Assume Linux 6.12 user-copy interfaces and a normal character-device file operation.

Common incorrect answer: Treating copy_to_user's nonzero return as a successful byte count or ignoring partial failure.

Follow-up: How would you avoid consuming an entire FIFO item when copying it to userspace fails partway?

Scoring out of 4: 1 point each: user-copy boundary; return semantics; context; state/initialization policy.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q124 A blocking read never wakes when a writer adds data. Where can the bug be?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Consistency of wait predicates and notification.

Spoken answer: Check that writer and reader use the same device instance, protected state, and wait queue, and that the writer updates the predicate before waking relevant waiters.

Deeper explanation: The reader must recheck after wake and handle interruption or removal. A predicate based on an unsynchronized cached count can remain false even though data exists. Poll and read should agree on what available data means. Use a deterministic test with a reader already blocked, then one write, and verify bounded completion without arbitrary polling sleeps.

Assumptions: Assume a stream-like character device with blocking and nonblocking contracts defined.

Common incorrect answer: Adding wakeups everywhere without identifying the predicate's owner and synchronization.

Follow-up: How should two readers behave when only one byte becomes available?

Scoring out of 4: 1 point each: shared instance/predicate; update-before-wake; competition; deterministic test.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q125 Poll reports readable but the next read blocks. Is the driver necessarily wrong?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Readiness races and API consistency.

Spoken answer: Not necessarily: another consumer may have consumed the data. Readiness is not a reservation. A nonblocking read should then return EAGAIN under the documented contract.

Deeper explanation: With only one consumer and no intervening state change, persistent disagreement suggests the driver's readiness predicate is wrong. Its poll implementation should register the wait queue and inspect the same state as read under compatible synchronization. Include removal and error readiness where appropriate. Test both single-consumer consistency and multi-consumer races rather than assuming one observation proves a defect.

Assumptions: Assume Linux poll semantics and a descriptor that supports nonblocking operation.

Common incorrect answer: Guaranteeing that readiness reserves bytes for the caller or returning readable unconditionally.

Follow-up: What ordering avoids missing a state change between checking readiness and registering a waiter?

Scoring out of 4: 1 point each: race distinction; nonblocking outcome; shared predicate; wait registration.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q126 A driver leaks memory whenever the second initialization step fails. How do you repair it?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Error-path ownership and acquisition order.

Spoken answer: List resources in acquisition order and unwind every successfully acquired resource in reverse dependency order. Use managed resources where their lifetime matches, without mixing incompatible cleanup models.

Deeper explanation: A failure test should inject an error after each acquisition, not only allocation failure at the first line. Set pointers or state so cleanup cannot double-release resources. If an interface becomes externally visible before initialization completes, a concurrent user can observe half-initialized state; delay publication until the object is ready. Review module init and per-device probe separately.

Assumptions: Assume each acquisition's release contract is known for the selected kernel APIs.

Common incorrect answer: Jumping to one generic cleanup label that releases resources never acquired.

Follow-up: Why might unregistering a visible interface need synchronization before freeing its backing object?

Scoring out of 4: 1 point each: resource inventory; reverse unwind; publish-last; failure injection.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q127 A driver's ioctl works for 64-bit applications but fails for 32-bit userspace. What do you inspect?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Stable UAPI layout and compatibility handling.

Spoken answer: Inspect argument structure sizes, alignment, pointer and long fields, command encoding, and the compat ioctl path. Use fixed-width UAPI types and explicit layout rules.

Deeper explanation: A structure containing a 32-bit field followed by a 64-bit field can have different padding across ABIs. Avoid native pointers and long in portable structures; validate reserved fields and initialize returned storage. Compatibility helpers apply only when the argument representation actually matches their assumptions. Preserve old command semantics when extending the interface, or add a new versioned command.

Assumptions: Assume a 64-bit kernel supporting 32-bit compatibility and Linux UAPI conventions.

Common incorrect answer: Adding a cast in the handler while leaving the userspace layout different.

Follow-up: How would you verify structure offsets in both a 32-bit and 64-bit test build?

Scoring out of 4: 1 point each: layout; fixed types; compat handler; ABI test/version strategy.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q128 A GPIO interrupt handler triggers continuously after the first event. What evidence matters?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Interrupt polarity, triggering, and source acknowledgment.

Spoken answer: Check electrical level, configured trigger type, polarity, and whether the device's interrupt condition is actually cleared. A level source stays asserted until its cause is removed.

Deeper explanation: Use the schematic and datasheet, inspect interrupt counters, and capture the line with suitable hardware tools when available. If clearing the condition requires a sleeping I2C transaction, a threaded handler may be appropriate. Masking the interrupt permanently hides the issue and loses future events. Contact bounce is a separate possibility with mechanical inputs and should be measured rather than assumed.

Assumptions: Assume the selected board's voltage and wiring are already verified and the interrupt source is known.

Common incorrect answer: Changing edge polarity randomly or adding long busy waits in a hard IRQ handler.

Follow-up: How do you prevent repeated handling while a threaded handler services a level-triggered source?

Scoring out of 4: 1 point each: trigger/electrical; acknowledgment; context; measured test.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q129 A device is removed while a process is blocked in read. What behavior must the driver define?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Disconnect semantics and object lifetime beyond unbind.

Spoken answer: Mark the device disconnected, prevent new hardware operations, wake blocked users, and return the documented EOF or error behavior. Keep backing state alive until existing references are released.

Deeper explanation: Unregistering a device node prevents new opens but does not necessarily eliminate existing file descriptions. A per-open or reference-counted state can distinguish dead hardware from still-valid bookkeeping. Stop interrupts, timers, and work before their resources vanish. Test removal during blocked read, active write, poll, and close, including repeated cycles in a disposable VM.

Assumptions: Assume a removable or unbindable device and a documented userspace ABI.

Common incorrect answer: Freeing all state in remove because the node no longer appears under /dev.

Follow-up: How would you make close safe after the underlying hardware has disappeared?

Scoring out of 4: 1 point each: disconnected state; wake behavior; references; teardown tests.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

#### Q130 A custom character driver duplicates a standard sensor interface. When should it be redesigned?

Track: Kernel BSP | Difficulty: Intermediate | Round: Driver review | Tags: debugging;scenario

Assesses: Subsystem selection and long-term interface cost.

Spoken answer: If the device fits an established subsystem such as IIO, input, hwmon, tty, or networking, use that subsystem's model unless a documented requirement cannot be represented.

Deeper explanation: A custom device node may be useful for teaching read/write and poll, but production integration benefits from established units, discovery, buffering, power management, and userspace tools. Evaluate the actual device class and required data rate before choosing. Replacing a public ABI later can be expensive, so document educational shortcuts and a migration path rather than presenting every toy driver as deployment-ready.

Assumptions: Assume the subsystem supports the device's essential semantics in the pinned kernel.

Common incorrect answer: Choosing a custom ioctl for every property because a character driver is familiar.

Follow-up: Which sensor features would you map to IIO channels, events, and buffered acquisition?

Scoring out of 4: 1 point each: subsystem fit; existing conventions; ABI cost; justified exception.

Learning connection: Module M10. Verification references: DEVRES, IOCTL, KERNEL, GPIO, IIO.

### Buses sensors DMA and power

#### Q131 Compare UART, I2C, and SPI from an integration perspective.

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Bus semantics and electrical/software responsibilities.

Spoken answer: UART is asynchronous serial communication, I2C is addressed shared-bus communication, and SPI commonly uses synchronous clocked transfers with chip selection. Each has distinct wiring, timing, and framing responsibilities.

Deeper explanation: Do not infer voltage compatibility from protocol names. UART needs baud and framing agreement; I2C needs suitable pull-ups and address handling; SPI needs clock mode, chip-select behavior, and device-specific transaction structure. Throughput and reliability depend on the selected controllers, board layout, and protocol overhead. Linux bus drivers and MCU peripheral drivers expose different APIs despite sharing electrical protocols.

Assumptions: Assume specific board schematics and component datasheets are available before hardware wiring.

Common incorrect answer: Calling I2C universally slower or SPI universally reliable without considering the actual configuration.

Follow-up: Which measurements would you collect when a peripheral occasionally returns all 0xff bytes?

Scoring out of 4: 1 point each: semantics; electrical compatibility; configuration; evidence-led diagnosis.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q132 What does an I2C register read typically require, and what must be checked?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Transactions, repeated starts, and device-specific behavior.

Spoken answer: Many devices require writing a register address then reading data, sometimes within one combined transaction with a repeated start. The datasheet determines the exact sequence.

Deeper explanation: Check adapter functionality, address width, register width, endianness, auto-increment, and transfer return values. An API returning fewer messages than requested is not complete success. Some devices have read-to-clear registers or require conversion delays, so a generic byte read is insufficient. Avoid using bus probing tools indiscriminately on devices whose writes or reads have side effects.

Assumptions: Assume the chosen sensor's transaction protocol, not a universal I2C register model.

Common incorrect answer: Using two unrelated transfers when the device requires a combined transaction or ignoring short-transfer results.

Follow-up: How would you handle an address conflict between two identical fixed-address sensors?

Scoring out of 4: 1 point each: sequence; adapter/device capabilities; result checking; side effects.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q133 What SPI parameters must agree between controller configuration and peripheral expectations?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Clocking, framing, and transaction boundaries.

Spoken answer: Clock polarity and phase, maximum frequency, word width, bit order where configurable, chip-select polarity, and chip-select timing must match the device's requirements.

Deeper explanation: A command and its response may need chip select held across multiple transfer segments. Some devices insert dummy cycles or return data one transaction later. Verify the controller actually supports the requested pattern. Use a logic analyzer to compare clock, select, and data to the datasheet, and keep a software loopback test separate from proof that the real peripheral interprets commands correctly.

Assumptions: Assume a specific peripheral and controller; supported transfer features vary.

Common incorrect answer: Trying all modes until output looks plausible without verifying timing or device identity.

Follow-up: How would you detect a one-bit sampling shift in a known device-ID response?

Scoring out of 4: 1 point each: mode; frequency/width; transaction CS; waveform evidence.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q134 Why does IIO expose raw values and scale rather than only a floating-point value?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Measurement representation and subsystem conventions.

Spoken answer: IIO commonly separates raw readings and conversion attributes so userspace can interpret samples consistently without requiring floating-point work in the kernel.

Deeper explanation: The conversion depends on the channel's documented scale, offset, units, and representation. A raw ADC code is not automatically volts or degrees. Buffered scans also define channel ordering, signedness, storage width, real bit width, shift, and endianness. Read the channel metadata and test against known input points; a plausible-looking number can still have the wrong scale or sign extension.

Assumptions: Assume Linux IIO and the selected device's documented channel attributes.

Common incorrect answer: Hard-coding one sensor's scaling formula for every IIO device or ignoring scan layout metadata.

Follow-up: How would you validate a signed 12-bit sample stored in a 16-bit scan element?

Scoring out of 4: 1 point each: raw/scale; units/offset; buffer layout; calibration test.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q135 How do direct reads and buffered acquisition differ for a sensor driver?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Sampling architecture and throughput requirements.

Spoken answer: Direct reads suit occasional measurements; buffered acquisition collects sequences using triggers or hardware FIFOs with defined timestamps and scan layouts. Choose based on rate and timing needs.

Deeper explanation: A high-rate sensor can lose samples if userspace repeatedly requests one value with uncontrolled scheduling delays. A buffer needs capacity, overrun policy, trigger lifecycle, and timestamp interpretation. Hardware timestamps and host receipt timestamps mean different things. Switching between modes may require mutual exclusion or configuration restrictions enforced by the subsystem. Test overrun and start-stop transitions, not just steady-state samples.

Assumptions: Assume the sensor and IIO driver support the chosen acquisition mode.

Common incorrect answer: Calling a polling loop a precise sampling system because its average sleep interval is correct.

Follow-up: Where should a timestamp be taken if acquisition time matters more than delivery time?

Scoring out of 4: 1 point each: mode tradeoff; buffering/overrun; timestamps; transition constraints.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q136 What problem does regmap solve, and what does it not know automatically?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Register-access abstraction and device-specific semantics.

Spoken answer: Regmap can centralize register formatting, bus access, caching, and common update patterns. It still needs accurate configuration of register widths, readable/writable ranges, volatility, and side effects.

Deeper explanation: A cached status register can be wrong if the device changes it asynchronously. Write-one-to-clear or read-to-clear registers need careful treatment; a generic update-bits operation may not be appropriate. Power loss can invalidate hardware state and require cache synchronization in a defined sequence. Use the device datasheet to classify registers, then test both ordinary access and suspend/resume behavior.

Assumptions: Assume a Linux regmap-backed device on a supported bus.

Common incorrect answer: Enabling caching for every register or assuming regmap removes the need to understand register behavior.

Follow-up: How would you handle a register whose contents change without software writing it?

Scoring out of 4: 1 point each: abstraction; configuration; side effects; cache/power coherence.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q137 Why is a DMA address different from a CPU virtual or physical address?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Address domains and portable device programming.

Spoken answer: A DMA address is the address the device uses after the platform's mapping rules, potentially including an IOMMU. It is not generally a cast of the CPU pointer.

Deeper explanation: Use dma_addr_t for device-visible addresses and the DMA API for mapping and ownership transitions. Set the supported DMA mask and check relevant failures. A 32-bit device may not address every buffer the CPU can access. Scatter-gather mapping can merge or transform segments, so use returned mapping information rather than assuming the original list directly matches hardware descriptors.

Assumptions: Assume Linux DMA APIs and a device with documented addressing limits.

Common incorrect answer: Programming hardware with virt_to_phys as a universal substitute for DMA mapping.

Follow-up: How would you test a driver on a system with an IOMMU to expose hidden address assumptions?

Scoring out of 4: 1 point each: domains; DMA API/type; mask; mapped-segment semantics.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q138 What is the difference between coherent and streaming DMA mappings?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Ownership and cache-synchronization contracts.

Spoken answer: Coherent mappings support shared visibility under the architecture's DMA contract; streaming mappings support explicit transfer ownership and synchronization. Neither choice removes the need for correct device protocol ordering.

Deeper explanation: Descriptor rings often use coherent memory while data buffers may use streaming mappings. For streaming access, map or sync in the correct direction before handing ownership to the other side, and do not let CPU and device race over the same bytes. A coherent descriptor still needs fields published in the documented order before its valid flag or doorbell exposes it.

Assumptions: Assume Linux's documented mapping model and the target's DMA capabilities.

Common incorrect answer: Calling coherent memory automatically race-free or skipping ownership transitions because host tests pass.

Follow-up: What can happen if CPU cache lines overlap a DMA buffer and unrelated writable data?

Scoring out of 4: 1 point each: mapping distinction; ownership; direction/sync; ordering/cache-line risk.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q139 How do system suspend and runtime suspend differ?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Power-management scope and state coordination.

Spoken answer: System suspend coordinates a broader system sleep transition. Runtime suspend idles an individual device while the system remains active, subject to dependencies and usage references.

Deeper explanation: A driver must stop or account for active work, preserve needed state, manage wake sources, and restore hardware correctly. Runtime-PM reference imbalance can keep a device awake forever or let it suspend during access. Error paths are as important as success paths. Tests should cover repeated cycles, ongoing requests, supplier dependencies, and removal in each state.

Assumptions: Assume Linux PM frameworks and a device with documented power-state behavior.

Common incorrect answer: Treating both callbacks as identical boilerplate or disabling power management to hide every race.

Follow-up: Which information is lost when a sensor's power rail is removed rather than merely clock-gated?

Scoring out of 4: 1 point each: scope; usage/dependencies; state restoration; transition tests.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

#### Q140 What does SocketCAN provide beyond sending raw CAN-controller registers?

Track: Kernel BSP and Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Using the Linux networking abstraction for a fieldbus.

Spoken answer: SocketCAN presents CAN communication through Linux networking interfaces and sockets, enabling established filtering, interface management, and integration with network tooling.

Deeper explanation: Distinguish Classic CAN from CAN FD frame limits and configuration. A DBC describes application signal interpretation; it is not the bus transport itself. Bit timing, termination, transceiver state, bus-off recovery, and message freshness remain system concerns. A virtual CAN interface tests software paths without proving physical arbitration, error confinement, or electrical compatibility on the target bus.

Assumptions: Assume a supported Linux CAN interface and explicitly selected Classic CAN or CAN FD mode.

Common incorrect answer: Claiming virtual CAN success validates transceiver wiring or interpreting every frame payload as a universal signal layout.

Follow-up: How would you detect stale or missing application messages independently of CAN frame checks?

Scoring out of 4: 1 point each: networking API; transport/DBC; physical limits; freshness policy.

Learning connection: Module M11. Verification references: IIO, GPIO, DMA, PM, I2C, SPI, REGMAP, CAN.

### Bare metal firmware

#### Q141 What must startup code establish before calling main on a microcontroller?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Reset-state assumptions and C runtime initialization.

Spoken answer: Startup establishes the stack and vector handling as required by the architecture, initializes writable data, clears zero-initialized storage, and performs necessary runtime setup before main.

Deeper explanation: Initialized data often has a load image in flash and a run address in RAM, while .bss needs zeroing rather than stored payload bytes. Clock setup, constructors for C++, and platform hooks depend on the toolchain and startup design. Verify the linker symbols and memory map; copying the wrong region can corrupt memory before source-level debugging reaches main.

Assumptions: Assume a chosen Cortex-M-style example; reset behavior and vector details are architecture-specific.

Common incorrect answer: Assuming main is the first instruction after reset or that RAM automatically starts at zero.

Follow-up: How would you diagnose a fault before main using the reset handler and linker map?

Scoring out of 4: 1 point each: stack/vector; data copy; bss clear; toolchain/platform qualification.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q142 How does a linker script affect firmware correctness?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Memory placement and load/run relationships.

Spoken answer: It maps sections to physical memory regions, defines entry and boundary symbols, and reserves resources such as stack, heap, and retained data according to the platform.

Deeper explanation: A DMA buffer may need a particular accessible RAM region; a vector table may need alignment and placement rules. Code executing while flash is being programmed may require RAM placement on some MCUs. Linker overflow checks should reject oversized images rather than allowing silent overlap. Preserve the map file and verify that startup uses the same symbols and section assumptions.

Assumptions: Assume the selected linker syntax and MCU memory map, not a universal flash/RAM layout.

Common incorrect answer: Treating the script as cosmetic or moving sections without updating startup and peripheral-access assumptions.

Follow-up: What would happen if initialized data's load address and run address were confused?

Scoring out of 4: 1 point each: placement; boundary symbols; special-region needs; startup/map consistency.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q143 How should an interrupt service routine be structured?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Bounded urgent work and safe deferred processing.

Spoken answer: Identify and acknowledge the event as required, capture minimal necessary data, and notify deferred processing through an ISR-safe mechanism. Keep execution bounded and respect interrupt-priority rules.

Deeper explanation: Some peripherals require reading status before data or clearing specific flags in a defined order. A long printf, blocking wait, general allocation, or mutex acquisition can violate latency and API constraints. Preserve overrun/error information instead of discarding it. Measure ISR duration and nesting behavior on hardware; host unit tests can validate state logic but not real interrupt timing.

Assumptions: Assume the MCU reference manual and RTOS port define acknowledgment and allowed API behavior.

Common incorrect answer: Doing all parsing and storage inside the ISR because it seems fastest to access data there.

Follow-up: How would you handle a burst larger than the ISR-to-task queue capacity?

Scoring out of 4: 1 point each: acknowledge; bounded capture; ISR-safe notification; measured overload behavior.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q144 What is the difference between a hardware timer, a software timer, and a busy wait?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Timing mechanisms and interference.

Spoken answer: A hardware timer counts independently according to its clock; a software timer schedules callbacks through an OS or scheduler; a busy wait occupies the CPU while checking progress.

Deeper explanation: Hardware compare or capture can provide timing behavior unavailable from a task sleep. Software timer callbacks may share a service task, so blocking one callback can delay others. Busy waits may be appropriate for tiny documented hardware sequences but can disrupt scheduling and power usage. Clock source, prescalers, interrupt latency, and rollover must be included in the timing calculation.

Assumptions: Assume specific timer hardware and scheduler behavior are known before setting accuracy claims.

Common incorrect answer: Equating a requested delay with guaranteed execution at exactly that wall-clock instant.

Follow-up: Which mechanism would you use to measure pulse width, and where would software latency enter?

Scoring out of 4: 1 point each: mechanism distinction; service context; clock/rollover; suitable application.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q145 How do you choose PWM frequency and resolution?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Timer arithmetic and application constraints.

Spoken answer: Choose a timer clock, prescaler, and period that meet the load's frequency requirement while providing enough duty-cycle steps. Frequency and resolution trade off for a fixed timer clock.

Deeper explanation: For a simple up-counter, the common relation is timer clock divided by prescaler factor and period count, but exact register conventions vary. A period of N counts gives roughly N duty positions, subject to output mode. Motors, LEDs, and power converters have different constraints; a learning LED demo is not evidence that a motor power stage is safe or controlled correctly.

Assumptions: Assume a documented timer mode and electrical load; verify register off-by-one conventions.

Common incorrect answer: Changing compare values without considering frequency, polarity, dead time, or load requirements.

Follow-up: How would center-aligned mode change the frequency calculation and switching behavior?

Scoring out of 4: 1 point each: arithmetic; tradeoff; register convention; application constraints.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q146 What should be considered before trusting an ADC measurement?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: The analog path and digital representation.

Spoken answer: Check reference voltage, input range, source impedance, sampling time, resolution, calibration, noise, and conversion format. The numeric code alone is not the physical quantity.

Deeper explanation: A high-impedance source may need more acquisition time or buffering to charge the sampling capacitor. Multiplexed channels can interact if settling is inadequate. Convert using the device's documented transfer behavior and calibration data, then verify with known inputs across the relevant range. DMA can move samples efficiently but does not fix analog settling or reference instability.

Assumptions: Assume the selected MCU or external ADC datasheet and board analog circuit are available.

Common incorrect answer: Converting code to voltage with a nominal supply constant and claiming calibrated accuracy.

Follow-up: How would you separate reference drift from sensor noise in a repeatable experiment?

Scoring out of 4: 1 point each: analog conditions; transfer/calibration; known-input test; DMA limitation.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q147 How should a watchdog be integrated into a multitask firmware system?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Detecting loss of useful progress rather than merely CPU activity.

Spoken answer: Feed the watchdog only when required subsystems demonstrate timely health under a defined policy. A dedicated feeder that runs regardless of failures can mask a dead system.

Deeper explanation: Use task heartbeats or supervised state transitions with bounded deadlines, and distinguish transient overload from an unrecoverable fault. Record reset reason and enough diagnostic state for post-reset analysis. Windowed watchdogs may also reject feeds that are too early. Verify behavior during debugger attachment, firmware update, and low-power states, because hardware may handle each differently.

Assumptions: Assume the selected watchdog's clock, window, and reset-domain behavior are documented.

Common incorrect answer: Feeding unconditionally from a high-priority timer interrupt even when application tasks are stuck.

Follow-up: What failures cannot a software-supervised watchdog detect if all tasks keep sending stale heartbeats?

Scoring out of 4: 1 point each: useful progress; supervision; reset evidence; special-state behavior.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q148 What information should a Cortex-M fault handler preserve?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Postmortem evidence with minimal additional failure risk.

Spoken answer: Preserve the relevant fault-status registers, stacked execution context, active stack selection, reset cause, and build identity using a bounded method appropriate to the target.

Deeper explanation: Do not assume printf or the normal allocator is safe after a memory fault. Validate stack addresses before dereferencing a possibly corrupted frame. A fault record can be stored in retained RAM or another target-specific mechanism, then reported after reboot. Exact register availability varies by Cortex-M generation; distinguish hard fault escalation from configurable fault sources using the reference manual.

Assumptions: Assume a selected Cortex-M part and verified retained-storage mechanism; no universal register set is promised.

Common incorrect answer: Blindly dumping a corrupted stack through an unbounded serial print loop.

Follow-up: How would you ensure a fault record corresponds to the firmware binary used for symbolication?

Scoring out of 4: 1 point each: context/status; bounded capture; stack validation; build matching.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q149 How should flash writes be designed when erase granularity exceeds record size?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Persistence under erase/program restrictions.

Spoken answer: Respect erase-block and program-unit rules, write new records with version and integrity metadata, and choose the newest valid committed record after restart.

Deeper explanation: A log or dual-record scheme can preserve the previous valid value until the new one is complete. Power can fail between data and commit marker, so recovery must reject incomplete records. Flash endurance requires write-rate budgeting and wear distribution. CRC can identify accidental corruption but does not authenticate a malicious change. Test interrupted operations on the actual device after modeling the state machine.

Assumptions: Assume the MCU datasheet specifies allowed programming sequences and endurance limits.

Common incorrect answer: Overwriting a few bytes in place as though flash were RAM or claiming a checksum prevents torn writes.

Follow-up: How would you recover when both records are invalid, and what configuration is safe to use?

Scoring out of 4: 1 point each: granularity; commit/recovery; endurance; fault tests.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

#### Q150 What belongs in a firmware bootloader versus the application?

Track: MCU Firmware | Difficulty: Practical | Round: Technical interview | Tags: conceptual

Assesses: Update and recovery boundaries.

Spoken answer: A bootloader establishes a trusted or validated launch decision and a recovery/update path according to the product's design. The application provides normal device behavior and health confirmation.

Deeper explanation: Keep image format, compatibility, signature verification where used, version policy, and failure recovery explicit. An application crash must not necessarily destroy the ability to recover. Bootloader update itself has different risk because it can affect the recovery mechanism. Partition layout, key storage, and reset behavior are target-specific; a host simulation validates decision logic without proving secure boot or power-failure behavior.

Assumptions: Assume an identified bootloader framework or carefully scoped learning implementation.

Common incorrect answer: Treating checksum verification as signature authentication or overwriting the sole recovery image without a recovery plan.

Follow-up: How would the bootloader decide that a newly installed image failed health confirmation?

Scoring out of 4: 1 point each: launch/recovery; image policy; bootloader-update risk; validation boundary.

Learning connection: Module M12. Verification references: ARM, FREERTOS, MCUBOOT.

### RTOS scheduling and communication

#### Q151 How do task priority and execution time interact in a preemptive RTOS?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Schedulability rather than priority folklore.

Spoken answer: Priority determines which ready task runs under the selected policy; execution time and blocking determine whether deadlines can actually be met. High priority does not create additional CPU capacity.

Deeper explanation: Budget worst-case execution, release rates, interrupt load, and shared-resource blocking. A task that never blocks can starve lower priorities under fixed-priority scheduling. Equal-priority time slicing depends on configuration. Assign priorities from timing and dependency requirements, then measure representative behavior and identify where formal analysis or stronger bounds are still needed.

Assumptions: Assume a stated FreeRTOS scheduling configuration; SMP and cooperative modes need additional analysis.

Common incorrect answer: Giving every task the maximum priority or using observed average execution time as a worst-case guarantee.

Follow-up: How would you choose priorities for acquisition, control, logging, and background diagnostics?

Scoring out of 4: 1 point each: policy; workload budget; blocking/starvation; evidence limits.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q152 When should you use a queue rather than a task notification?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Communication semantics and ownership.

Spoken answer: Use a queue when distinct data items or multiple producers need buffered transfer. A task notification can be a lightweight signal or count for a specific task under its documented semantics.

Deeper explanation: Notifications are not a universal replacement for message storage: coalescing, overwrite, and counting modes mean different things. If an ISR signals that DMA data is ready, ownership of the buffer must remain valid until the consumer finishes. Queue item copying versus pointer transfer changes lifetime responsibilities. Define what happens when the receiver is late or a queue is full.

Assumptions: Assume the selected FreeRTOS release and configured notification-index support.

Common incorrect answer: Sending pointers to temporary buffers or assuming every notification preserves a separate payload.

Follow-up: How would you transfer a DMA buffer between producer and consumer without copying its contents?

Scoring out of 4: 1 point each: data/signal distinction; notification semantics; ownership; overload.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q153 Why can a mutex be unsuitable in an interrupt handler?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Ownership, blocking, and priority inheritance context.

Spoken answer: An interrupt cannot block waiting for a task-owned resource, and ordinary mutex ownership/priority-inheritance semantics are designed for tasks. Use an appropriate ISR-safe signaling or data-transfer mechanism.

Deeper explanation: In FreeRTOS, choose documented FromISR APIs and comply with the port's interrupt-priority restrictions. A binary semaphore or notification can wake a task, but does not by itself protect shared multi-field data. If the ISR and task share a buffer, define a bounded critical section or ownership handoff. Never assume an API is ISR-safe merely because it usually returns immediately.

Assumptions: Assume the FreeRTOS port documentation for the selected MCU and priority configuration.

Common incorrect answer: Calling a normal mutex API with zero timeout from an ISR and assuming that makes it valid.

Follow-up: What does a higher-priority-task-woken flag mean for the return-from-interrupt path?

Scoring out of 4: 1 point each: no blocking/ownership; FromISR; priority restrictions; data ownership.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q154 What are the limits of FreeRTOS priority inheritance?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Avoiding overclaims about a simplified mechanism.

Spoken answer: FreeRTOS mutexes provide a simplified inheritance mechanism to reduce inversion. It is not a complete solution to arbitrary nested-lock timing, deadlocks, or long critical sections.

Deeper explanation: The behavior while a task holds multiple mutexes depends on the kernel's implementation and release. Keep lock graphs simple, bound hold times, and avoid slow I/O under locks. A binary semaphore used for signaling does not provide the same ownership-based inheritance semantics as a mutex. Review the selected release's documentation and test the actual nesting scenario relevant to the system.

Assumptions: Assume a pinned FreeRTOS kernel version and documented mutex configuration.

Common incorrect answer: Claiming every semaphore has priority inheritance or that inheritance guarantees a deadline.

Follow-up: How would a message-passing design remove a problematic shared peripheral lock?

Scoring out of 4: 1 point each: simplified PI; mutex/semaphore distinction; nested/long-hold limits; redesign.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q155 How do you size and validate RTOS task stacks?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Worst-case stack demand and meaningful evidence.

Spoken answer: Estimate call depth, local objects, interrupt/exception behavior, library use, and context overhead, then measure high-water behavior under demanding paths with margin and overflow detection.

Deeper explanation: A clean short run does not exercise every branch, recursion depth, error path, or printf formatting case. Stack units differ across APIs, so distinguish bytes from StackType_t elements. Interrupt stacks may be separate or share resources depending on the port. Large local arrays can be moved into explicit owned storage when justified, but shared static buffers introduce concurrency issues.

Assumptions: Assume the chosen RTOS port documents stack units and exception-stack behavior.

Common incorrect answer: Treating one high-water measurement as a proof of a safe worst-case stack size.

Follow-up: Which rarely executed failure path could consume more stack than steady-state operation?

Scoring out of 4: 1 point each: estimate; units; stressful measurement; margin and coverage limits.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q156 How should a periodic RTOS task avoid cumulative drift?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Release-time scheduling and overrun policy.

Spoken answer: Use the RTOS's absolute or previous-wake-time periodic-delay facility where appropriate, and track overruns separately from the nominal period.

Deeper explanation: A delay after work includes the work duration in each cycle. A delay-until pattern targets successive release times but does not guarantee the task will actually run exactly then; higher-priority work and interrupts can delay it. Define how missed releases affect processing, especially for control or sampling. Tick resolution and tickless idle behavior influence what can be represented and measured.

Assumptions: Assume the selected FreeRTOS API/version and a valid tick-period conversion.

Common incorrect answer: Equating a one-tick delay with one millisecond on every configuration.

Follow-up: How would a task whose execution sometimes exceeds its period report and recover from overruns?

Scoring out of 4: 1 point each: periodic schedule; scheduler delay; tick conversion; overrun policy.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q157 What must a Linux to MCU protocol specify beyond a payload structure?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Interoperability and failure recovery.

Spoken answer: Specify framing, length bounds, byte order, version, message type, sequence or request ID, integrity check, timeouts, and recovery behavior. Define ownership and state transitions on both sides.

Deeper explanation: A command acknowledgment should distinguish received, accepted, and completed if those events matter. Duplicate requests need idempotency or a documented replay policy. A CRC detects accidental corruption but is not authentication. Test resets, truncation, corrupt length, stale commands, and incompatible versions. Keep transport-specific behavior separate so the same parser can be exercised through a PTY before hardware testing.

Assumptions: Assume a bounded point-to-point learning protocol with explicitly documented trust assumptions.

Common incorrect answer: Sending a packed C structure and calling it a complete protocol specification.

Follow-up: How would each peer recover when the other reboots and its sequence counter restarts?

Scoring out of 4: 1 point each: wire contract; command semantics; reset/replay; fault tests.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q158 How do you make a command timeout meaningful for a robot demonstrator?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Freshness, authority, and local control boundaries.

Spoken answer: The MCU should enforce a local deadline for motion commands and transition to a defined stopped or disabled state when commands become stale, independent of Linux remaining alive.

Deeper explanation: Use monotonic local timing, explicit arm/disarm state, bounded command values, and clear recovery rules. A network heartbeat is not proof that a current control command is safe. Test dropped packets, Linux stalls, repeated old frames, MCU reset, and reconnect. This establishes a learning demonstrator behavior, not a certified emergency-stop or functional-safety system.

Assumptions: Assume simulated motion first; physical hardware requires separate electrical and hazard controls.

Common incorrect answer: Continuing the last motor command indefinitely or calling a software timeout a certified safety function.

Follow-up: Should a recovered link automatically re-arm motion, or require an explicit operator action?

Scoring out of 4: 1 point each: MCU-local deadline; state transition; stale/replay tests; safety scope.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q159 What does deterministic execution mean in an RTOS discussion?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Bounded behavior and precise claims.

Spoken answer: State which quantity is bounded under which assumptions: execution time, interrupt latency, response time, or memory use. A repeatable average is not a deterministic worst-case guarantee.

Deeper explanation: Bounded loops, fixed-capacity storage, controlled blocking, and documented interrupt load make analysis possible. Cache effects, DMA contention, flash wait states, and other tasks can affect timing. A measurement campaign can establish observed distributions for its conditions; formal worst-case claims require stronger assumptions and analysis. Explain the required deadline and acceptable failure behavior before choosing an RTOS or priority scheme.

Assumptions: Assume a documented workload and hardware configuration; no universal zero-jitter guarantee exists.

Common incorrect answer: Saying an RTOS makes every task execute at its requested instant.

Follow-up: What evidence would you present for a 2 ms response requirement after only host simulation?

Scoring out of 4: 1 point each: precise quantity; assumptions; bounded design; measurement-versus-proof.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

#### Q160 How do you recover from a stalled I2C transaction in firmware without wedging every task?

Track: MCU Firmware | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual;scenario

Assesses: Timeout boundaries, peripheral recovery, and system isolation.

Spoken answer: Apply bounded waits, classify controller and bus state, and use the MCU and device's documented recovery sequence. Keep one failed peripheral from blocking unrelated work indefinitely.

Deeper explanation: Recovery may involve aborting DMA, resetting the peripheral, inspecting stuck lines, or a documented bus-clear sequence. Electrical faults and devices holding a line require hardware-aware diagnosis; blindly toggling pins can be inappropriate. After recovery, reset protocol state and reinitialize affected devices as needed. Count failures and enter degraded operation after a defined retry budget.

Assumptions: Assume a selected MCU and sensor with documented recovery behavior; no universal sequence is prescribed.

Common incorrect answer: Looping forever on a ready flag or retrying immediately without a bound or state reset.

Follow-up: What should happen if the bus remains physically stuck after the retry budget expires?

Scoring out of 4: 1 point each: bounded wait; documented recovery; isolation/degraded mode; observable retries.

Learning connection: Module M13. Verification references: FREERTOS, ARM, MCUBOOT.

### Firmware and protocol coding

#### Q161 Implement comparison of wrapping 16-bit sequence numbers.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Freshness semantics and half-range limits.

Spoken answer: A candidate is newer when the unsigned 16-bit difference from the previous value is nonzero and below 0x8000. Treat equality as duplicate and half-range distance as ambiguous.

Deeper explanation: seq_newer accepts 0 after 65535 and rejects 65535 after 0 under the bounded-distance contract. Sequence comparison alone cannot distinguish a restarted sender from a very old session, so include a session identifier or explicit handshake. Test equality, adjacent values, wraparound, and exactly half the range. Do not silently assign meaning to an excluded ambiguous case.

Assumptions: Assume no legitimate forward jump or observation gap reaches half the sequence space.

Common incorrect answer: Using ordinary greater-than or claiming modular comparison solves reboot identity.

Follow-up: How would you reject a replay from a previous boot after the sequence counter resets?

Scoring out of 4: 1 point each: difference rule; duplicate; half-range condition; session boundary.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q162 Encode payload bytes with delimiter 0x7e and escape 0x7d.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Framing expansion and bounded output.

Spoken answer: Emit a delimiter, escape each delimiter or escape byte by prefixing 0x7d and XORing the byte with 0x20, then emit a closing delimiter. Check worst-case output size first.

Deeper explanation: escape_frame needs at most twice the payload length plus two bytes and rejects insufficient capacity without writing. Payload 7e 7d 01 becomes 7e 7d 5e 7d 5d 01 7e. This framing exercise does not itself add length, CRC, authentication, or a timeout; those are separate protocol layers. The decoder must reject a dangling escape at a boundary under a defined policy.

Assumptions: Assume a nonoverlapping output buffer and the exact stated escaping convention.

Common incorrect answer: Allocating only payload length plus two or escaping only the delimiter but not the escape byte.

Follow-up: How would you recover when a stream ends immediately after an escape byte?

Scoring out of 4: 1 point each: encoding; expansion bound; transactional capacity failure; malformed recovery.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q163 Implement a watchdog supervisor for three required task heartbeats.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Progress windows and stale-health rejection.

Spoken answer: Track the last observed progress time for each required task. Report healthy only when every task has progressed within its configured timeout and initialization has completed.

Deeper explanation: supervisor_ok accepts injected monotonic timestamps and rejects a task that has never reported. Use unsigned wrap-aware elapsed arithmetic under a bounded interval contract. A heartbeat should advance only after useful work or an accepted idle-state check, not from an unrelated timer. Unit tests can validate time-window logic; hardware testing must still verify watchdog clocks and reset behavior.

Assumptions: Assume timestamps and limits stay within the supported modular interval and task progress reports are trustworthy.

Common incorrect answer: Feeding the watchdog because any one task is alive or treating an old stored heartbeat as current.

Follow-up: How would you supervise a task that legitimately performs a long firmware update operation?

Scoring out of 4: 1 point each: all-task policy; initialization; stale timeout; useful-progress qualification.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q164 Implement a robot command state machine with DISARMED ARMED and FAULT states.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Explicit authority and fault transitions.

Spoken answer: Accept motion only while armed, disarm on command timeout, and latch a fault on a defined fault event. Require an explicit clear and re-arm sequence rather than resuming automatically.

Deeper explanation: control_step models events without touching hardware. The test trace arms, accepts a command, times out to DISARMED, enters FAULT, rejects arm, clears to DISARMED, then arms again. Outputs include whether motion is permitted, so state and actuator policy cannot diverge silently. Integration must independently ensure the hardware output changes on each disabling transition.

Assumptions: Assume a learning demonstrator; the state machine is not a certified safety function.

Common incorrect answer: Automatically restoring motion when communications return or clearing a fault on any incoming packet.

Follow-up: How would you verify that an MCU reset starts with outputs disabled before application initialization?

Scoring out of 4: 1 point each: guarded motion; timeout; latched fault/re-arm; output integration boundary.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q165 Implement sensor validity classification using range and maximum age.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Separating measurement value from trust in that value.

Spoken answer: A sample is valid only if its status is good, value lies within the configured range, and its monotonic age is no greater than the allowed maximum.

Deeper explanation: sensor_valid uses explicit minimum, maximum, and age limit inputs. Test exact range boundaries, out-of-range values, stale timestamps, and an invalid status carrying an otherwise plausible number. Do not replace invalid data with zero without a validity marker, because zero may be a legitimate reading. The policy should tell downstream control or logging how to handle missing data.

Assumptions: Assume a bounded unsigned timestamp interval and configured range in the same units as the sample.

Common incorrect answer: Checking only the numeric range and ignoring that an old value can remain plausible indefinitely.

Follow-up: When would a rate-of-change plausibility check add useful evidence, and when could it reject real events?

Scoring out of 4: 1 point each: range; age; status; downstream missing-data contract.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q166 Implement bounded exponential retry delays with a maximum delay.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Recovery behavior without overflow or retry storms.

Spoken answer: Double the delay while below the cap, saturating before multiplication would overflow or exceed the cap. Track a retry budget separately from the delay calculation.

Deeper explanation: retry_delay returns min(initial times 2 to the attempt, cap) using guarded steps, not an unchecked shift. Test zero attempts, the cap transition, large attempts, and initial greater than cap. Deterministic tests omit jitter; a deployed multi-device system may add bounded randomization to avoid synchronized reconnect storms. Permanent configuration errors should not be retried endlessly as transient failures.

Assumptions: Assume positive cap and a documented zero-initial-delay policy.

Common incorrect answer: Using initial << attempt with an unchecked shift count or confusing capped delay with capped retry count.

Follow-up: How would you reset the retry state after a connection briefly succeeds then immediately fails?

Scoring out of 4: 1 point each: saturation; shift avoidance; budget distinction; recovery policy.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q167 Write a median-of-three filter for signed 32-bit samples without addition.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Small bounded filtering and overflow avoidance.

Spoken answer: Use comparisons and swaps to order three values, then return the middle. Avoid summing all three and subtracting extremes because the intermediate sum can overflow.

Deeper explanation: median3 is constant in storage and has a fixed small comparison bound. Test already sorted, reversed, equal, and extreme values including INT32_MIN and INT32_MAX. A median suppresses an isolated impulse among three samples but changes signal behavior and adds a window interpretation. It is not a substitute for checking sensor validity or measuring control-loop delay.

Assumptions: Assume signed 32-bit samples and a three-sample count-based window.

Common incorrect answer: Using arithmetic averaging as a median or ignoring overflow in a sum-based shortcut.

Follow-up: How would you define output during the first two samples of a streaming implementation?

Scoring out of 4: 1 point each: comparison algorithm; extremes; ties; filtering/startup limitations.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q168 Scale a signed sensor code by an integer ratio with checked output range.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Fixed-point arithmetic and explicit rounding.

Spoken answer: Promote the product to int64_t before multiplication, reject a zero denominator, divide under the stated rounding policy, and check the quotient fits int32_t before storing.

Deeper explanation: scale_i32 uses an int32_t numerator and a positive int32_t denominator; their product fits int64_t. C integer division truncates toward zero, so negative values need a different formula if floor rounding is required. Test negative input, fractional output, zero denominator, and an out-of-range result. Units and calibration constants belong in the interface contract.

Assumptions: Assume the specified operand ranges; extending to int64_t operands needs additional multiplication checks.

Common incorrect answer: Casting only after the 32-bit multiplication has already overflowed.

Follow-up: How would round-to-nearest be defined at exact half steps for negative values?

Scoring out of 4: 1 point each: promote-before-multiply; denominator; output bounds; rounding/units.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q169 Decode a signed 12-bit two's-complement value from a 16-bit container.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Bit-width-specific signed interpretation without accidental promotions.

Spoken answer: Mask to 12 bits, then if bit 11 is set subtract 4096 in a sufficiently wide signed type. Otherwise return the masked value.

Deeper explanation: decode_s12 yields 2047 for 0x07ff, -2048 for 0x0800, and -1 for 0x0fff. This avoids relying on arithmetic right shift of a negative value or implementation-defined out-of-range conversion. Container byte order and any bit shift must be handled before this function. Reserved upper bits may need validation rather than masking in a stricter protocol.

Assumptions: Assume the wire field is explicitly 12-bit two's complement, not a sign-magnitude or offset-binary ADC.

Common incorrect answer: Casting the whole 16-bit container to int16_t and expecting bit 11 to become the sign bit.

Follow-up: How would the code change for a left-aligned 12-bit field with four low padding bits?

Scoring out of 4: 1 point each: mask; sign threshold; subtraction; representation boundary tests.

Learning connection: Module M13. Verification references: C11, FREERTOS.

#### Q170 Implement a token bucket that limits diagnostic events per time window.

Track: MCU Firmware | Difficulty: Practical | Round: Coding round | Tags: coding

Assesses: Rate limiting with bounded arithmetic and observability.

Spoken answer: Store available tokens and last refill time, add whole refill intervals up to capacity, and consume one token only when available. Define whether partial intervals are retained.

Deeper explanation: bucket_take uses integer millisecond timestamps and retains the unconsumed time remainder by advancing last refill only by complete intervals. Saturate before adding a large refill count. Test initial burst capacity, rejection when empty, one refill, long idle time, and timestamp wrap under the documented interval limit. Count suppressed events so rate limiting does not hide repeated failures completely.

Assumptions: Assume single-threaded access and a nonzero refill interval; concurrency requires synchronization.

Common incorrect answer: Resetting the bucket to full on every call or overflowing tokens after a long idle period.

Follow-up: How would you reserve capacity for critical alarms separately from verbose diagnostics?

Scoring out of 4: 1 point each: token/refill invariant; retained remainder; saturation; suppressed-count policy.

Learning connection: Module M13. Verification references: C11, FREERTOS.

### Images updates and reliability

#### Q171 How would you choose between Buildroot and Yocto for a learning or product image?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Build-system scope and maintenance tradeoffs.

Spoken answer: Choose from required customization, package workflow, reusable metadata, team familiarity, supported BSPs, and maintenance needs. Buildroot and Yocto can both produce embedded Linux systems through different models.

Deeper explanation: A small appliance can make Buildroot's configuration and package model approachable. A product family may benefit from Yocto layers, recipes, SDK generation, and policy separation, at the cost of more concepts and build resources. Neither choice automatically proves reproducibility, security, or support for a board. Pin releases and external inputs, then demonstrate the actual image and update path.

Assumptions: Assume the exact target BSP and team requirements are evaluated; this is not an employer requirement claim.

Common incorrect answer: Calling one universally professional and the other only for toys without examining the use case.

Follow-up: Which evidence would justify switching build systems after an initial prototype?

Scoring out of 4: 1 point each: requirements; models; maintenance/resource tradeoff; validated target support.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q172 What do a Yocto layer, recipe, and image each represent?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Metadata organization and build artifacts.

Spoken answer: A layer groups related metadata; a recipe describes how to obtain, configure, build, install, and package a component; an image selects a filesystem composition and related outputs.

Deeper explanation: Keep product policy, board support, and application metadata organized so upgrades remain reviewable. A bbappend modifies a matching recipe without copying its entire implementation, but version matching must be intentional. BitBake task signatures and shared state affect rebuild behavior. Inspect the actual task logs and environment when a variable appears ineffective instead of changing random global configuration.

Assumptions: Assume the pinned Yocto 5.0-family documentation and matching layer branches.

Common incorrect answer: Putting every customization in local.conf or treating an image recipe as the implementation of every package.

Follow-up: How would you package one telemetry service in your own layer without editing upstream recipes?

Scoring out of 4: 1 point each: three roles; layer separation; append/version; task diagnostics.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q173 What belongs in a Buildroot external tree for a custom appliance?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Keeping product changes separate and reproducible.

Spoken answer: Store the product's board configuration, custom package definitions, overlays or post-build steps, and documented external-tree metadata so the build can be reproduced from versioned inputs.

Deeper explanation: Avoid making unrecorded changes directly inside an output root filesystem and calling that the product source. A package should install files through its build rules; an overlay can supply static configuration but should not hide architecture-specific binaries. Track defconfig, source revisions, hashes, and build logs. Test startup and persistence behavior in the generated image rather than only compiling the application on the host.

Assumptions: Assume a pinned Buildroot release and its documented BR2_EXTERNAL layout.

Common incorrect answer: Copying the entire output directory into version control as the sole recipe for rebuilding.

Follow-up: How would you prove a clean build includes your application without manual post-build copying?

Scoring out of 4: 1 point each: external organization; versioned inputs; package/overlay boundary; image test.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q174 What is the difference between a rebuildable and a bit-reproducible image?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Precise reproducibility claims.

Spoken answer: Rebuildable means documented inputs can produce a functioning artifact again. Bit-reproducible means independent builds with the specified inputs yield identical bytes under the defined conditions.

Deeper explanation: Timestamps, source paths, locale, toolchain revisions, archive ordering, and external downloads can change output. Pin inputs and compare hashes, then use structured binary comparison to investigate differences. Functional equivalence is useful but weaker than byte identity. Record which artifact was compared: a root filesystem, signed bundle, compressed image, and full disk image can have different nondeterminism sources.

Assumptions: Assume reproducibility is evaluated against an explicit input and environment specification.

Common incorrect answer: Claiming reproducibility because two builds both boot or because a Docker tag was reused.

Follow-up: How can signature timestamps or generated filesystem UUIDs complicate bitwise comparison?

Scoring out of 4: 1 point each: claim distinction; input control; artifact scope; nondeterminism diagnosis.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q175 What should an SBOM contribute to an embedded product workflow?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Dependency traceability and its limits.

Spoken answer: A software bill of materials records components and relevant versions, identities, and relationships so maintenance, licensing, and vulnerability review have a traceable starting point.

Deeper explanation: Generate it from the actual build where possible and retain it with artifact hashes and configuration. An SBOM is not a vulnerability scan result or proof that all vulnerabilities are exploitable. Version strings alone may miss backported fixes, so preserve source revisions and patch context. Link component findings to deployed products and update decisions rather than producing a file no one can interpret.

Assumptions: Assume the selected build system and SBOM format are documented and verified for the artifact.

Common incorrect answer: Treating a generated SBOM as proof of security or license compliance without review.

Follow-up: How would you determine whether a reported vulnerability affects a feature disabled in your image?

Scoring out of 4: 1 point each: inventory; build linkage; backport/config context; triage limits.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q176 How do signed updates differ from secure or verified boot?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Integrity boundaries across installation and startup.

Spoken answer: Signed update verification authenticates an update package before accepting it under the update policy. Verified boot checks the boot chain or loaded components according to a trust model at startup.

Deeper explanation: One does not automatically provide the other. An accepted image could later be modified on writable storage unless boot/runtime protections address that risk. Key storage, revocation, rollback policy, and recovery paths matter in both designs. CRCs and hashes without an authenticated trust anchor detect corruption but do not prove authorization. State exactly which components and transitions are covered.

Assumptions: Assume a chosen update and boot framework; hardware root-of-trust claims require target evidence.

Common incorrect answer: Calling a checksum-secured download secure boot or assuming HTTPS alone authorizes every image.

Follow-up: How would you rotate an update signing key without stranding deployed devices?

Scoring out of 4: 1 point each: two boundaries; trust anchor; uncovered modification; key/recovery plan.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q177 What must an A/B updater do before marking a new slot good?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Health confirmation and rollback semantics.

Spoken answer: Boot the candidate slot, run meaningful bounded health checks, and confirm only when required functions and persistent-state compatibility meet the defined policy. Retain rollback until that decision.

Deeper explanation: A process merely starting may not prove its sensor path, storage, or network configuration works. Use a boot-attempt counter or framework-supported status so repeated failed boots revert. The updater should not overwrite the currently known-good slot during installation. Shared application data needs a migration strategy compatible with rollback, or rollback can boot old software against unusable new data.

Assumptions: Assume the bootloader and updater share a validated slot-selection protocol.

Common incorrect answer: Marking good immediately at the start of boot or equating two partitions with guaranteed recovery.

Follow-up: What health tests should run locally when the network is unavailable?

Scoring out of 4: 1 point each: bounded meaningful health; attempt/rollback; inactive-slot write; data compatibility.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q178 How should a CI pipeline validate an embedded change at several levels?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Test layering and honest coverage.

Spoken answer: Run fast host unit and parser tests, static checks, target builds, emulator integration where supported, and hardware tests for behavior that depends on the real board.

Deeper explanation: A change touching an I2C driver needs more than a host lint pass; a pure parser change may not require every expensive hardware test on every edit. Preserve exact inputs, artifacts, logs, and failure seeds. Separate authored, compiled, host-tested, emulator-tested, and hardware-tested evidence. Flaky tests should be investigated, not rerun silently until green.

Assumptions: Assume available runners and targets are explicitly listed; missing hardware gates remain pending.

Common incorrect answer: Reporting full validation from compile success or treating an emulator as an electrical test bench.

Follow-up: Which smallest regression test would you add for a malformed length-field crash?

Scoring out of 4: 1 point each: layers; risk-based scope; evidence labels; meaningful regression.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q179 How do you investigate boot time without optimizing the wrong stage?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Measurement boundaries and causal experiments.

Spoken answer: Define start and ready endpoints, measure stage durations, and identify the dominant delay before changing configuration. Bootloader, kernel, device probing, and userspace readiness are separate intervals.

Deeper explanation: A serial console can itself add delay, and a service manager's active state may precede useful application readiness. Record power/reset conditions, storage state, and network availability. Change one cause at a time and compare multiple runs, including outliers. Removing a timeout can improve a benchmark while damaging recovery, so retain required failure behavior in the acceptance criteria.

Assumptions: Assume an observable hardware or emulator start event and a product-specific readiness definition.

Common incorrect answer: Using only kernel timestamps as total power-on-to-service-ready time.

Follow-up: How would you measure a service that must initialize a sensor before it is operational?

Scoring out of 4: 1 point each: endpoints; stage breakdown; controlled trials; recovery tradeoff.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

#### Q180 What makes a reliability test more useful than leaving a device running overnight?

Track: Embedded Linux and Kernel | Difficulty: Intermediate | Round: Technical interview | Tags: conceptual

Assesses: Targeted failure coverage and measurable acceptance.

Spoken answer: Define workload, faults, metrics, duration, and pass conditions, then preserve evidence. Long duration is useful only when it exercises relevant states and detects failure meaningfully.

Deeper explanation: A soak test can track memory, descriptor counts, reconnects, missed deadlines, and data loss. Add targeted disconnects, malformed packets, storage-full conditions, process restarts, and update interruptions according to scope. Record whether the system recovers and whether data integrity survives. A clean overnight run does not cover rare transitions that never occurred during it.

Assumptions: Assume a disposable test environment and explicit limits for disruptive fault injection.

Common incorrect answer: Calling lack of visible crashes proof of reliability while ignoring silent data loss or unbounded growth.

Follow-up: How would you make a storage-full failure test verify both diagnosis and recovery?

Scoring out of 4: 1 point each: specified workload; metrics; targeted faults; recovery/integrity criteria.

Learning connection: Module M14. Verification references: BUILDROOT, YOCTO, RAUC, REPRO, SBOM.

### System design exercises

#### Q181 Design a bounded industrial sensor gateway using one MCU and one Linux computer.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Requirements decomposition, interfaces, and failure containment.

Spoken answer: Keep acquisition and local timing on the MCU, transfer versioned framed telemetry over one primary link, and let Linux validate, persist, expose health, and manage remote integration.

Deeper explanation: Specify sampling rate, payload bounds, queue capacity, freshness deadlines, configuration ownership, and recovery after either side resets. Start with a PTY simulator and deterministic fault injection, then add the selected board path. Use a known-good configuration and observable degraded states. Acceptance should include corrupted frames, slow consumers, storage full, restart, and incompatible versions, with separate host and hardware evidence.

Assumptions: Assume a learning gateway with modest telemetry volume, not an unspecified safety-critical product.

Common incorrect answer: Adding multiple clouds and buses before proving the primary path and recovery behavior.

Follow-up: How would you demonstrate that disconnecting the network cannot block MCU acquisition indefinitely?

Scoring out of 4: 1 point each: boundary architecture; bounded interface; recovery; measurable acceptance.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q182 Design a Linux to MCU robot control demonstrator with stale-command protection.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Control authority, simulation, and explicit safety scope.

Spoken answer: Linux handles planning or user commands and diagnostics; the MCU handles bounded local control, command validation, freshness, and output disable. Begin with simulated actuators and an explicit arming state.

Deeper explanation: Define command units, ranges, sequence/session behavior, timeout, fault latch, and recovery. Separate telemetry from control authority so a live dashboard cannot keep an old command valid. Test Linux pause, dropped frames, replay, MCU reset, and reconnect. Physical deployment needs independent hazard analysis, electrical protection, and verified stopping behavior beyond this learning design. C02’s minimum simulation resumes on a fresh valid command after ordinary command expiry; a separate arming/session handshake is an extension. Its local stop latch requires deliberate rearming.

Assumptions: Assume differential-drive simulation as a teaching target; no certified safety claim is made.

Common incorrect answer: Putting the only motion timeout in a Linux UI, or resuming motion without an explicit, justified recovery policy or while a fault remains latched.

Follow-up: What evidence would justify moving from simulation to wheels-off-ground hardware testing?

Scoring out of 4: 1 point each: responsibilities; local freshness; fault tests; staged hardware boundary.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q183 Design a character-device interface for a bounded event queue.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: ABI semantics, concurrency, and removal.

Spoken answer: Define record format, maximum queue size, read semantics, nonblocking behavior, poll readiness, overflow policy, and disconnection behavior before writing file operations.

Deeper explanation: Choose whether reads return whole records or byte streams and how too-small user buffers behave. Synchronize producers and readers, avoid sleeping under incompatible locks, and preserve state on failed user copies according to the contract. Existing opens must remain safe after device removal. Acceptance includes concurrent readers, full queue, signals, invalid user buffers, blocked read during removal, and repeated bind/unbind.

Assumptions: Assume an educational device; use an existing subsystem instead if it fits the real hardware.

Common incorrect answer: Returning inconsistent readiness and read behavior or freeing state when the node is unregistered.

Follow-up: How would userspace detect that records were dropped due to queue overflow?

Scoring out of 4: 1 point each: precise ABI; synchronization; lifetime; adversarial acceptance.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q184 Design a sensor acquisition path using Linux IIO and a user-space logger.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Subsystem reuse, units, timestamps, and data integrity.

Spoken answer: Expose the sensor through IIO channels and the suitable direct or buffered interface, then let a bounded userspace service decode scan metadata, validate samples, and persist records.

Deeper explanation: Specify rate, timestamp meaning, scaling, channel order, and overrun behavior. Keep hardware reset and register recovery in the driver while storage policy remains in userspace. Test known sensor inputs, bus errors, suspend/resume, buffer overrun, logger restart, and storage failure. A synthetic IIO or parser test validates software interpretation but does not establish analog accuracy or real interrupt timing.

Assumptions: Assume the selected sensor supports the required acquisition mode and a documented datasheet.

Common incorrect answer: Inventing a private ioctl for every channel or hard-coding raw sample bytes without metadata.

Follow-up: How would you correlate MCU and Linux timestamps if measurements come from both domains?

Scoring out of 4: 1 point each: subsystem mapping; representation/time; failure boundaries; layered validation.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q185 Design a signed A/B update workflow for a small appliance.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Trust, persistent state, and recovery transitions.

Spoken answer: Verify an authorized bundle, write only the inactive slot, update boot-selection metadata through the supported mechanism, boot the candidate, and confirm it after bounded health checks.

Deeper explanation: Define failure handling before download, during writing, during metadata changes, and after first boot. Preserve a recovery image or path and plan signing-key rotation and rollback policy. Application data migrations must not make the previous slot unusable without an explicit strategy. Test interruption points in disposable images and later on actual bootloader/storage hardware, recording each transition outcome.

Assumptions: Assume a selected framework such as RAUC with a compatible bootloader integration; host simulation is not deployment proof.

Common incorrect answer: Calling two copied directories a complete secure updater or marking good before the application works.

Follow-up: How would you prevent installation of an image for the wrong board revision?

Scoring out of 4: 1 point each: signed inactive-slot flow; boot/health; compatibility/data; interruption matrix.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q186 Design a reliable serial protocol for commands and telemetry with 128-byte payloads.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Complete protocol specification and resynchronization.

Spoken answer: Choose a bounded frame with version, type, length, session/request identity, payload, and CRC, plus escaping or another unambiguous framing method and explicit deadlines.

Deeper explanation: Specify byte order, CRC parameters, maximum encoded length, duplicate handling, and which messages require acknowledgment. A command should distinguish accepted from completed if execution is asynchronous. Recovery must handle truncation, bad length, bad CRC, unknown version, and peer restart without unbounded buffering. Provide fixed vectors and tests at every fragmentation boundary; electrical validation remains separate.

Assumptions: Assume an accidental-error model unless authentication is explicitly added; CRC is not a security mechanism.

Common incorrect answer: Using a magic prefix alone with no escape, length, or bounded resynchronization policy.

Follow-up: How would a firmware update preserve compatibility with an older Linux peer?

Scoring out of 4: 1 point each: full wire contract; semantics; bounded recovery; vectors/compatibility.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q187 Design a real-time acquisition task plus best-effort logging under overload.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Scheduling and queue policy under conflicting priorities.

Spoken answer: Keep acquisition bounded and independent of slow storage. Transfer samples through a fixed-capacity queue with a stated drop or backpressure policy and expose overrun counters.

Deeper explanation: Prioritize from deadlines and execution budgets, not component importance alone. Avoid storage calls or unbounded allocation in the acquisition path. If losing any sample is unacceptable, prove the required throughput and buffering under the specified outage duration or reduce acquisition rate by policy. Test sustained overload, not only bursts, and distinguish measured worst observed latency from a guaranteed bound.

Assumptions: Assume requirements state which data loss is acceptable and what local control must continue.

Common incorrect answer: Using an unbounded queue or blocking the acquisition ISR until a flash write finishes.

Follow-up: How much storage is required to survive a 30-second downstream outage at the stated sample rate?

Scoring out of 4: 1 point each: timing boundary; capacity arithmetic; overload policy; evidence limits.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q188 Design a diagnostic subsystem that remains useful during failure.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Observability with bounded resource and privacy costs.

Spoken answer: Use structured event IDs, monotonic timing, severity, bounded buffers, rate limiting, and persistent reset/fault summaries where supported. Separate high-value fault evidence from verbose tracing.

Deeper explanation: A log path must not block a critical control path or recursively fail while reporting its own failure. Include build identity, configuration revision, counters, and state transitions, while redacting credentials and unnecessary payloads. Define behavior when storage is full and when clocks are unsynchronized. Verify that injected failures produce actionable evidence and that excessive logging cannot exhaust memory.

Assumptions: Assume a learning system with a defined retention budget and no invented hardware persistence guarantees.

Common incorrect answer: Logging every byte synchronously in an ISR or assuming more logs always improve diagnosis.

Follow-up: Which five fields would you preserve for a communication timeout after a reboot?

Scoring out of 4: 1 point each: structured evidence; bounded path; failure/retention; useful diagnostic test.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q189 Design a small optional anomaly detector without giving it control authority.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Data methodology and safe integration boundaries.

Spoken answer: Collect timestamped sensor data with operating context, define a simple non-AI baseline, separate training/validation/test by run or time, and evaluate a small model in userspace.

Deeper explanation: Measure false alarms, missed anomalies, inference latency, memory, CPU, and model size using actual execution. Prevent leakage from near-duplicate windows across splits. Keep existing deterministic checks and a clear fallback if inference fails or data is out of distribution. The model may annotate diagnostics; it should not replace MCU timing or motion limits in this learning system.

Assumptions: Assume labeled or carefully characterized data exists; synthetic results must be labeled and not presented as field accuracy.

Common incorrect answer: Reporting training accuracy as deployed performance or running inference in a kernel interrupt handler.

Follow-up: How would you detect that a new sensor revision invalidates the model's input distribution?

Scoring out of 4: 1 point each: baseline/data split; measured evaluation; application boundary; fallback/drift.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

#### Q190 Design a test strategy for a driver and firmware pair before boards arrive.

Track: All three tracks | Difficulty: Advanced | Round: System design | Tags: system-design

Assesses: Maximizing useful evidence without overstating emulation.

Spoken answer: Extract pure parsing and state logic for host tests, compile target code against pinned interfaces, simulate the communication peer, and use supported QEMU targets for integration where meaningful.

Deeper explanation: Create a hardware-validation matrix for remaining electrical, timing, DMA, interrupt, power, and peripheral behaviors. Use synthetic logs and faults with explicit labels. Test malformed packets, restart ordering, queue overflow, and teardown now, preserving command lines and outputs. When boards arrive, prioritize the highest-risk assumptions rather than repeating only happy-path software tests.

Assumptions: Assume emulator device models do not reproduce every selected physical peripheral.

Common incorrect answer: Calling host compilation hardware validation or delaying all testing until every board is available.

Follow-up: Which failure would you expect a PTY test to reveal, and which requires a logic analyzer?

Scoring out of 4: 1 point each: portable logic; target build/simulation; explicit gaps; risk-based hardware plan.

Learning connection: Module M17. Verification references: IIO, RAUC, KERNEL, FREERTOS, QEMU.

### Project defense behavioral and AI judgment

#### Q191 How would you explain a project you built with AI assistance honestly?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Ownership, understanding, and evidence.

Spoken answer: Describe the problem, your decisions and implementation work, where AI assisted, how you verified suggestions, and which results you personally reproduced. Separate generated code from demonstrated competence.

Deeper explanation: A credible explanation can say that AI proposed parser edge cases while you defined the protocol, reviewed code, wrote tests, and diagnosed a failing case. Do not invent percentages or claim independent authorship of work you cannot explain. Prepare to modify one requirement live and predict which tests change. Keep provenance and validation notes with the project.

Assumptions: Assume the interview permits discussion of AI-assisted personal projects; obey its stated tool rules.

Common incorrect answer: Hiding material assistance or claiming all generated code is correct because it compiled once.

Follow-up: Which AI suggestion did you reject, and what evidence led you to reject it?

Scoring out of 4: 1 point each: truthful role; decisions; independent verification; live understanding.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q192 What should you say when you do not know an API or answer?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Intellectual honesty and a useful investigation method.

Spoken answer: State the boundary of your knowledge, explain the relevant principles you do know, and outline how you would verify the missing detail from the selected version's documentation or source.

Deeper explanation: For example, distinguish confidence in a lifetime requirement from uncertainty about one helper's exact signature. Ask permission to consult documentation when the interview format allows it. Avoid inventing a plausible function name or presenting a guess as fact. A structured hypothesis with a proposed check is stronger than a memorized but unsupported claim.

Assumptions: Assume the interviewer has specified whether documentation or coding tools are allowed.

Common incorrect answer: Bluffing an API or answering only that you do not know without showing any reasoning.

Follow-up: How would you test whether a helper cancels in-flight work synchronously?

Scoring out of 4: 1 point each: knowledge boundary; valid principle; verification plan; honest uncertainty.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q193 How do you turn a learning project into a credible resume bullet?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Evidence-based claims and personal contribution.

Spoken answer: State what you built, the relevant technical responsibility, and a measured result or verified capability with its test scope. Use numbers only when recorded evidence supports them.

Deeper explanation: A template is: built a bounded serial telemetry parser in C; added fragmentation and malformed-frame tests; verified the listed host cases and documented hardware checks still pending. Replace template fields only after completion. Avoid calling a simulator production deployment or inventing latency improvements. Link the repository, reproducible command, and a short demonstration when appropriate.

Assumptions: Assume no employer or production experience beyond what the candidate actually has.

Common incorrect answer: Writing improved reliability by 80 percent without a baseline, metric, workload, and recorded result.

Follow-up: How would you phrase a kernel module that compiles but has not yet been loaded on its target?

Scoring out of 4: 1 point each: concrete artifact; personal work; scoped evidence; truthful limitations.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q194 How should you defend a design decision that has a clear drawback?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Tradeoff reasoning and willingness to revise.

Spoken answer: State the requirement that drove the choice, alternatives considered, its cost, and the evidence that made the tradeoff acceptable. Explain what change would cause you to revisit it.

Deeper explanation: For a fixed-size queue, bounded memory may outweigh occasional telemetry loss if loss is counted and the product permits it. That defense fails if every sample is required. Use actual constraints rather than personal preference, and distinguish a deliberate learning shortcut from a production recommendation. Acknowledge evidence that weakens the decision instead of defending it reflexively.

Assumptions: Assume requirements and test evidence are available for the project being discussed.

Common incorrect answer: Saying the chosen tool is always best or pretending there were no disadvantages.

Follow-up: What specific requirement change would make your current queue policy unacceptable?

Scoring out of 4: 1 point each: requirement; alternatives/cost; evidence; revision trigger.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q195 How do you answer a behavioral question about a failure without inventing experience?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Reflection, accountability, and truthful scope.

Spoken answer: Use a real work, study, or personal-project example. Explain the situation, your responsibility, the action you took, the observed outcome, and what you changed afterward.

Deeper explanation: A small debugging mistake can demonstrate sound judgment if you identify its consequence and prevention. Do not inflate a class project into a production incident or attribute a team's work solely to yourself. If you have no relevant example, say so and discuss a related genuine situation, clearly labeling hypothetical approaches as hypothetical.

Assumptions: Assume the candidate's actual history has not been supplied; all answer structures are templates.

Common incorrect answer: Memorizing a fabricated rescue story or removing every sign of personal responsibility.

Follow-up: What permanent change in your test or review process resulted from that experience?

Scoring out of 4: 1 point each: real context; personal responsibility; observable outcome; learning action.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q196 An AI suggests a kernel helper that is absent from your headers. What is your workflow?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Version-aware verification and resisting plausible inventions.

Spoken answer: Check the exact target source and documentation, determine whether the helper exists in another version or subsystem, and redesign against supported APIs rather than forcing an undeclared symbol.

Deeper explanation: Provide the model with kernel version, configuration, relevant declarations, and the actual build error. Ask it to identify evidence for every proposed API. Compile a focused change and run the tests relevant to lifetime and failure behavior. A function with a similar name may have different context or ownership semantics, so replacing names mechanically is unsafe.

Assumptions: Assume access to the pinned kernel tree and permission to share the code context being used.

Common incorrect answer: Adding a hand-written prototype for an unavailable function or trusting a confident generated explanation.

Follow-up: How would you distinguish a missing Kconfig dependency from a completely invented API?

Scoring out of 4: 1 point each: source check; version/context; supported redesign; focused validation.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q197 How can AI-generated tests give false confidence?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Independent oracles and adversarial test design.

Spoken answer: A model may repeat the implementation's assumptions, test only happy paths, or assert the same incorrect formula. Use independently specified behavior and fixed external vectors where appropriate.

Deeper explanation: For CRC, test a known check value rather than only encode-decode round trips. For a parser, include every fragmentation boundary, malformed lengths, and bounded recovery. Mutation testing can show whether tests detect intentional defects, but does not prove completeness. Review expected outputs before running them and keep at least one assessment independent of AI assistance.

Assumptions: Assume tests are used as engineering evidence, not as proof of exhaustive correctness.

Common incorrect answer: Counting many generated test functions as coverage without examining their assertions.

Follow-up: Which single mutation would reveal whether your overflow test actually exercises the overflow branch?

Scoring out of 4: 1 point each: independent oracle; edge cases; mutation limit; independent competence.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q198 How would you present benchmark results from an emulator or shared host?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Measurement honesty and practical limits.

Spoken answer: State platform, configuration, workload, sample count, clock, measurement method, and interference conditions. Report observed distributions and avoid treating them as target hardware guarantees.

Deeper explanation: Compare changes under the same conditions and preserve raw data. A noisy shared host can still reveal a large algorithmic regression, but tail latency may reflect host scheduling. Do not invent before/after numbers or call one fast run an improvement. Hardware timing, cache behavior, bus delays, and physical control performance remain separate measurements.

Assumptions: Assume the benchmark was actually executed; otherwise label the metric as a proposed acceptance target.

Common incorrect answer: Reporting an emulator's best latency as the robot's worst-case response time.

Follow-up: What measurements would you repeat first when the selected board becomes available?

Scoring out of 4: 1 point each: method metadata; distributions; comparison control; scope limits.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q199 How should you prioritize preparation when a JD contains far more topics than you can master soon?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Requirement extraction and evidence-based gap closure.

Spoken answer: Separate explicit must-haves from nice-to-haves and inferred suggestions, map each to evidence, and prioritize shared foundations plus the target track's essential practical tasks.

Deeper explanation: Mark unassessed skills as unassessed rather than assigning optimistic scores. For a driver role, strengthen C, context/locking, resource lifetime, bus/subsystem knowledge, and one defensible driver project before optional cloud breadth. A firmware role may instead prioritize startup, interrupts, RTOS timing, and hardware debugging. Reassess after a timed mock and use failures to choose remediation.

Assumptions: Assume an actual JD is supplied; otherwise call the matrix a reference framework.

Common incorrect answer: Treating one employer's stack as universal or learning every keyword to the same depth.

Follow-up: Which missing evidence would most change your confidence in readiness for this specific role?

Scoring out of 4: 1 point each: explicit/inferred split; evidence; role priority; reassessment.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

#### Q200 What demonstrates interview readiness beyond knowing 200 answers?

Track: All three tracks | Difficulty: Practical | Round: Project defense | Tags: project-defense;behavioral

Assesses: Transfer of knowledge into implementation and explanation.

Spoken answer: Solve representative tasks independently, debug unfamiliar failures methodically, explain tradeoffs and limitations, and defend a reproducible project with actual evidence under time constraints.

Deeper explanation: Use the bank for retrieval practice, then change inputs, requirements, and failure modes so memorized wording is insufficient. A readiness gate should include no critical correctness failures, a passing practical task, a role-specific mock, and an honest project demonstration. Selection also depends on role fit and interview conditions, so preparation cannot guarantee an offer.

Assumptions: Assume the candidate has completed foundations and the relevant track projects, not merely read the answer file.

Common incorrect answer: Equating a high multiple-choice score or an AI-produced repository with independent engineering competence.

Follow-up: Can you change one protocol requirement live, update tests, and explain the new failure modes?

Scoring out of 4: 1 point each: independent task; debugging; project evidence; realistic readiness limits.

Learning connection: Module M18. Verification references: GCC, KERNEL, FREERTOS, YOCTO.

# 9 Assessment and exercise answer keys

Attempt the associated task first. Use these answers to compare reasoning and required evidence, then retest a changed case independently.

## Debugging challenge answer key

DBG01: dereferencing input[0] requires nonnull input and available at least one. Copying requires length no greater than 16 and length no greater than available minus one. Validate before memcpy; a success-only output contract keeps caller state intact. Empty/one-byte-short/oversized/truncated cases fail; 02 41 42 succeeds with AB. See parser_repair.c. Award bounds correctness 30, order/reasoning 15, targeted regression 25, resource/state 20, explanation 10.

DBG02: a spurious wake can leave count zero; a second consumer can consume the sole item before the first reacquires the mutex; shutdown never appears in the predicate and may never wake waiters. Use while (!count && !stopping) wait, return a distinct stopped result when empty and stopping, otherwise decrement only when positive. Define drain behavior and notify all required waiters during shutdown. Every wait uses the same predicate mutex. Do not join while holding it.

DBG03: the trace supports a live loop repeatedly processing hangup/EOF without a state transition. It does not prove cable, voltage or peer reset cause. On readable plus hangup, drain available data under the API contract, then close/transition; empty EOF ends the connection. Back off bounded reconnect attempts. Verify a disconnected descriptor is removed from the interest set.

DBG04: deferred work may retain a pointer released during remove. Confirm the exact object's ownership and freeing path against matching source and binary; the synthetic excerpt alone does not prove every step. Prevent new work and rearm, mark disconnected, synchronize worker completion without holding a lock the worker requires, then release state or final references. Devm release alone does not quiesce asynchronous callbacks. Existing opens may need a separate reference-counted state.

DBG05: acquisition and control last progressed 400 ms before now; logger last progressed 350 ms before now. Only the control timeout is supplied, so control is demonstrably overdue by 300 ms; do not invent acquisition/logger timeout limits. Supported chain: control waits for log mutex, owned by logger, which waits for storage. Move logging behind a bounded queue so control does not block on storage; supervise useful progress. Reset-cause evidence says watchdog, not a particular CPU fault. Later validate real clocks, debugger effects and reset-domain behavior.

DBG06: hard IRQ cannot perform the proposed sleeping lock/bus operations under the conventional context assumptions. An unbounded list violates resource limits; user pointers must not be retained and dereferenced as ordinary kernel addresses; user-copy APIs need proper context and size checks. Queue bounded data, expose read/poll, keep copy operations in task context, and stop IRQ/work sources before state release. Follow a fitting subsystem such as IIO when appropriate. Review PREEMPT_RT separately rather than using it as an excuse for unspecified context.

DBG07: 100 samples cannot support a strong empirical 99.9th-tail characterization; the observed maximum is only the maximum of those samples under those conditions. Specify duration and workload, preserve raw measurements, repeat across relevant interference states, and state uncertainty. Formal hard deadlines require bounded assumptions and analysis beyond the reported experiment. No benchmark numbers should be invented.

DBG08: active-slot overwrite threatens known-good recovery; CRC is not authorization; boot metadata can select incomplete data; PID 1 existence is weak health evidence. Verify a signed compatible bundle, write inactive slot, sync and activate through the supported boot protocol, confirm useful health with a deadline, and roll back after defined failures. Test interruption at each persistent transition and data migration compatibility. Host simulation does not prove physical storage power-loss behavior.

## Diagnostic answer key

D01: the array's automatic lifetime ends on return. A caller-owned buffer with capacity or explicit heap ownership is valid; immutable static storage has shared lifetime and reentrancy limitations. Award 3 lifetime, 3 alternatives, 2 tradeoffs.

D02: under C11, signed overflow is undefined; unsigned addition wraps modulo the range; conflicting unsequenced increments are undefined. No universal output exists for UB. Award 2 each classification and 2 explicit version/assumption.

D03: reject null; if b != 0 && a > SIZE_MAX/b, fail before multiplication; otherwise store the product. The first two pairs succeed, the third fails, null fails. Award 5 precheck, 2 zero, 2 unchanged failure, 3 tests. See coding_reference.c checked_mul. Out-of-bounds or overflow-based validation fails the critical gate.

D04: one valid sequence is mktemp -d, create files under that path, ls -l, command 2>errors.txt, rg 'ERROR' file, then initialize and commit in that disposable directory. Verify actual output; do not award execution points for a copied transcript. Award 2 permissions, 2 redirection, 2 search, 2 Git evidence. Configure a local temporary Git identity only as needed, without modifying global settings.

D05: streams lack application message boundaries. Positive write results advance offset; EINTR retries under API semantics; EAGAIN waits for readiness in nonblocking mode; read zero indicates EOF for ordinary streams. Award 3 stream model, 3 progress, 2 errors.

D06: hold the mutex; loop while empty and not stopping; wait atomically releases/reacquires; on stop follow drain/discard policy; otherwise consume under the lock. Wake all necessary waiters on shutdown and join before destruction. Award 3 loop, 3 predicate/lock, 2 shutdown, 2 lifetime.

D07: ROM/firmware/bootloader/kernel/init is representative, not universal. MCU startup initializes a bare-metal or RTOS runtime; an application processor commonly provides MMU and Linux support. Award 3 stages, 3 platform distinction, 2 board qualification.

D08: these potentially sleeping operations are inappropriate in a conventional hard-IRQ context. Capture/acknowledge bounded state and use a documented threaded handler or workqueue where appropriate. PREEMPT_RT requires configuration-aware reasoning. Award 3 context, 3 deferred design, 2 caveat.

D09: define buffer states FREE, FILLING, READY, CONSUMING, with a single owner at each transition; use an ISR-safe notification and queue or descriptor ring, and do not rearm a consumed buffer until returned. DMA cache synchronization and interrupt priority restrictions remain target-specific. Award 4 ownership, 2 notification, 2 hardware qualification.

D10: repeated POLLHUP and read zero support an unhandled EOF/hangup spin. They do not prove the remote electrical cause. Transition out of the active read loop, drain correctly, and reconnect with a bounded policy. Award 3 evidence, 2 restraint, 3 test/fix.

D11: CRC is accidental-error detection, signatures authenticate under a trust policy, verified boot checks startup components in a trust chain. None alone guarantees availability, physical safety, or every runtime state. Award 4 distinctions, 3 limits.

D12: award 2 concrete requirement and role, 2 supported result, 2 failure/lesson, 1 limitation. Fabrication fails the truthfulness gate. A small truthful practice example can receive full credit.

## Final practical and defense marking key

A strong submission makes one path complete and bounded before adding features. The parser has explicit partial-frame state, rejects excessive length before writing, validates CRC before exposing data, and specifies recovery after malformed input. Tests use known vectors and all split points rather than only self-round-trips.

The queue has a fixed limit and a visible overload policy. The accepted, rejected/dropped and processed counts reconcile under that policy. A peer session change invalidates old command authority; raw sequence numbers alone do not prove a new boot's identity. Staleness uses a monotonic deadline local to the responsible component.

Persistence preserves a valid configuration through the injected pre-replacement failure. The result is labeled host-filesystem evidence; real power-cut behavior is pending without target storage testing. Shutdown wakes every blocking path and releases state after users quiesce. A two-second observed host test pass is not a real-time bound for arbitrary deployment.

For C02, all startup, stale, disconnected and FAULT paths must leave output permission false until the defined explicit re-arm transition. A simulator demonstrates state logic, not a certified stopping function. For C01, stale measurements carry invalid/freshness state rather than silently remaining apparently current.

There is no single mandatory architecture beyond the stated requirements. Award full points to simpler correct designs with clear contracts. Do not require networking, cloud services or AI inference. Optional AI must not substitute for protocol, state, resource or recovery work. Penalize overclaims and missing failure tests, not a transparent pending hardware item.

During defense, trace one actual object through allocation, publication, use, cancellation and destruction. Ask the learner to predict a test outcome before running it. A live requirement change must update validation, buffer/limit assumptions, protocol compatibility and documentation where relevant. If the learner cannot explain delivered generated code, record that skill as assisted or unassessed rather than mastered.

## Phase exit marking guidance

Use the rubric in Scoring_and_Evidence.md and the specific criteria below. A missing board can leave a hardware check pending, but it cannot be converted into a passed execution criterion. Grade the available host/emulator scope explicitly.

### G01 Foundations

Award correctness only when the stated test conditions are demonstrated or accurately analyzed. Required evidence: C source/tests, ELF inspection transcript, explanation of UB and padding. Reject a pass if this gate fails: Bounds and unchanged-failure behavior are mandatory. Compare mechanisms to the relevant interview answers and project acceptance tests; do not award points merely for matching wording. For retest, use different input sizes or fault order and require the learner to explain why the correction generalizes. Remediation: Repeat M02 integer/lifetime exercises and B01 with changed capacities.

### G02 Linux systems

Award correctness only when the stated test conditions are demonstrated or accurately analyzed. Required evidence: Source, PTY/pipe or injected-read tests, thread shutdown transcript, failure table. Reject a pass if this gate fails: No silent truncation, data race or indefinite required shutdown. Compare mechanisms to the relevant interview answers and project acceptance tests; do not award points merely for matching wording. For retest, use different input sizes or fault order and require the learner to explain why the correction generalizes. Remediation: Repeat partial-I/O lab and condition-variable exercise before retesting.

### G03 Images and drivers

Award correctness only when the stated test conditions are demonstrated or accurately analyzed. Required evidence: Build evidence, source review, matching kernel config and an explicitly pending hardware matrix. Reject a pass if this gate fails: No unsafe user pointer or asynchronous use-after-free; never invent a boot log. Compare mechanisms to the relevant interview answers and project acceptance tests; do not award points merely for matching wording. For retest, use different input sizes or fault order and require the learner to explain why the correction generalizes. Remediation: Repeat I02/I03 lifecycle tests in a disposable VM and verify target headers.

### G04 Firmware and RTOS

Award correctness only when the stated test conditions are demonstrated or accurately analyzed. Required evidence: Portable tests, state table, stack/timing measurement plan and pending board checks. Reject a pass if this gate fails: Stale command disables permission and fault never clears implicitly. Compare mechanisms to the relevant interview answers and project acceptance tests; do not award points merely for matching wording. For retest, use different input sizes or fault order and require the learner to explain why the correction generalizes. Remediation: Repeat explicit state machine, watchdog and RTOS ownership lessons.

### G05 Production and diagnosis

Award correctness only when the stated test conditions are demonstrated or accurately analyzed. Required evidence: Failure matrix, raw available measurements, update/configuration evidence, limitations. Reject a pass if this gate fails: No forged benchmark, unauthorized update acceptance or destruction of the only recovery path. Compare mechanisms to the relevant interview answers and project acceptance tests; do not award points merely for matching wording. For retest, use different input sizes or fault order and require the learner to explain why the correction generalizes. Remediation: Revisit measured-versus-proven timing, persistence sequencing and A03 recovery.

### G06 Capstone integration

Award correctness only when the stated test conditions are demonstrated or accurately analyzed. Required evidence: Reproducible demo, requirement-to-test table, results, evidence gaps and 5-minute architecture explanation. Reject a pass if this gate fails: No undocumented control continuation, fabricated pass, unbounded queue or unexplained lifecycle fault. Compare mechanisms to the relevant interview answers and project acceptance tests; do not award points merely for matching wording. For retest, use different input sizes or fault order and require the learner to explain why the correction generalizes. Remediation: Repair the failing requirement, add regression, then retake with changed event order.

## Role mock marking guide

For MOCK L use Q052, Q059 and Q071 criteria, coding_reference.c parse_u32, posix_reference.c and DBG03/DBG02 keys. Expect explicit blocking-helper limits, signed syscall checking, suffix progress, shutdown wakeups and a current-evidence boundary. Ask a follow-up that changes the descriptor to nonblocking; a candidate must not retain the same retry loop without a readiness/deadline design.

For MOCK K expect a short protected state transfer or a suitable mutex strategy, user copies in an allowed context, defined partial-copy/consumption semantics, and state lifetime beyond unbind. There is no universal single implementation for the proposed FIFO, so require consistency between its contract and code. Probe deferral must be tied to a supplier rather than every error. The synthetic KASAN report supports a lifetime hypothesis, not a proven line-number root cause. Use Q123, Q125, Q129 and DBG04.

For MOCK F use control_step, seq_newer and DBG05 keys. Sequence freshness needs a half-range and session/reboot contract. ARM does not clear FAULT; CLEAR returns to DISARMED. A heartbeat is useful progress, not merely periodic execution. Stack estimates and host tests do not establish target worst-case timing. Ask what happens if the MCU resets while Linux keeps transmitting an old session.

Across roles, award communication for stating assumptions, defining interfaces and explaining tradeoffs. Do not reward unnecessary jargon. A candidate may consult permitted documentation and still earn full points if the reasoning and validation are their own. Apply the critical gates even when the total would otherwise pass. Retain the original score, remediation and retake evidence separately.

## Timed coding answer key

C15 expected: empty rejected; 0 accepted as 0; maximum accepted; overflow, trailing text, sign and whitespace rejected. Failure preserves the previous output. Reference: coding_reference.c parse_u32, Q028. Critical failure: an overflowing intermediate or accepting a partial parse as complete.

C30 expected: after filling count is eight; ninth push leaves it eight; after three pops head is zero, tail three, count five; wraparound writes retain FIFO; final drain yields 3 through 10 exactly once. Reference: ring_push/ring_pop, Q023. Critical failure: full/empty ambiguity or storage access outside capacity.

C60 expected: parse header and payload in separate states, cap length before payload writes, preserve partial state, and reset to header state only by the documented explicit reset after invalid length. Empty chunk consumes zero. A complete zero-length record is emitted immediately after its header. Unknown type may be structurally accepted by this exercise. The answer is not allowed to silently claim recovery from arbitrary junk without a framing rule. Reference implementation: streaming_reference.c. Critical failure: any input writes beyond 16 bytes or an invalid length causes unbounded buffering.

C90 expected: mutex protects head, tail, count and stop. Producer waits while full and not stopped; consumer waits while empty and not stopped, and exits only when stopped and empty. Both predicates loop. Shutdown broadcasts. No thread is joined while the joining thread holds the queue mutex. See Q069 and the B04 project for a complete queue integration. Score independently from the delivered reference; actual candidate tests remain required.

C60F expected: FAULT event from any state enters FAULT; CLEAR only changes FAULT to DISARMED; ARM changes DISARMED to ARMED and leaves ARMED armed; timeout/disarm disable nonfault states; command cannot arm; fault state ignores arm/command/timeout except explicit clear. Reference: control_step and tick_due. Critical failure: any path allows motion in FAULT or re-arms solely because a command arrived.

For all tests, use the 100-point rubric and critical gates. Award tests points for independent expected values and boundary coverage, not merely calling the function many times. An unfinished optional extension must not erase a complete minimum solution, but document it rather than claiming it passed.

## Topic quiz marking key

Use the indicated spoken answer as a core mechanism and the four criteria as the rubric. Accept equivalent correct reasoning. An answer that repeats the wording but cannot handle its follow-up is not strong evidence; ask for a changed example.

### T01 Module M01

Q041: Preprocessing expands includes and macros, compilation produces assembly, assembly produces objects, and linking resolves references and creates an executable or library. Marking: 1 point each: stages; plausible symbol causes; object inspection; declaration/definition distinction.

Q049: Define a reliable good/bad test, identify known boundary revisions, and bisect while controlling environment. Mark untestable commits explicitly rather than labeling them good. Marking: 1 point each: classifier; environment; intermittent handling; exposure versus root cause.

Q050: Warnings catch many suspicious constructs, static analysis reasons about paths without normal execution, and sanitizers instrument executed paths to detect selected runtime violations. Marking: 1 point each: tool roles; separate builds/support; coverage limitation; recorded evidence.

### T02 Module M02

Q001: An array owns a sequence of elements; a pointer holds an address. An array parameter is adjusted to a pointer, so pass its length separately. Marking: 1 point each: object distinction; parameter adjustment; length contract; correct one-past rule.

Q006: No. The two modifications of the same scalar are unsequenced in C11, so the expression has undefined behavior and no valid universal output. Marking: 1 point each: UB classification; unsequenced conflict; valid rewrite; distinction from unspecified order.

Q018: Volatile does not make an increment atomic or establish synchronization between threads. Use a mutex or a suitable C atomic operation, depending on the required invariant. Marking: 1 point each: race; atomic RMW; relaxed limit; multi-field invariant.

### T03 Module M03

Q023: Track head, tail, and count. Push only when count is below capacity; pop only when count is nonzero. Advance indices modulo the fixed capacity. Marking: 1 point each: invariant; wraparound; no mutation on failure; concurrency qualification.

Q028: Require at least one decimal digit and reject signs, whitespace, and trailing characters under the stated grammar. Before each step, ensure the next multiply-add fits. Marking: 1 point each: grammar; precheck; status contract; maximum and malformed cases.

Q039: Turn the alarm on at or above the high threshold and off at or below the low threshold. Between thresholds, preserve the prior alarm state. Marking: 1 point each: two transitions; hold band; boundary tests; invalid/stale-data policy.

### T04 Module M04

Q043: An API defines source-level interfaces and behavior. An ABI defines binary conventions such as calling convention, data layout, symbol naming, and object format. Marking: 1 point each: API/ABI distinction; concrete conventions; compatibility checks; kernel qualification.

Q044: Resource Acquisition Is Initialization ties resource ownership to an object's lifetime so destructors release resources on every normal scope-exit path, including exceptions. Marking: 1 point each: lifetime cleanup; move-only ownership; nonthrowing destructor; explicit error tradeoff.

Q047: Many traditional linkers scan archives to satisfy unresolved symbols known at that point, so objects that require symbols commonly precede their providing archives. Marking: 1 point each: unresolved-symbol scan; example order; linker qualification; dependency reasoning.

### T05 Module M05

Q051: A process provides a resource context and address space; threads execute within it and share many resources. A file descriptor is a per-process integer handle to an open file description. Marking: 1 point each: process; thread sharing; open description; descriptor reuse consequence.

Q101: An application processor commonly runs a rich OS with virtual memory and external memory; a microcontroller commonly emphasizes integrated peripherals, bounded resources, and direct or RTOS-controlled execution. Marking: 1 point each: practical distinction; hardware verification; software services; portable boundary.

Q102: An MMU can translate virtual addresses and enforce permissions through mappings. An MPU typically applies access permissions and attributes to regions without providing the same general virtual-memory model. Marking: 1 point each: translation; region protection; limitations; non-CPU access caveat.

### T06 Module M06

Q052: A successful read or write may transfer fewer bytes than requested. Track progress, handle interruptions and nonblocking conditions, and treat EOF according to the operation's contract. Marking: 1 point each: partial progress; errors; stream framing; absolute deadline.

Q055: Convert the signal into normal control flow, stop accepting work, notify blocked workers, drain or discard by policy, join them, then release shared resources. Marking: 1 point each: safe notification; blocked-worker wake; join-before-free; deadline policy.

Q059: A monotonic clock is intended for measuring elapsed time without civil-time jumps. Use a single deadline and recompute remaining time after interruptions or partial progress. Marking: 1 point each: elapsed-time clock; absolute deadline; suspend semantics; conversion/expiry.

### T07 Module M07

Q071: The condition variable is a notification mechanism; shared state is the truth. Protect the predicate with a mutex and wait in a loop that rechecks it after waking. Marking: 1 point each: predicate; atomic wait transition; while loop; correct notification scope.

Q074: A high-priority task can wait for a lower-priority lock owner while unrelated medium-priority work delays that owner. Priority inheritance temporarily raises the owner's effective priority. Marking: 1 point each: three-task mechanism; temporary boost; limits; measurement.

Q077: A flag expresses intent but does not wake blocked I/O, condition waits, or sleeping threads. Cancellation must reach each blocking point and define ownership of unfinished work. Marking: 1 point each: wake paths; synchronized state; completion race; lifetime barrier.

### T08 Module M08

Q103: A platform may execute immutable ROM, early firmware, a bootloader, the kernel, and init or a service manager. Each stage initializes enough state and verifies or loads the next as configured. Marking: 1 point each: stages; responsibilities; board qualification; stage-specific evidence.

Q105: Device tree describes hardware connections and properties according to bindings; the driver implements the device's behavior using those resources and subsystem interfaces. Marking: 1 point each: description; binding contract; resource dependencies; validation.

Q108: A sysroot provides the target-facing headers and libraries used by a cross toolchain. It prevents accidental use of incompatible host interfaces when building target applications. Marking: 1 point each: sysroot role; ABI/libc match; ELF check; dependency isolation.

### T09 Module M09

Q112: Process context runs on behalf of a schedulable task, although held locks or disabled preemption can restrict it. Hard interrupt context cannot perform operations that may sleep. Marking: 1 point each: task context; IRQ restriction; held-lock caveat; deferred-context selection.

Q114: A mutex is a sleeping lock for eligible task contexts. A conventional spinlock protects short non-sleeping sections; PREEMPT_RT changes spinlock_t semantics, so configuration matters. Marking: 1 point each: sleeping distinction; context; RT qualification; per-state locking contract.

Q116: Managed resources simplify release when a device detaches or probe fails. They do not automatically stop work, timers, open-file users, or callbacks that still reference those resources. Marking: 1 point each: managed scope; asynchronous users; teardown order; open-file lifetime.

### T10 Module M10

Q123: A user pointer must be accessed through the appropriate user-copy APIs, with validated sizes and error handling. Direct dereference can fault or violate the kernel's access rules. Marking: 1 point each: user-copy boundary; return semantics; context; state/initialization policy.

Q125: Not necessarily: another consumer may have consumed the data. Readiness is not a reservation. A nonblocking read should then return EAGAIN under the documented contract. Marking: 1 point each: race distinction; nonblocking outcome; shared predicate; wait registration.

Q129: Mark the device disconnected, prevent new hardware operations, wake blocked users, and return the documented EOF or error behavior. Keep backing state alive until existing references are released. Marking: 1 point each: disconnected state; wake behavior; references; teardown tests.

### T11 Module M11

Q134: IIO commonly separates raw readings and conversion attributes so userspace can interpret samples consistently without requiring floating-point work in the kernel. Marking: 1 point each: raw/scale; units/offset; buffer layout; calibration test.

Q137: A DMA address is the address the device uses after the platform's mapping rules, potentially including an IOMMU. It is not generally a cast of the CPU pointer. Marking: 1 point each: domains; DMA API/type; mask; mapped-segment semantics.

Q139: System suspend coordinates a broader system sleep transition. Runtime suspend idles an individual device while the system remains active, subject to dependencies and usage references. Marking: 1 point each: scope; usage/dependencies; state restoration; transition tests.

### T12 Module M12

Q141: Startup establishes the stack and vector handling as required by the architecture, initializes writable data, clears zero-initialized storage, and performs necessary runtime setup before main. Marking: 1 point each: stack/vector; data copy; bss clear; toolchain/platform qualification.

Q143: Identify and acknowledge the event as required, capture minimal necessary data, and notify deferred processing through an ISR-safe mechanism. Keep execution bounded and respect interrupt-priority rules. Marking: 1 point each: acknowledge; bounded capture; ISR-safe notification; measured overload behavior.

Q147: Feed the watchdog only when required subsystems demonstrate timely health under a defined policy. A dedicated feeder that runs regardless of failures can mask a dead system. Marking: 1 point each: useful progress; supervision; reset evidence; special-state behavior.

### T13 Module M13

Q153: An interrupt cannot block waiting for a task-owned resource, and ordinary mutex ownership/priority-inheritance semantics are designed for tasks. Use an appropriate ISR-safe signaling or data-transfer mechanism. Marking: 1 point each: no blocking/ownership; FromISR; priority restrictions; data ownership.

Q157: Specify framing, length bounds, byte order, version, message type, sequence or request ID, integrity check, timeouts, and recovery behavior. Define ownership and state transitions on both sides. Marking: 1 point each: wire contract; command semantics; reset/replay; fault tests.

Q158: The MCU should enforce a local deadline for motion commands and transition to a defined stopped or disabled state when commands become stale, independent of Linux remaining alive. Marking: 1 point each: MCU-local deadline; state transition; stale/replay tests; safety scope.

### T14 Module M14

Q171: Choose from required customization, package workflow, reusable metadata, team familiarity, supported BSPs, and maintenance needs. Buildroot and Yocto can both produce embedded Linux systems through different models. Marking: 1 point each: requirements; models; maintenance/resource tradeoff; validated target support.

Q172: A layer groups related metadata; a recipe describes how to obtain, configure, build, install, and package a component; an image selects a filesystem composition and related outputs. Marking: 1 point each: three roles; layer separation; append/version; task diagnostics.

Q173: Store the product's board configuration, custom package definitions, overlays or post-build steps, and documented external-tree metadata so the build can be reproduced from versioned inputs. Marking: 1 point each: external organization; versioned inputs; package/overlay boundary; image test.

### T15 Module M15

Q081: Capture a short syscall trace and loop counters. Look for repeated readiness or hangup returns followed by zero-byte reads, unhandled errors, or a retry path with no wait. Marking: 1 point each: trace hypothesis; EOF interpretation; state transition; regression/CPU check.

Q088: A loop that does work and then sleeps a fixed period adds its work and scheduling delay to every cycle. Schedule against successive absolute deadlines instead. Marking: 1 point each: relative-drift mechanism; absolute schedule; overrun policy; latency measurements.

Q098: Inspect use of the DMA API, transfer direction, map/unmap or sync transitions, and ownership between CPU and device. CPU virtual addresses are not interchangeable with DMA addresses. Marking: 1 point each: ownership; API mapping; address domains; target coherency test.

### T16 Module M16

Q176: Signed update verification authenticates an update package before accepting it under the update policy. Verified boot checks the boot chain or loaded components according to a trust model at startup. Marking: 1 point each: two boundaries; trust anchor; uncovered modification; key/recovery plan.

Q177: Boot the candidate slot, run meaningful bounded health checks, and confirm only when required functions and persistent-state compatibility meet the defined policy. Retain rollback until that decision. Marking: 1 point each: bounded meaningful health; attempt/rollback; inactive-slot write; data compatibility.

Q180: Define workload, faults, metrics, duration, and pass conditions, then preserve evidence. Long duration is useful only when it exercises relevant states and detects failure meaningfully. Marking: 1 point each: specified workload; metrics; targeted faults; recovery/integrity criteria.

### T17 Module M17

Q181: Keep acquisition and local timing on the MCU, transfer versioned framed telemetry over one primary link, and let Linux validate, persist, expose health, and manage remote integration. Marking: 1 point each: boundary architecture; bounded interface; recovery; measurable acceptance.

Q186: Choose a bounded frame with version, type, length, session/request identity, payload, and CRC, plus escaping or another unambiguous framing method and explicit deadlines. Marking: 1 point each: full wire contract; semantics; bounded recovery; vectors/compatibility.

Q190: Extract pure parsing and state logic for host tests, compile target code against pinned interfaces, simulate the communication peer, and use supported QEMU targets for integration where meaningful. Marking: 1 point each: portable logic; target build/simulation; explicit gaps; risk-based hardware plan.

### T18 Module M18

Q191: Describe the problem, your decisions and implementation work, where AI assisted, how you verified suggestions, and which results you personally reproduced. Separate generated code from demonstrated competence. Marking: 1 point each: truthful role; decisions; independent verification; live understanding.

Q196: Check the exact target source and documentation, determine whether the helper exists in another version or subsystem, and redesign against supported APIs rather than forcing an undeclared symbol. Marking: 1 point each: source check; version/context; supported redesign; focused validation.

Q197: A model may repeat the implementation's assumptions, test only happy paths, or assert the same incorrect formula. Use independently specified behavior and fixed external vectors where appropriate. Marking: 1 point each: independent oracle; edge cases; mutation limit; independent competence.

## Reference implementation validation

Date: 27 October 2026. Host compiler observed: cc (Ubuntu 13.3.0-6ubuntu2~24.04) 13.3.0. These are tests of the delivered reference routines, not scores for the learner.

coding_reference.c compiled with -std=c11 -Wall -Wextra -Werror -pedantic. Execution passed the included assertions for 30 routines covering boundaries, invalid input, wraparound and state transitions. posix_reference.c compiled with the same flags and passed its normal pipe and invalid-descriptor tests. Its test does not claim to force every partial-write or EINTR path.

A build of coding_reference.c with -fsanitize=address,undefined -fno-omit-frame-pointer succeeded. The initial execution failed in LeakSanitizer runtime initialization because the container could not read another task's /proc/.../task under its execution/tracing restrictions. Re-running with ASAN_OPTIONS=detect_leaks=0 passed. Therefore AddressSanitizer and UndefinedBehaviorSanitizer execution are recorded as passed for these inputs, while leak detection is explicitly not validated. The routines use fixed storage; no claim of general memory-safety proof is made.

The supplied broken source and synthetic traces are intentional assessment fixtures. They are not passing implementations, real crash evidence, or hardware measurements. No board, real UART, DMA, IRQ, analog sensor, flash power-cut or motor-control validation was performed by this assessment suite. Target-specific checks remain pending.

Additional executed checks: streaming_reference.c and parser_repair.c both compiled with the same strict C11 warning flags and passed. Streaming tests covered every split of a valid frame, zero-length and coalesced records, excessive length, latched error and explicit reset, truncated-state detection, and maximum payload. The parser repair passed supplied valid, truncated, oversized and unchanged-on-failure cases. No sanitizer run is claimed for these two additional files.

## Learning module exercise solutions

### M01 reference solution and reasoning

Use strtol with errno, end-pointer, range and trailing-character checks. Check argc before reading argv[1]. Build with warnings enabled and test 0, a negative value and a mixed value such as 12x. Capture the failing return code. A reproducibility note names compiler version and source commit. A shell transcript without source and build flags is incomplete.

### M02 reference solution and reasoning

Reject used > capacity first, then test input_length > capacity - used so the check itself cannot overflow. Validate pointers under the stated zero-length convention. If overlap is supported, use memmove; otherwise document and detect only what is safely detectable without invalid pointer comparisons. Update used only after a successful copy. Concurrency requires serialized access or appropriate atomics; volatile does not make the compound increment indivisible.

### M03 reference solution and reasoning

Keep parser state, buffered count and a deadline based on a monotonic clock. On timeout discard the incomplete frame and restart prefix search. Reject unsupported versions according to the published resynchronization rule; do not trust their payload length without a bound. Test that every call consumes input or returns a defined need-more state. Sequence handling belongs to a session with reset rules. A fixed-size buffer bounds memory independently of corrupted lengths.

### M04 reference solution and reasoning

Initialize the descriptor to -1. Delete copy construction and copy assignment. Move construction takes the descriptor and resets the source to -1. Move assignment first releases the destination resource when objects differ, then takes the source. release() returns the integer and leaves -1; the caller now owns cleanup. Do not retry close blindly after EINTR on Linux because descriptor reuse can make a retry close an unrelated resource. std::string must be copied/moved through its language-defined operations, not raw bytes.

### M05 reference solution and reasoning

Place periodic acquisition and immediate local limits on the MCU, with Linux handling storage, configuration and presentation. Timestamp samples at acquisition, carry sequence numbers and age information, and bound both queues. Define the MCU action on communication timeout independently of Linux. Mark every timing budget as a requirement or estimate until measured; host simulation cannot establish electrical or ISR latency.

### M06 reference solution and reasoning

Maintain used length and scan each received chunk for delimiters. Append only within the bound. On overflow, either reject the whole stream or discard through the next delimiter under a documented policy. EOF is a state transition; handle the last partial record explicitly. Retry EINTR with the same absolute deadline. On nonblocking EAGAIN wait for readiness. Close every owned descriptor on all exit paths.

### M07 reference solution and reasoning

Keep every predicate read and write under the same mutex. Wait in a while loop and return from the loop only when work or the shutdown rule permits progress. Broadcast on shutdown, then join before destroying mutexes/conditions. A second consumer may consume the item between another waiter being signalled and acquiring the mutex; that waiter must recheck. Tests should verify the aggregate record sequence/count rather than a scheduler-specific order between consumers.

### M08 reference solution and reasoning

Use file/readelf on the executable, verify its interpreter path inside the target root and confirm architecture. For boot failures, work from earliest verified stage: power/console/boot media before early logs; bootloader image/load addresses before kernel; root device, filesystem and built-in drivers for mount; executable/runtime/configuration for init or service. Every hypothesis should name an observation that can falsify it.

### M09 reference solution and reasoning

Stop publication of new requests and prevent the interrupt/source from requeueing. Synchronize the relevant interrupt handling and cancel or flush the work with APIs appropriate to its type. Only after no callback can use the state may storage be freed. Include remove-under-load, probe-failure, repeated bind/unbind and pending-work tests. The plan must avoid waiting for work while holding a lock that the worker needs.

### M10 reference solution and reasoning

A partial user copy can lose bytes already dequeued. Choose an implementation that commits only the transferred portion or a documented transactional policy with suitable buffering and locks. Do not retain a spinlock across user access. Define readable when data exists, writable when capacity exists, and hangup/error conditions according to teardown. Test with ordinary small buffers and controlled invalid user pages in a disposable guest; never claim a host mock establishes uaccess correctness.

### M11 reference solution and reasoning

Assemble bytes into an unsigned 16-bit value, shift the 12-bit field into the low bits and explicitly sign-extend values >= 0x800 by subtracting 4096. For exact scale, 1 LSB = 62.5 milliunits; use a wider integer representing half-milliunits or document rounding. +25 uses raw 400, -1 uses raw -16 and minimum is -2048. Board evidence includes correct voltage/wiring, address/mode, bus transactions, errors, binding and driver runtime tests. The exercise scale is a worked example; use A01's actual device format there.

### M12 reference solution and reasoning

Each critical task advances a progress token after completing its required work; the supervisor checks all required tokens within a defined window and feeds only if they progressed. A task that loops while repeatedly setting a Boolean is weak evidence; use sequence/time conditions tied to actual completion. Persist reset reason with a bounded write policy, and on repeated failures enter recovery. Hardware reset timing and output state require board measurement.

### M13 reference solution and reasoning

Use a local monotonic receive timestamp for the freshness deadline; do not trust a remote clock without synchronization. Only a valid current-session command updates the deadline. Duplicate or corrupt commands do not extend authorization unless the protocol deliberately defines accepted retransmission. On stale input transition to DISABLED/FAULT and apply bounded local output behavior. A new boot/session requires re-arming. Test boundary values around 200 ms in deterministic simulated time, then measure physical timing separately.

### M14 reference solution and reasoning

Place reproducible source/configuration in the external tree or own layer. Install with explicit modes and directories, include the resulting package in the image, and declare the correct init integration. Keep factory defaults under immutable image control and device settings on a separately defined writable path. On first boot create settings atomically only when absent; do not overwrite an existing user configuration on each service start.

### M15 reference solution and reasoning

Correlate application timestamps with scheduling/wakeup events, I/O syscall durations, lock waiting and queue depth. A blocked fsync suggests storage-related delay; runnable-but-not-scheduled suggests contention; long mutex hold time suggests serialization; periodic queue release suggests batching. Repeat with the same workload and compare tail distributions and correctness. A single improved average cannot justify a deadline claim.

### M16 reference solution and reasoning

Authenticate metadata as well as payload digests, validate board compatibility before installation and compare versions with an explicit policy. Recovery to a previously committed slot may require a carefully defined exception to normal downgrade rejection; otherwise a failed update can brick the system. The policy must survive power loss and resist unauthorized state modification. Host tests can validate the logical transitions, while protected counters/key storage need real platform design.

### M17 reference solution and reasoning

Name the counter semantics in the requirement. Keep integrity rejection separate from valid-command acceptance so corruption never refreshes authorization. Test exact threshold boundaries and reset behavior. Store diagnostics under a bounded policy to avoid a bad link filling storage. If consecutive means since last valid frame, reset only after the specified valid event. Link the requirement, test and observed output in the tracker.

### M18 reference solution and reasoning

Check declarations in official documentation/source, compile under the selected flags and run boundary/failure tests defined before the change. Retain a suggestion only when it improves a stated requirement without regressing another. If no wrong suggestion occurs, report that honestly and analyze one plausible risk rather than inventing an error. Independent explanation and a fresh variant test establish learning better than a polished generated review.

# 10 Validation and progress

## Validation summary

Recorded 27 October 2026. The results below concern the delivered reference implementations. Learner competence remains not assessed. A status is claimed only when a corresponding log exists.

### Portable package check

A fresh relocated copy passed 14 offline host suites/builds with zero failures and zero unexpected skips. Ten external or optional portions were explicitly skipped by that runner; some have separate earlier evidence, such as B05 emulation and I05 RTOS/cross-builds. The runner never silently promotes a skip to a pass. Its durations are not performance benchmarks.

### Evidence by project

| ID | Executed or verified | Still pending or outside that evidence |
| --- | --- | --- |
| B01 | Host build and tests; ASan and UBSan with leak detection disabled | Physical MCU port optional; leak checking unavailable in the recorded environment |
| B02 | Host build live proc and deterministic fixture tests | Other target kernels and permission policies require retesting |
| B03 | Host build PTY framing corruption timeout and terminal cleanup tests | Physical UART electrical/timing checks; production session freshness |
| B04 | Host build queue shutdown output-failure tests and separate TSan run | Target timing and sustained-load qualification |
| B05 | AArch64 static/dynamic cross builds and QEMU user execution | Full-system guest boot not part of B05; GDB session not executed |
| I01 | Host persistence tests and Buildroot external configuration resolution | Full toolchain/image build QEMU boot reboot and power-loss checks |
| I02 | Userspace build and Linux 6.12 kernel composite-object compile | Final module linking loading runtime uaccess poll and unload stress |
| I03 | Linux 6.12 object compile and offline overlay application | Loadable module and real Raspberry Pi GPIO IRQ and remove tests |
| I04 | Native telemetry build/test and complete Yocto metadata authored | BitBake image/SDK builds service integration and guest boot |
| I05 | Real FreeRTOS POSIX tests and STM32 firmware cross-build/link | Flashing physical UART ISR watchdog timing and stack measurements |
| A01 | Conversion tests kernel object compile and offline overlay application | Real TMP102 IIO binding bus errors and power/runtime checks |
| A02 | Host functional tests and raw idle/loaded latency experiments | Target scheduler traces where unavailable and physical timing bounds |
| A03 | Signature tamper interruption health and rollback host simulation tests | Bootloader trust anchor protected rollback state and flash power cuts |
| C01 | Gateway tests and real FreeRTOS to FIFO to gateway integration | Full Yocto appliance build/boot physical sensor and recovery qualification |
| C02 | Deterministic robot tests simulation and optional synthetic AI evaluation | Real Linux MCU transport motors dynamics stopping and hardware safety |

### Evidence interpretation

Code authored means the reference source/configuration is present. Build verified means the stated compiler/link/configuration step completed. Host tested means the named checks ran in this Linux environment. Emulator tested means the named emulator and target ran; B05 uses Linux user mode, not a full guest kernel. Hardware tested would require the named physical board and setup. No physical hardware test is claimed in this edition.

I02, I03 and A01 compiled into kernel composite objects. Final loadable-module linking and kernel runtime checks remain pending. I01's Buildroot configuration resolved its selected components; this is not an image build. I04 and C01 include complete Yocto metadata, but no BitBake image or SDK build is claimed. I05's STM32 source produced an ELF/binary with 6296 bytes of text, 4 bytes of initialized data and 33084 bytes of BSS in the recorded cross-build. This is a link-size observation, not a measured runtime stack margin.

The real RTOS-to-gateway host integration accepted 20 samples with zero rejections in its recorded run. C02 generated 201 deterministic simulation samples. Its approximate scripted travel and command-expiry time are simulation observations, not physical stopping performance. Optional AI metrics use synthetic data and do not establish industrial detection accuracy.

The assessment reference suite also compiled and executed four complete C files with strict C11 warnings: 30 portable routines, a POSIX complete-write helper, a bounded streaming decoder and a repaired parser. AddressSanitizer and UndefinedBehaviorSanitizer passed for the portable suite with leak detection disabled. LeakSanitizer could not initialize under the recorded container restrictions; leak checking is not claimed. See 04_Skill_Assessments/answers/VALIDATION.md for exact commands and results.

### Independent review and regression

Independent review reproduced a UART cancellation defect: SIGTERM left terminal settings in raw mode. The implementation now uses signal-aware cancellation and restores settings on SIGINT, SIGTERM and broken output. PTY regression tests passed after the change. The project includes a debugging case study. This demonstrates a concrete review-to-fix-to-test cycle; it does not imply the tool has been exhaustively verified.

### Next validation work

Run the full Buildroot image and guest boot first, then complete a matching-kernel build and load/runtime tests in a disposable guest. Build the Yocto image/SDK on its documented host. Flash the Nucleo firmware and measure serial, watchdog and stack behavior. Test the GPIO and sensor drivers on the named Raspberry Pi configuration. Only then extend capstones toward physical devices, with separate electrical and control requirements.

Every new target or source revision requires a new version record and affected acceptance tests. Keep raw outputs and test inputs. Do not convert an earlier host pass into a hardware pass by editing a status cell.

## Record learning evidence

Keep learner progress separate from package validation. The supplied implementation logs describe checks performed while producing the package. They do not establish your current proficiency. Your tracker begins at not assessed.

For each assessment save the task, your attempt, source version, exact commands, tool versions, raw outputs, elapsed time, assistance allowed/used, score, critical failures and next correction. Use a stable evidence path in the tracker. Preserve the failed version when practical so a reviewer can see that the regression test catches the original defect.

Use five learner states: not assessed, learning, implemented with assistance, independently demonstrated and needs refresh. Mark independently demonstrated only after completing a new variant without reference code and explaining the result. Revisit important skills after two and six weeks using different input data. An unrun test is recorded as not run; it is never a pass.

The score is subordinate to critical gates. A memory overwrite, proven data race, invalid lifetime, acceptance of an unsafe stale command or invented evidence blocks a pass until corrected. If a tool is unavailable, record the missing check and use another method only for the portion it can actually establish.

## Package progress and remaining checks

### Authored deliverables

The core program contains 18 lessons with independent exercises and separate solutions, a 52-week roadmap, a 58-row reference skills matrix, interview strategy, AI workflows, portfolio guidance, 200 interview questions and structured answers, assessments and exactly 15 project reference implementations. The editable package keeps questions apart from answer keys and code apart from exercises. The Word handbook consolidates the reading material; the workbook and CSVs provide editable planning and evidence tracking.

### Completed verification

Project evidence records distinguish host tests, foreign-architecture emulation, RTOS execution, source/object builds and pending hardware. A relocated copy passed 14 offline host checks. See 10_Progress_Tracking/Validation_Summary.md and individual evidence folders for the scope of each result.

### Pending execution

Full Buildroot and Yocto image/SDK builds, full-system guest acceptance, final kernel-module link/load/runtime checks, all physical board execution, electrical/timing checks, real bootloader power-cut recovery and physical robot integration remain pending. These are validation or target-integration tasks, not claims of completed execution. The minimum authored reference implementations are present for the targets described in each README.

### Next learner task

Take the initial diagnostic independently, record its evidence, then begin M01 and M02 and build B01. If the diagnostic is weak on C ownership or bounds, repeat those exercises before concurrency or drivers. Use the primary job description to choose depth; no employer JD or personal proficiency has been assumed.

### Continuation instruction

To continue, provide your diagnostic attempt and target JD plus the project evidence folder you want reviewed. A precise first continuation is: Review my diagnostic evidence, update only the assessed skills, and guide me through B01 without revealing the reference solution before my attempt. No background task is scheduled by this package.

# 11 Primary references

## Research method and refresh policy

The program uses official project documentation and source, Linux man-pages/POSIX interface documentation, language committee material, and board/manufacturer references. Primary references were checked on 27 October 2026. The reference skills framework is authored for the three tracks; no employer posting is presented as the user's JD.

Version-dependent code is tied to a teaching baseline in each project's VERSIONS.md. The Linux 6.12, Buildroot 2025.02.1 and Yocto 5.0.9 examples are deliberately frozen for reproducibility. They are not claims about the latest patch level. For a new product or a refreshed learning environment, select a maintained release, inspect migration notes/source declarations, update the lock/configuration records and rerun the affected builds and acceptance tests.

The current online Buildroot manual and other rolling documentation can describe features absent from the pinned release. Use release-local manuals and exact source/configuration when executing the projects. Likewise, the QEMU master documentation explains emulation concepts; actual evidence records the installed executable version. A source link is not evidence that the code was executed.

The complete source lists for specialized projects are retained beside those projects. The consolidated source catalogue deduplicates URLs while preserving original titles and claims. Bibliography identifiers in the lessons refer to Core_References.md. Interview and project files also contain direct references so they remain useful independently.

Refresh after a toolchain, kernel, BSP, board revision, datasheet revision or build-system change. Review source API declarations, configuration symbols, register behavior and relevant security advisories. Record the new build and target evidence rather than carrying forward old pass labels. Hardware behavior is claimed only for the named tested setup.

## Core primary references

Accessed 27 October 2026. Versioned references govern the frozen examples; unversioned pages support concepts and must be checked against the selected source before changing code. The public C11 draft is a historical primary language reference. Source citations support factual contracts; project designs, examples, schedules and exercises are authored teaching material.

### C11 WG14 N1570 public C11 committee draft

https://www.open-std.org/jtc1/sc22/wg14/www/docs/n1570.pdf

Use: C language behavior and foundational semantics; not the newest C standard. Accessed 2026-10-27.

### GCC-WARN GCC warning options

https://gcc.gnu.org/onlinedocs/gcc/Warning-Options.html

Use: Diagnostic flags; consult installed-version manual for exact availability. Accessed 2026-10-27.

### GCC13 GCC 13.3 instrumentation options

https://gcc.gnu.org/onlinedocs/gcc-13.3.0/gcc/Instrumentation-Options.html

Use: User-space sanitizer scope and combinations. Accessed 2026-10-27.

### GCC-VOL GCC volatile access rules

https://gcc.gnu.org/onlinedocs/gcc/Volatiles.html

Use: Volatile does not establish ordering of ordinary memory. Accessed 2026-10-27.

### GIT Git official tutorial

https://git-scm.com/docs/gittutorial

Use: Source history and review workflow. Accessed 2026-10-27.

### LD GNU ld linker scripts

https://sourceware.org/binutils/docs/ld/Scripts.html

Use: Section placement and linker-script role. Accessed 2026-10-27.

### CMAKE CMake 3.28 toolchains

https://cmake.org/cmake/help/v3.28/manual/cmake-toolchains.7.html

Use: Cross-compilation and sysroot configuration. Accessed 2026-10-27.

### CPP C++ Core Guidelines

https://isocpp.github.io/CppCoreGuidelines/CppCoreGuidelines

Use: Resource ownership and RAII design guidance. Accessed 2026-10-27.

### READ Linux read manual

https://man7.org/linux/man-pages/man2/read.2.html

Use: Short reads EOF and error handling. Accessed 2026-10-27.

### WRITE Linux write manual

https://man7.org/linux/man-pages/man2/write.2.html

Use: Partial writes and interruption. Accessed 2026-10-27.

### CLOSE Linux close manual

https://man7.org/linux/man-pages/man2/close.2.html

Use: Descriptor release and retry hazards. Accessed 2026-10-27.

### COND POSIX condition wait manual

https://man7.org/linux/man-pages/man3/pthread_cond_wait.3p.html

Use: Predicate mutex and spurious wakeup contracts. Accessed 2026-10-27.

### EPOLL Linux epoll control manual

https://man7.org/linux/man-pages/man2/epoll_ctl.2.html

Use: Readiness registration and event semantics. Accessed 2026-10-27.

### SIGNAL Linux signal safety manual

https://man7.org/linux/man-pages/man7/signal-safety.7.html

Use: Asynchronous signal-safe functions. Accessed 2026-10-27.

### PROC Linux proc pid stat manual

https://man7.org/linux/man-pages/man5/proc_pid_stat.5.html

Use: Process field definitions and access limits. Accessed 2026-10-27.

### FSYNC Linux fsync manual

https://man7.org/linux/man-pages/man2/fsync.2.html

Use: File and directory durability requirements. Accessed 2026-10-27.

### RENAME Linux rename manual

https://man7.org/linux/man-pages/man2/rename.2.html

Use: Atomic replacement scope and filesystem qualifications. Accessed 2026-10-27.

### K-VOL Linux 6.12 volatile guidance

https://www.kernel.org/doc/html/v6.12/process/volatile-considered-harmful.html

Use: Kernel synchronization and I/O accessors. Accessed 2026-10-27.

### KLOCK Linux 6.12 lock types

https://www.kernel.org/doc/html/v6.12/locking/locktypes.html

Use: Context rules and PREEMPT_RT qualifications. Accessed 2026-10-27.

### KALLOC Linux 6.12 memory allocation guide

https://www.kernel.org/doc/html/v6.12/core-api/memory-allocation.html

Use: Allocation flags and context. Accessed 2026-10-27.

### WORK Linux 6.12 workqueues

https://www.kernel.org/doc/html/v6.12/core-api/workqueue.html

Use: Deferred execution and work lifecycle. Accessed 2026-10-27.

### DRIVER Linux 6.12 driver model

https://www.kernel.org/doc/html/v6.12/driver-api/driver-model/driver.html

Use: Driver binding and lifecycle. Accessed 2026-10-27.

### RCU Linux 6.12 RCU explanation

https://www.kernel.org/doc/html/v6.12/RCU/whatisRCU.html

Use: Publication grace periods and reclamation. Accessed 2026-10-27.

### UACCESS Linux 6.12 memory management APIs

https://www.kernel.org/doc/html/v6.12/core-api/mm-api.html

Use: User-space memory access contracts. Accessed 2026-10-27.

### GPIO Linux 6.12 GPIO consumer API

https://www.kernel.org/doc/html/v6.12/driver-api/gpio/consumer.html

Use: Descriptor-based GPIO and sleepability. Accessed 2026-10-27.

### IIO Linux 6.12 Industrial I/O

https://www.kernel.org/doc/html/v6.12/driver-api/iio/index.html

Use: Sensor subsystem channels buffers and triggers. Accessed 2026-10-27.

### DMA Linux 6.12 DMA mapping guide

https://www.kernel.org/doc/html/v6.12/core-api/dma-api-howto.html

Use: CPU and DMA addresses and ownership. Accessed 2026-10-27.

### RPM Linux 6.12 runtime power management

https://www.kernel.org/doc/html/v6.12/power/runtime_pm.html

Use: Device runtime suspend resume and usage. Accessed 2026-10-27.

### DT Linux 6.12 device tree usage

https://www.kernel.org/doc/html/v6.12/devicetree/usage-model.html

Use: Hardware descriptions and bindings. Accessed 2026-10-27.

### DTSCHEMA Linux 6.12 binding schemas

https://www.kernel.org/doc/html/v6.12/devicetree/bindings/writing-schema.html

Use: Schema validation for device descriptions. Accessed 2026-10-27.

### FTRACE Linux 6.12 ftrace

https://www.kernel.org/doc/html/v6.12/trace/ftrace.html

Use: Kernel tracing capabilities and tracefs. Accessed 2026-10-27.

### KASAN Linux 6.12 KASAN

https://www.kernel.org/doc/html/v6.12/dev-tools/kasan.html

Use: Kernel memory detector modes and support. Accessed 2026-10-27.

### KCSAN Linux 6.12 KCSAN

https://www.kernel.org/doc/html/v6.12/dev-tools/kcsan.html

Use: Sampling race detector scope. Accessed 2026-10-27.

### KUNIT Linux 6.12 KUnit

https://www.kernel.org/doc/html/v6.12/dev-tools/kunit/index.html

Use: In-kernel unit testing. Accessed 2026-10-27.

### SOCKETCAN Linux 6.12 SocketCAN

https://www.kernel.org/doc/html/v6.12/networking/can.html

Use: CAN sockets and networking interface. Accessed 2026-10-27.

### QEMU-USER QEMU user-space emulation

https://www.qemu.org/docs/master/user/main.html

Use: Foreign application execution using host syscall translation. Accessed 2026-10-27.

### QEMU-VIRT QEMU generic Arm virt machine

https://www.qemu.org/docs/master/system/arm/virt.html

Use: Generic full-system machine and versioning limits. Accessed 2026-10-27.

### BUILDROOT Buildroot official manual

https://buildroot.org/downloads/manual/manual.html

Use: Build system concepts; execute pinned release-local instructions. Accessed 2026-10-27.

### YOCTO-HOST Yocto 5.0.9 system requirements

https://docs.yoctoproject.org/5.0.9/ref-manual/system-requirements.html

Use: Frozen release host compatibility and resource guidance. Accessed 2026-10-27.

### YOCTO-SBOM Yocto 5.0.9 SBOM creation

https://docs.yoctoproject.org/5.0.9/dev-manual/sbom.html

Use: SPDX inventory generation. Accessed 2026-10-27.

### CMSIS Arm CMSIS Cortex-M startup

https://arm-software.github.io/CMSIS_6/latest/Core/startup_c_pg.html

Use: Reset handler and vector initialization. Accessed 2026-10-27.

### NVIC Arm CMSIS NVIC

https://arm-software.github.io/CMSIS_6/latest/Core/group__NVIC__gr.html

Use: Interrupt and exception interfaces. Accessed 2026-10-27.

### NUCLEO ST NUCLEO-F446RE board

https://www.st.com/en/evaluation-tools/nucleo-f446re.html

Use: Selected physical MCU board documentation. Accessed 2026-10-27.

### PI-HW Raspberry Pi hardware documentation

https://www.raspberrypi.com/documentation/computers/raspberry-pi.html

Use: Selected Linux board family hardware reference. Accessed 2026-10-27.

### FREERTOS-MUTEX FreeRTOS mutexes

https://www.freertos.org/Documentation/02-Kernel/02-Kernel-features/02-Queues-mutexes-and-semaphores/04-Mutexes

Use: Simplified priority inheritance and task-context mutex usage. Accessed 2026-10-27.

### GDB GDB backtraces

https://sourceware.org/gdb/current/onlinedocs/gdb.html/Backtrace.html

Use: Stack inspection and frame meaning. Accessed 2026-10-27.

### UBOOT U-Boot v2025.01 verified boot

https://docs.u-boot.org/en/v2025.01/usage/fit/verified-boot.html

Use: Verified boot and signing concepts. Accessed 2026-10-27.

### RAUC RAUC basics

https://rauc.readthedocs.io/en/latest/basic.html

Use: Image-based update and boot integration concepts. Accessed 2026-10-27.

### LITERT Google LiteRT for Microcontrollers

https://developers.google.com/edge/litert/microcontrollers/overview

Use: MCU inference deployment and limitations. Accessed 2026-10-27.

### ONNX ONNX Runtime quantization

https://onnxruntime.ai/docs/performance/model-optimizations/quantization.html

Use: Quantization choices and evaluation needs. Accessed 2026-10-27.

## Additional project and interview references

These primary references supplement the core bibliography. Their identifiers are local to this consolidated list; project files preserve their own source identifiers and direct links. All entries record the access date.

### P001 Clang AddressSanitizer

https://clang.llvm.org/docs/AddressSanitizer.html

Use: Instrumented runtime detection, build flags, and limitations. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P002 Linux poll manual

https://man7.org/linux/man-pages/man2/poll.2.html

Use: Readiness, hangup and error behavior. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P003 Linux epoll manual

https://man7.org/linux/man-pages/man7/epoll.7.html

Use: Level and edge triggering, drain-to-EAGAIN convention. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P004 Linux clock_gettime manual

https://man7.org/linux/man-pages/man2/clock_gettime.2.html

Use: Monotonic clocks and suspend semantics. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P005 Linux futex manual

https://man7.org/linux/man-pages/man7/futex.7.html

Use: Userspace synchronization and kernel waiting foundation. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P006 Linux 6.12 lock types and rules

https://docs.kernel.org/6.12/locking/locktypes.html

Use: Sleeping and spinning locks; PREEMPT_RT qualification. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P007 Linux 6.12 managed device resources

https://docs.kernel.org/6.12/driver-api/driver-model/devres.html

Use: Managed resource scope and release behavior. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P008 Linux 6.12 DMA mapping guide

https://docs.kernel.org/6.12/core-api/dma-api-howto.html

Use: Device address domains, coherent/streaming mapping, ordering. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P009 Linux 6.12 runtime power management

https://docs.kernel.org/6.12/power/runtime_pm.html

Use: Usage references, runtime suspend and resume. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P010 Linux 6.12 IIO introduction

https://docs.kernel.org/6.12/driver-api/iio/intro.html

Use: Sensor subsystem role and channel model. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P011 Linux 6.12 ioctl interfaces

https://docs.kernel.org/6.12/driver-api/ioctl.html

Use: UAPI fixed-width types, layout, compat handling and information leaks. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P012 Linux 6.12 RCU overview

https://docs.kernel.org/6.12/RCU/whatisRCU.html

Use: Publication, grace periods and delayed reclamation. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P013 Linux 6.12 workqueues

https://docs.kernel.org/6.12/core-api/workqueue.html

Use: Worker context and asynchronous work lifecycle. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P014 Linux 6.12 device tree usage model

https://docs.kernel.org/6.12/devicetree/usage-model.html

Use: Hardware description and device population. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P015 Linux 6.12 driver model

https://docs.kernel.org/6.12/driver-api/driver-model/index.html

Use: Device matching, binding and driver-model structure. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P016 U-Boot v2025.01 documentation

https://docs.u-boot.org/en/v2025.01/

Use: Board-specific bootloader documentation entry point. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P017 Arm CMSIS Cortex-M core documentation

https://arm-software.github.io/CMSIS_6/latest/Core/index.html

Use: Cortex-M core/startup support; exact MCU manual still required. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P018 QEMU system emulation manual

https://www.qemu.org/docs/master/system/index.html

Use: Machine models and emulation scope; latest documentation is conceptual only. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P019 Linux 6.12 I2C driver implementation

https://docs.kernel.org/6.12/i2c/writing-clients.html

Use: Bus device drivers and transfer checks. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P020 Linux 6.12 SPI support overview

https://docs.kernel.org/6.12/spi/spi-summary.html

Use: SPI controller/device model and transfers. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P021 Linux v6.12 regmap interface source

https://raw.githubusercontent.com/torvalds/linux/v6.12/include/linux/regmap.h

Use: Register map configuration and access interface. Reference checked 2026-10-27. Retrieval scope: Official pinned source URL identified; contents unavailable through web retrieval. Verify in the local v6.12 source tree.

### P022 MCUboot bootloader design

https://docs.mcuboot.com/design.html

Use: Image validation, swap and recovery design concepts. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P023 Yocto 5.0.12 creating layers

https://docs.yoctoproject.org/5.0.12/dev-manual/layers.html

Use: Layers, recipes and metadata organization. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P024 Yocto 5.0.12 reproducible builds

https://docs.yoctoproject.org/5.0.12/test-manual/reproducible-builds.html

Use: Bitwise reproducibility goals and validation. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P025 Yocto 5.0.12 creating an SBOM

https://docs.yoctoproject.org/5.0.12/dev-manual/sbom.html

Use: Build-generated SPDX software inventory. Reference checked 2026-10-27. Retrieval scope: Official documentation retrieved or official search result inspected; exact claim scope is listed.

### P026 GNU GCC instrumentation options

https://gcc.gnu.org/onlinedocs/gcc/Instrumentation-Options.html

Use: Address undefined behavior and thread instrumentation. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P027 Linux man pages termios

https://man7.org/linux/man-pages/man3/termios.3.html

Use: Raw serial settings and baud configuration. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P028 Linux pthread condition documentation

https://man7.org/linux/man-pages/man3/pthread_cond_init.3.html

Use: Condition predicate loops and mutex relationship; historical LinuxThreads specifics on this page do not describe modern NPTL. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P029 Bootlin official toolchain archive

https://toolchains.bootlin.com/downloads/releases/toolchains/aarch64/tarballs/

Use: Pinned AArch64 2024.05-1 glibc toolchain availability. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P030 Buildroot release archive

https://buildroot.org/downloads/

Use: Archived 2025.02.1 archive exists; newer releases also listed. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P031 Linux 6.12 miscellaneous devices

https://docs.kernel.org/6.12/driver-api/misc_devices.html

Use: misc registration lifetime and private data. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P032 Linux 6.12 kernel API

https://docs.kernel.org/6.12/core-api/kernel-api.html

Use: kfifo user copies and FIFO helpers. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P033 Linux 6.12 external modules

https://docs.kernel.org/6.12/kbuild/modules.html

Use: External builds prepared trees symbol versions. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P034 Raspberry Pi hardware

https://www.raspberrypi.com/documentation/computers/raspberry-pi.html#gpio

Use: 3.3 V logic, physical pin 11 is GPIO17, pin 6 ground. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P035 Yocto 5.0.9 documentation

https://docs.yoctoproject.org/5.0.9/singleindex.html

Use: Pinned Scarthgap environment and layer/image/SDK concepts. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P036 Yocto quick build

https://docs.yoctoproject.org/5.0/brief-yoctoprojectqs/index.html

Use: QEMU build and run model. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P037 FreeRTOS GCC POSIX simulator

https://www.freertos.org/Documentation/02-Kernel/03-Supported-devices/04-Demos/03-Emulation-and-simulation/Linux/FreeRTOS-simulator-for-Linux

Use: Real kernel hosted port and simulation limits. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P038 FreeRTOS V11.1.0 source

https://github.com/FreeRTOS/FreeRTOS-Kernel/tree/V11.1.0

Use: Pinned RTOS source and POSIX port. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P039 STM32F446RE product and datasheet

https://www.st.com/en/microcontrollers-microprocessors/stm32f446re.html

Use: Cortex M4F memory map target. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P040 STM32F446 RM0390

https://www.st.com/st-web-ui/static/active/en/resource/technical/document/reference_manual/DM00135183.pdf

Use: RCC USART GPIO IWDG register configuration. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P041 Nucleo MB1136 UM1724

https://www.st.com/resource/en/user_manual/dm00105823.pdf

Use: USART2 PA2 PA3 ST LINK bridge mapping. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P042 TMP102 product and datasheet

https://www.ti.com/lit/ds/symlink/tmp102.pdf

Use: 12-bit normal mode signed format 0.0625 C step, config and address. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P043 IIO core

https://www.kernel.org/doc/html/v6.12/driver-api/iio/core.html

Use: IIO device registration and channel interface. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P044 Kernel event tracing

https://docs.kernel.org/6.12/trace/events.html

Use: Scheduler event correlation and trace collection. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P045 Python time

https://docs.python.org/3.12/library/time.html

Use: Monotonic time and sleep can exceed requested duration. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P046 OpenSSL dgst

https://docs.openssl.org/3.0/man1/openssl-dgst/

Use: Detached signing and public-key verification. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P047 RAUC using updates

https://rauc.readthedocs.io/en/latest/using.html

Use: Health confirmation after trial boot and real-system distinction. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P048 Python selectors

https://docs.python.org/3.12/library/selectors.html

Use: Nonblocking descriptor readiness. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P049 FreeRTOS queues

https://www.freertos.org/Documentation/02-Kernel/02-Kernel-features/02-Queues-mutexes-and-semaphores/01-Queues

Use: MCU producer consumer integration. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P050 FreeRTOS task scheduling

https://www.freertos.org/Documentation/02-Kernel/02-Kernel-features/01-Tasks-and-co-routines/03-Task-states

Use: Control and supervisory scheduling concepts. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.

### P051 Python random

https://docs.python.org/3.12/library/random.html

Use: Seeded synthetic data reproducibility. Reference checked 2026-10-27. Retrieval scope: See the project source record and validation scope.